Casa Logbook

# Part 66 Basic Practical Experience Exclusion Removal Logbook

July 2026

Acknowledgement of Country

The Civil Aviation Safety Authority (CASA) respectfully acknowledges the Traditional Custodians of the lands on which our offices are located and the places to which we travel for work. We also acknowledge the Traditional Custodians’ continuing connection to land, water and community. We pay our respects to Elders, past and present.

Artwork: James Baban.

© Civil Aviation Safety Authority

All material presented in this Guidance document is provided under a Creative Commons Attribution 4.0 International licence, with the exception of the Commonwealth Coat of Arms (the terms of use for the Coat of Arms are available from the It’s an Honour website). The details of the relevant licence conditions are available on the Creative Commons website, as is the full legal code for the CC BY 4.0 license.

Attribution

Material obtained from this document is to be attributed to CASA as:

© Civil Aviation Safety Authority 2023.

Contents

Introduction	6

Introduction																				7

Background	7

Logbook structure and completion requirements	7

Eligibility requirements for exclusion removal	7

Basic knowledge requirements for exclusion removal	7

Maintenance experience required for exclusion removal	8

Exclusion removal tasks – responsibilities	8

Part 66 Category Exclusion List	10

Personal Information	11

Part 1 – Personal Information																12

Personal Data	12

Category exclusion(s) applied for:	12

Employment Record	13

CASA acceptance of logbook category exclusion removal sections (CASA use only)	16

Part 2 Exclusion(s) Removal Maintenance Practical Experience	19

Category B1 Mechanical Licence															19

Part 2 – Exclusion(s) Removal Maintenance Practical Experience	20

Category B1 Mechanical Licence exclusion removal task list	20

Exclusions E1, E4 – Category B1.1 / B1.2 / B1.3 / B1.4	20

Exclusion E2 – Category B1.1 / B1.2 Aeroplane (Airframe)	28

Exclusion E2 – Category B1.3 / B1.4 Helicopters (Airframe)	43

Exclusion E3 – Category B1.1 / B1.2 Powerplant (Turbine)	56

Exclusion E3 – Category B1.2 / B1.4 Powerplant (Piston)	66

Exclusion E5 – Category B1.1 / B1.2 / B1.3 / B1.4	73

Exclusion E6 – Category B1.1 / B1.2 / B1.3 / B1.4	77

Exclusion E9 – Category B1.1 / B1.2	79

Exclusion E10 – Category B1.2	82

Exclusion E12 – Category B1.1 / B1.2	85

Exclusion E13 – Category B1.1 / B1.2 / B1.3 / B1.4	90

Exclusion E14 – Category B1.1 / B1.2 / B1.3 / B1.4	92

Exclusion E15 – Category B1.1 / B1.2 / B1.3 / B1.4	95

Exclusion E16 – Category B1.1 / B1.2	98

Exclusion E32 – Category B1.1 / B1.2 / B1.3 / B1.4	100

Exclusion E33 – Category B1.2 / B1.4	101

Exclusion E35 – Category B1.1 / B1.2	104

Exclusion E36 – Category B1.2 / B1.4	107

Exclusion E37 – Category B1.2 / B1.4	110

Exclusion E38 – Category B1.2 / B1.4	113

Exclusion E39 – Category B1.1 / B1.3	116

Exclusion E40 – Category B1.1 / B1.2 / B1.3 / B1.4	119

Exclusion E41 – Category B1.1 / B1.2	121

Exclusion E42 – Category B1.1 / B1.2 / B1.3 / B1.4	124

Exclusion E43 – Category B1.1 / B1.2	126

Exclusion E44 – Category B1.1	128

Part 3 Exclusion(s) Removal Maintenance Practical Experience	130

Category B2 Avionics Licence																130

Part 3 – Exclusion(s) Removal Maintenance Practical Experience	131

Category B2 Avionics Licence exclusion(s) removal task list	131

Exclusions E1, E4 – Category B2	131

Exclusion E5 – Category B2	140

Exclusion E7 – Category B2	144

Exclusion E8 – Category B2	154

Exclusion E11 – Category B2	170

Exclusion E18 – Category B2	173

Exclusion E19 – Category B2	176

Exclusion E20 – Category B2	179

Exclusion E21 – Category B2	184

Exclusion E22 – Category B2	187

Exclusion E23 – Category B2	190

Exclusion E24 – Category B2	193

Exclusion E25 – Category B2	196

Exclusion E26 – Category B2	199

Exclusion E27 – Category B2	202

Exclusion E28 – Category B2	205

Exclusion E29 – Category B2	209

Exclusion E30 – Category B2	212

Exclusion E31 – Category B2	216

Exclusion E32 – Category B2	220

Exclusion E34 – Category B2	223

Part 4 Exclusion(s) Removal Maintenance Practical Experience	226

Category B1 / B2																	226

Additional Practical Maintenance Experience for Category Exclusion Removal – Category B1/B2	227

Introduction

## Introduction

### Background

This logbook has been developed by CASA for applicants seeking removal of Part 66 licence category or subcategory exclusions through the self-study pathway under paragraph 66.072(3)(b) of CASR and paragraph 66.A.70(b) of the Part 66 MOS.

Use this logbook to record the required practical maintenance experience.

### Logbook structure and completion requirements

The logbook is organised into four parts to help applicants and supervisors complete and verify requirements in a consistent and logical order:

Part 1 – Personal information

Part 2 – Practical experience for removal of Category B1 mechanical exclusions (airframe, engine and electrical)

Part 3 – Practical experience for removal of Category B2 avionics exclusions (electrical, instrument and radio)

Part 4 – Additional Practical Maintenance Experience

Complete all relevant sections of this logbook to demonstrate that you meet the practical maintenance experience requirements for the exclusion(s) you are applying to have removed.

### Eligibility requirements for exclusion removal

To remove exclusion(s) through the self-study pathway, applicants must meet both the knowledge requirements and the practical maintenance experience requirements.

Part 66 modular licence holders applying for removal of an exclusion that would result in a new category or subcategory being added to their licence will also need to meet the applicable experience, as documented in Table 3A and paragraph 66.A.30(ba) of the Part 66 MOS.

### Basic knowledge requirements for exclusion removal

Applicants must demonstrate that they have met the applicable Part 66 basic knowledge requirements for the excluded system(s) and provide supporting evidence to CASA with their application.

Complete the basic knowledge module(s) relevant to the exclusion(s) being removed, in accordance with the applicable Part 66 MOS syllabus and examination standard.

Evidence of successful completion may include CASA examination results for the relevant module(s), or other evidence accepted by CASA, where applicable.

Note 1:	Applicants can do the self-assessment to determine what credits for completed CASA Basics exams apply towards the Part 66 basic knowledge requirements. To conduct a self-assessment for completed CASA Basic exams, review the module tables listed in Annex A of the AMC/GM Part 147. This will determine what credits from completed CASA Basics exams are accepted towards the Part 66 basic knowledge modules.
AMC-GM Part 147 annex A: Mapping CASA basics exam syllabus to Part 66 basic knowledge syllabus

Note 2:	CASA will provide credit for a Part 66 basic knowledge module if all the topics contained in the Part 66 basic knowledge module were covered in the CASA Basics exam(s). No Part 66 basic knowledge module exam credit is given for partial completion of a module with CASA basic knowledge gaps in self-assessment.

### Maintenance experience required for exclusion removal

Applicants must complete the required practical maintenance experience tasks for the excluded system(s) in this logbook, with each entry verified by an appropriately qualified supervisor. Completion of the listed tasks demonstrates competence to exercise the privileges that would apply once the exclusion(s) are removed.

Tasks should be performed on in-service aircraft or aircraft components, as applicable, using approved maintenance data and in accordance with the organisation’s procedures.

Each task must be supervised at the time of performance and verified by a supervisor who holds Part 66 privileges appropriate to the relevant system(s), is authorised by a Part 147 maintenance training organisation, or is otherwise authorised by CASA.

Each task entry must include a valid work order, job card reference, or equivalent maintenance record reference, and be supported by traceable evidence if requested.

When applying to CASA for exclusion removal, submit the completed exclusion removal section(s) of the logbook and retain supporting maintenance documentation, such as job cards, worksheets, and relevant maintenance records, as CASA may request this material to validate entries.

### Exclusion removal tasks – responsibilities

Logbook holder responsibilities

When recording practical maintenance experience in this logbook, ensure that:

At least 80% of the practical maintenance tasks listed in each relevant exclusion removal section have been completed, unless otherwise specified and covered by additional maintenance tasks.

The applicant has initialled each task entry.

Each task completed by the applicant has been supervised and signed by the supervisor at the time the task was performed.

The applicant does not certify or sign off their own maintenance experience entries.

Each entry includes a valid work order reference, job card reference, or equivalent evidence of the maintenance performed.

All entries are supported by verifiable documentation, such as job cards, if requested by CASA.

Tasks not performed are left blank.

All logbook entries are clearly legible – black ink is recommended.

Assessor/Supervisor responsibilities

Each entry must be verified by an appropriately qualified Part 66 licence holder with privileges in the relevant category or subcategory, a person authorised by a Part 147 MTO, or another person authorised by CASA, who directly supervised the logbook holder performing the task.

By signing to verify completion of a task, the supervisor confirms that the logbook holder has:

Successfully performed the task under supervision and in accordance with approved maintenance data.

Correctly identified and applied the relevant maintenance documentation and standards.

Selected and used the correct tools to complete the task.

Signed and dated the entry at the time the task is performed (as close as reasonably practical).

Note:	Retain verifiable supporting documentation for all logbook entries, as CASA may request it.

### Part 66 Category Exclusion List

Logbook Part 2 – B1 Mechanical

E1	Excluding eclectically systems

E2	Excluding mechanical or structural systems

E3	Excluding powerplant systems

E4	Excluding electrical subsystems of mechanical, powerplant or
structural systems

E5	Excluding instrument subsystems of mechanical, powerplant or 
structural systems

E6	Excluding avionic LRUs

E9	Excluding fabric surfaces

E10	Excluding wooden structures

E12	Excluding propellers

E13	Excluding hydraulics — ATA 29

E14	Excluding vapour cycle air-conditioning aspects of ATA 21

E15	Excluding air-conditioning aspects of ATA 21

E16	Excluding pressurisation aspects of ATA 21

E33	Excluding all supercharging systems

E35	Excluding pressurised structures

E36	Excluding carburettor systems

E37	Excluding fuel injection systems

E38	Excluding turbo supercharging systems

E39	Excluding airframe ice protection systems

E40	Excluding airframe fire protection systems

E41	Excluding oxygen systems

E42	Excluding landing gear retraction systems

E43	Excluding fabric other than flight controls

E44	Excluding wiring repairs
Logbook Part 3 – B2 Avionics

E1	Excluding electrical systems

E4	Excluding electrical subsystems of mechanical, powerplant or 
structural systems

E5	Excluding instrument subsystems of mechanical, powerplant or 
structural systems

E6	Excluding avionic LRUs

E7	Excluding instrument aspects of avionics systems – ATA 22, 27, 31, 
34 and 42

E8	Excluding radio aspects of avionics systems – ATA 23, 34, 42 and 44

E11	Excluding audio CVR systems

E18	Excluding ADF systems

E19	Excluding VOR systems E20

E21	Excluding ILS systems

E22	Excluding weather radar systems

E23	Excluding ATC transponder systems

E24	Excluding radio altimeter systems

E25	Excluding doppler systems

E26	Excluding satellite navigation systems

E27	Excluding autopilots

E28	Excluding multi-axis autopilots

E29	Excluding remote indicating compass systems

E30	Excluding inertial navigation and reference systems

E31	Excluding pressurisation systems

E32	Excluding electrical systems in aircraft equipped with multi-generator 
powered systems

E34	Excluding digital systems

Personal Information

## Part 1 – Personal Information

### Personal Data

| Personal Data/Information |  |
| --- | --- |
| First Name: | Surname: |
| ARN: | Contact Number: |
| Date of Birth: | Nationality: |
| Present address: |  |

Privacy statement: Any personal information you provide to CASA is protected by the Privacy Act 1988 (Cth). CASA can only collect, use, and disclose that information in accordance with that Act. CASA will use the information collected in this form for purposes associated with performing its functions under the Civil Aviation Act 1988, the Airspace Act 2007, the Aviation Transport Security Act 2004, or the regulations made under those Acts. For full details on how CASA collects, protects, and uses personal information, please refer to CASA’s Privacy Policy.

### Category exclusion(s) applied for:

Annotate the category or subcategory and the exclusion(s) to be removed in the blank field.

| Category B1 Mechanical (Airframe/Engine/Electrical |  |  |  |  | Category B2 Avionics (Electrical/Instruments/Radio) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category or Sub-category |  | Exclusion(s) to be removed |  |  | Category |  | Exclusion(s) to be removed |  |

### Employment Record

| Employer One |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

| Employer Two |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

Employment Record Continued

| Employer Three |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

| Employer Four |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

Employment Record Continued

| Employer Five |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

| Employer Six |  |  |  |
| --- | --- | --- | --- |
| Employer Name: |  |  |  |
| From: | To: | Position: |  |
| Nature of duties: |  |  |  |
| Type of aircraft or other products: |  |  |  |
| Confirmed by: Signature: |  | Position: | Date: |

### CASA acceptance of logbook category exclusion removal sections (CASA use only)

| Date | Assessed category exclusion(s) of the logbook | MPL AWI Name | MPL AWI Signature |
| --- | --- | --- | --- |

CASA acceptance of logbook category exclusion removal sections (CASA use only)

| Date | Assessed category exclusion(s) of the logbook | MPL AWI Name | MPL AWI Signature |
| --- | --- | --- | --- |

CASA acceptance of logbook category exclusion removal sections (CASA use only)

| Date | Assessed category exclusion(s) of the logbook | MPL AWI Name | MPL AWI Signature |
| --- | --- | --- | --- |

Part 2
Exclusion(s) Removal
Maintenance Practical Experience

## Category B1 Mechanical Licence

### Part 2 – Exclusion(s) Removal Maintenance Practical Experience

#### Category B1 Mechanical Licence exclusion removal task list

###### Exclusions E1, E4 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Use test meters to measure volts, amps, and resistance in practical task circumstances |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Using a multimeter and wiring diagram, check an aircraft electrical circuit for continuity and resistance |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Using a megger and wiring diagram, check an aircraft electrical circuit for insulation resistance |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Read and interpret electrical wiring diagrams, including symbols used |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Carry out bonding tests for equipment bonding, metallic surface bonding, and static bonds |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Select and use appropriate wire stripping tools to strip various gauges of aircraft copper and aluminium wires |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Select, crimp, and install terminal lugs on different gauges of copper and aluminium wires |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Select and splice pre- and post-insulated splices on different gauges of aircraft wires |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Using approved documents, identify aircraft electrical connectors (class, contact style, insert arrangement, and polarisation) |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Using approved documents, identify aircraft electrical terminal blocks (class, contact style, insert arrangement) |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Insert and extract electrical contacts (pins and sockets) in a variety of electrical connectors using the correct insertion/extraction tools |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace different types of aircraft electrical connectors |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Select appropriate crimping tools and terminate pins/sockets in aircraft electrical connectors |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect coaxial cable installations for damage; correct cause if necessary |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Check the integrity of coaxial cables using an ohmmeter |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect electrical looms/harnesses to ensure minimum bend radius is maintained, cables are not in tension, plugs are aligned, and insulation is not chafed |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate familiarity with aircraft wiring specifications and standards |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace aircraft wires by identifying the part number (or alternate) and wire type/class |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect wiring for damage tolerance and repair wiring damage per the aircraft wiring specification manual |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Interpret and apply wire/cable identification and marking using direct and indirect marking methods |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Prepare and terminate a wire loom per the standard wiring practices manual using crimping and/or soldering techniques |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect cables and wire bundles as installed through bulkhead holes |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify, inspect, and replace wiring clamps used in high-vibration, low-vibration, and hot areas |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Apply wiring protection techniques including sleeving and heat-shrink |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform GVI of wiring installations at wheel bays, wings, engines, pylon, and nacelle areas |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform wiring repairs at wheel bays and wing areas |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform wiring repairs at engine and pylon areas |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform simple soldering tasks |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Solder cables to single- and multi-pin connectors and inspect solder joints |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect the main aircraft battery installation for any cracks, leaks, swelling, or loose electrical plugs |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/refit main battery and perform system test per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform battery capacity checks |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect engine generator housing, electrical plugs, and wiring harnesses for loose connections, wire rubbing |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install generator, alternator, or IDG and perform functional test per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of the electrical load shedding system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of generator priority to AC bus per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install generator control unit and perform functional tests per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install voltage regulators and perform functional tests per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install transformer rectifier units (TRUs) and perform functional tests per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install rotary inverter and/or static inverter and perform operational test per AMM |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install electrical control panel and perform operational test per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace circuit breaker using the Electrical Standard Wiring Practices Manual |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify Electrical Power System MMEL requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot Electrical Power System faults and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace external lights and check operation |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace cabin internal lights and passenger service lights and check operation |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of passenger “No Smoking” and “Fasten Seat Belt” signs |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace emergency lights power pack and test the system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace and check operation of internal and external emergency lights |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify internal/external and emergency lighting systems MMEL requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install flight deck map/grimes light |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Perform replacement of ovens, boilers, and beverage makers in the galley and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install engine fire detector element per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect engine fire detecting elements per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform engine fire switch operational test per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace engine fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install cargo smoke detector unit per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Test cargo smoke detection and warning system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Test cargo fire extinguishing system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Test lavatory smoke detection and warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot engine fire warning and cargo/lavatory smoke detection systems and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace and test a fuel boost pump per AMM |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install propeller electrical de-ice boot |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of propeller electric/pneumatic de-icing system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install window heat control unit and test per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of pitot/static/AOA sensor heating per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot anti-ice/de-ice system faults and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install nose and main landing gear downlock sensor |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot nose wheel steering system fault |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install main wheel speed sensor per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of anti-skid system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of autobrake system per AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install toilet flush control unit and test |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect engine ignition leads and starter connections for frayed metal braiding, and loose mounting clamps |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install starter/generator and perform functional test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install engine starter relay and perform functional tests. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/install engine start switch |  |  |  |  |  |
|  | B1.1/B1.3 | Perform ignition system test |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E2 – Category B1.1 / B1.2 Aeroplane (Airframe)

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Use hand & power tools to position and drill rivet holes to an accuracy of ± 0.030ins/0.75mm |  |  |  |  |  |
|  | B1.1/B1.2 | Identify a range of solid and blind rivets and fasteners |  |  |  |  |  |
|  | B1.1/B1.2 | Identify, select, and use a range of rivet setting equipment |  |  |  |  |  |
|  | B1.1/B1.2 | Set a range of raised and countersunk rivets in aluminium sheet using various methods |  |  |  |  |  |
|  | B1.1/B1.2 | Select and use a range of appropriate rivet closing tools |  |  |  |  |  |
|  | B1.1/B1.2 | Perform inspection; identify faulty rivet settings (aircraft structure) |  |  |  |  |  |
|  | B1.1/B1.2 | Remove defective rivets without causing further damage to skin |  |  |  |  |  |
|  | B1.1/B1.2 | Select and install oversize rivets as instructed by Structure Repair Manual (SRM) |  |  |  |  |  |
|  | B1.1/B1.2 | Set a range of different fasteners in aluminium sheet |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Inspect control cable end fittings |  |  |  |  |  |
|  | B1.1/B1.2 | Perform a rigging procedure of control cables with tensiometer |  |  |  |  |  |
|  | B1.1/B1.2 | Identify alclad sheeting by part number |  |  |  |  |  |
|  | B1.1/B1.2 | Use a range of hand tools, folding and bending machines and guillotine to shape aluminium alloy to an accuracy of ± 0.5 mm x of bend angle, ±0.030ins / 0.075mm |  |  |  |  |  |
|  | B1.1/B1.2 | Interpret engineering drawings and calculate size of material required to produce a component of material with one or more bends |  |  |  |  |  |
|  | B1.1/B1.2 | Demonstrate removal of corrosion and reprotection on an aluminium sheet |  |  |  |  |  |
|  | B1.1/B1.2 | Cut and shape sheet metal material to required profile, finish edges and deburr using approved procedures |  |  |  |  |  |
|  | B1.1/B1.2 | Demonstrate use of correct fastener size and type |  |  |  |  |  |
|  | B1.1/B1.2 | Perform correct removal and installation of solid or blind fastener |  |  |  |  |  |
|  | B1.1/B1.2 | Perform correct removal and installation of hi lock/ taper lock type fastener |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Identify the characteristics and properties of common composite and non-metallic materials |  |  |  |  |  |
|  | B1.1/B1.2 | Identify a range of sealing and bonding agents used in composite and non-metallic material |  |  |  |  |  |
|  | B1.1/B1.2 | Explain methods to detect defects/deterioration in composite and non-metallic material |  |  |  |  |  |
|  | B1.1/B1.2 | Perform a tap test of composite material and identify de-bonding |  |  |  |  |  |
|  | B1.1/B1.2 | Perform/discuss updating load data sheet after change in weight and CG due to aircraft alteration/modification |  |  |  |  |  |
|  | B1.1/B1.2 | Prepare aircraft for weighing |  |  |  |  |  |
|  | B1.1/B1.2 | Participate in weighing an aircraft in accordance with the applicable AMM procedure |  |  |  |  |  |
|  | B1.1/B1.2 | Replenish oxygen system, including portable oxygen bottles |  |  |  |  |  |
|  | B1.1/B1.2 | Perform visual inspection of aircraft structure and recognise for signs of deformation defects or damage like impact damage, fatigue cracking, and corrosion, in accordance with maintenance documentation |  |  |  |  |  |
|  | B1.1/B1.2 | Perform corrosion removal, assessment and reprotection on an aircraft part |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform an assessment of aircraft structure damage using the aircraft structural repair manual (SRM) and determine the repair scheme in accordance with SRM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform blending of damaged aircraft skin or structure within dimensional accuracy of± 0.010 in/ 0.25 mm |  |  |  |  |  |
|  | B1.1/B1.2 | In an abnormal event of hard / overweight landing, show the unscheduled inspection procedure and required action according to AMM |  |  |  |  |  |
|  | B1.1/B1.2 | In an abnormal event of flight through severe turbulence, show the unscheduled inspection procedure and required action according to AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Locate Aircraft Doors/Panels using zonal and station identification Method |  |  |  |  |  |
|  | B1.1/B1.2 | Discuss the procedure and precautions of aircraft exterior cleaning |  |  |  |  |  |
|  | B1.1/B1.2 | Perform Zonal visual inspection of fuselage skin/ structure |  |  |  |  |  |
|  | B1.1/B1.2 | Perform zonal visual inspection of wing skin/structure |  |  |  |  |  |
|  | B1.1/B1.2 | Perform zonal visual inspection of stabilizer skin/ structure |  |  |  |  |  |
|  | B1.1/B1.2 | Perform conditional hard landing / lightning strike inspection |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove/ install main entry door |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install emergency exit door/hatch |  |  |  |  |  |
|  | B1.1/B1.2 | Check main doors and emergency exit doors seal and replace, if required |  |  |  |  |  |
|  | B1.1/B1.2 | Discuss procedure for cleaning / polishing windows |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install cockpit windshield |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install cabin window |  |  |  |  |  |
|  | B1.1/B1.2 | Perform visual inspection of cockpit windshield for scratches, damage delamination etc. |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install passenger seat |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install crew seat |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install air cycle machine (ACM) |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install pack flow control valve |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove/ install ACM primary heat exchanger |  |  |  |  |  |
|  | B1.1/B1.2 | Remove /install temperature control valve |  |  |  |  |  |
|  | B1.1/B1.2 | Functionally test air cycle air conditioning system IAW AMM for evidence of serviceability or malfunction |  |  |  |  |  |
|  | B1.1/B1.2 | Locate air cycle air conditioning system faults, identify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install combustion heaters and inspection checks in accordance with maintenance manual |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install Vapor Cycle system blower motor assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Test Vapour cycle air conditioning system for correct function and freedom from refrigerant leaks |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install various temperature sensors of air-conditioning system and check operation |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install pressure outflow valve/safety valve in accordance with maintenance manual |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install cabin pressure controller in accordance with maintenance manual |  |  |  |  |  |
|  | B1.1/B1.2 | Check operation of pressurisation system in accordance with maintenance manual |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Identify air-conditioning and pressurisation system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot faulty air-conditioning /pressurisation Systems and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect cabin / cockpit emergency equipment for serviceability |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect passenger seat belts and crew seat belt/harness for serviceability |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install passenger seat belt |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install crew inertia harness seat belt |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and refit passenger/crew seat |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install cargo bay netting equipment |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install engine fire detector element IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform inspection of engine fire detecting elements IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform engine fire switch operation test IAW AMM |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove / install cargo smoke detector unit IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform test of lavatory smoke detection and warning system |  |  |  |  |  |
|  | B1.1/B1.2 | Identify engine fire warning and cargo/lavatory smoke detection system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ Install trailing edge flap assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Perform flap rigging and functional checks IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Run a functional check on auto flap retraction (if applicable) |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install primary flight control surface |  |  |  |  |  |
|  | B1.1/B1.2 | Run a functional check on ailerons, elevators, rudder, and spoilers |  |  |  |  |  |
|  | B1.1/B1.2 | Perform balance check procedure on primary flight control surface |  |  |  |  |  |
|  | B1.1/B1.2 | Perform rigging of flight controls following components replacement |  |  |  |  |  |
|  | B1.1/B1.2 | Discuss/review replacement procedure for a hydraulic flight control actuator |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform horizontal stabilizer electric trim system functional tests |  |  |  |  |  |
|  | B1.1/B1.2 | Perform gust lock operational check |  |  |  |  |  |
|  | B1.1/B1.2 | Perform inspection and operational checks of horizontal stabilizer system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform inspection and operational test of speed brake system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Identify flight control system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot faulty flight control system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install fuel tank access panels IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Replace and test a fuel boost pump IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Carry out operational test of engine fuel feed shut off valve IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Carry out functional test fuel cross feed/transfer valve |  |  |  |  |  |
|  | B1.1/B1.2 | Carry out wing to wing fuel transfer IAW AMM |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform magnetic/manual reading of fuel level indication system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install fuel drain valve IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install fuel bladder cell |  |  |  |  |  |
|  | B1.1/B1.2 | Carry out aircraft refuelling and defueling |  |  |  |  |  |
|  | B1.1/B1.2 | Identify fuel system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot fuel system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect hydraulic filters for foreign particles IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install hydraulic pressure filter assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install hydraulic return filter assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of hydraulic supply shutoff valve IAW AMM |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform operational test of hydraulic system reservoir quantity indication IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of hydraulic system temperature indication IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Check/replenish hydraulic and/or pneumatic accumulator pressure. |  |  |  |  |  |
|  | B1.1/B1.2 | Perform visual inspection of hydraulic filter bypass indicator |  |  |  |  |  |
|  | B1.1/B1.2 | Identify hydraulic system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot hydraulic system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install pneumatic de-ice boot |  |  |  |  |  |
|  | B1.1/B1.2 | Perform patch repair de-ice boot |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of electric / pneumatic de-icing boots on wing and horizontal stabilizer IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of wing anti-ice system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install windshield wiper arm assembly IAW AMM |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform windshield rain repellent application |  |  |  |  |  |
|  | B1.1/B1.2 | Identify anti ice/de-ice system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot anti ice/de-ice system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install nose wheel assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install main gear wheel assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect, remove, and install wheel brake assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Bleed hydraulic brakes |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install landing gear door assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Assess shock strut fluid level and recharge if required |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect shock strut oleo for correct pressure vs extension |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install landing gear extension retraction actuator |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Assist or discuss replacement of seals on shock strut |  |  |  |  |  |
|  | B1.1/B1.2 | Prepare airplane for landing gear retraction / extension operation |  |  |  |  |  |
|  | B1.1/B1.2 | Perform normal operation landing gear extension retraction procedure |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test emergency landing gear extension |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of nose wheel steering system |  |  |  |  |  |
|  | B1.1/B1.2 | Trouble shoot nose wheel steering system fault |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test of landing gear position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test of landing gear doors position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test of air-Ground sensing and warnings |  |  |  |  |  |
|  | B1.1/B1.2 | Identify landing gear and doors position indication system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot landing gear and doors position indication system and rectify defects |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform functional test of anti-skid system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test of autobrake system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Identify hazards and discuss precaution and safety measures required for oxygen system |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install oxygen cylinder |  |  |  |  |  |
|  | B1.1/B1.2 | Observe safety precautions and Replenish oxygen system |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of crew and passenger oxygen system |  |  |  |  |  |
|  | B1.1/B1.2 | Perform drain and replenish the potable water system |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect and service the toilets |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install toilet assembly |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install water faucet |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test waste tank qty indication |  |  |  |  |  |

E2 – Excluding (aeroplane) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform functional test potable water qty indication |  |  |  |  |  |
|  | B1.1/B1.2 | Trouble shoot water/waste system fault |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E2 – Category B1.3 / B1.4 Helicopters (Airframe)

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Use hand & power tools to position and drill rivet holes to an accuracy of ± 0.030ins/0.75mm |  |  |  |  |  |
|  | B1.3/B1.4 | Identify a range of solid and blind rivets and fasteners |  |  |  |  |  |
|  | B1.3/B1.4 | Identify, select, and use a range of rivet setting equipment |  |  |  |  |  |
|  | B1.3/B1.4 | Set a range of raised and countersunk rivets in aluminium sheet using various methods |  |  |  |  |  |
|  | B1.3/B1.4 | Select and use a range of appropriate rivet closing tools |  |  |  |  |  |
|  | B1.3/B1.4 | Perform inspection; identify faulty rivet settings (aircraft structure) |  |  |  |  |  |
|  | B1.3/B1.4 | Remove defective rivets without causing further damage to skin |  |  |  |  |  |
|  | B1.3/B1.4 | Select and install oversize rivets as instructed by Structure Repair Manual (SRM) |  |  |  |  |  |
|  | B1.3/B1.4 | Set a range of different fasteners in aluminium sheet |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Inspect swaging of control cable end fitting |  |  |  |  |  |
|  | B1.3/B1.4 | Perform a rigging procedure of control cables with tensiometer |  |  |  |  |  |
|  | B1.3/B1.4 | Identify alclad sheeting by part number |  |  |  |  |  |
|  | B1.3/B1.4 | Use a range of hand tools, folding and bending machines and guillotine to shape aluminium alloy to an accuracy of ± 0.5 degrees of bend angle, ±0.030ins linear dimensions |  |  |  |  |  |
|  | B1.3/B1.4 | Interpret engineering drawings and calculate size of material required to produce a component of material with one or more bends. |  |  |  |  |  |
|  | B1.3/B1.4 | Demonstrate removal of corrosion and reprotection on an aluminium sheet |  |  |  |  |  |
|  | B1.3/B1.4 | Cut and shape sheet metal material to required profile, finish edges and deburr using approved procedures. |  |  |  |  |  |
|  | B1.3/B1.4 | Demonstrate use of correct fastener size and type |  |  |  |  |  |
|  | B1.3/B1.4 | Perform correct removal and installation of solid or blind fastener |  |  |  |  |  |
|  | B1.3/B1.4 | Perform correct removal and installation of hi lock/ taper lock type fastener |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Perform correct procedure for fastener oversize |  |  |  |  |  |
|  | B1.3/B1.4 | Identify the characteristics and properties of common composite and non-metallic materials |  |  |  |  |  |
|  | B1.3/B1.4 | Identify a range of sealing and bonding agents used in composite and non-metallic material |  |  |  |  |  |
|  | B1.3/B1.4 | Explain methods to detect defects/deterioration in composite and non-metallic material |  |  |  |  |  |
|  | B1.3/B1.4 | Perform repair of composite structure |  |  |  |  |  |
|  | B1.3/B1.4 | Perform a tap test of composite material and identify de-bonding |  |  |  |  |  |
|  | B1.3/B1.4 | Perform/discuss updating load data sheet after change in weight and CG due to aircraft alteration/modification |  |  |  |  |  |
|  | B1.3/B1.4 | Prepare aircraft for weighing |  |  |  |  |  |
|  | B1.3/B1.4 | Participate in weighing an aircraft in accordance with the applicable AMM procedure. |  |  |  |  |  |
|  | B1.3/B1.4 | Perform visual inspection of aircraft structure and recognise for signs of deformation defects or damage like impact damage, fatigue cracking, and corrosion, in accordance with maintenance documentation |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Perform corrosion removal, assessment and reprotection on an aircraft part |  |  |  |  |  |
|  | B1.3/B1.4 | Perform an assessment of aircraft structure damage using the aircraft structural repair manual (SRM) and determine the repair scheme in accordance with SRM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform blending of damaged aircraft skin or structure within dimensional accuracy of± 0.010 in/ 0.25 mm |  |  |  |  |  |
|  | B1.3/B1.4 | In an abnormal event of hard / overweight landing, show the unscheduled inspection procedure and required action according to AMM |  |  |  |  |  |
|  | B1.3/B1.4 | In an abnormal event of flight through severe turbulence, show the unscheduled inspection procedure and required action according to AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Locate Aircraft Doors/Panels using zonal and station identification Method |  |  |  |  |  |
|  | B1.3/B1.4 | Discuss the procedure and precautions of aircraft exterior cleaning |  |  |  |  |  |
|  | B1.3/B1.4 | Perform Zonal visual inspection of fuselage skin/ structure |  |  |  |  |  |
|  | B1.3/B1.4 | Demonstrate helicopter mooring and picketing |  |  |  |  |  |
|  | B1.3/B1.4 | Perform main / tail rotor blades tie down procedure |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Remove / install main / tail rotor blades |  |  |  |  |  |
|  | B1.3/B1.4 | Perform main/ tail rotor flight control rigging check |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install main rotor head |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect main rotor head assembly for damage |  |  |  |  |  |
|  | B1.3/B1.4 | Check main rotor static and dynamic balancing |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install and adjust main rotor pitch change links |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / refit swashplate drive link |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install tail rotor blades |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install tail rotor hub and blade assembly |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install tail rotor pitch change mechanism |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect tail rotor blades for damage |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Remove / install collective stick |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install collective / cyclic balance spring |  |  |  |  |  |
|  | B1.3/B1.4 | Identify flight control systems MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot flight control system faults, rectify and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Check main rotor / tail rotor track and balance |  |  |  |  |  |
|  | B1.3/B1.4 | Perform mast torque system calibration check |  |  |  |  |  |
|  | B1.3/B1.4 | Check main / tail rotor drive shaft dynamic balance |  |  |  |  |  |
|  | B1.3/B1.4 | Identify blade tracking and vibration systems MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot blade tracking and vibration system faults, rectify and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install main rotor gear box |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect main gear box mounts |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Remove / install main rotor drive shaft |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect main transmission and transmission mount for damage |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / refit min gear box chip detector and functional check |  |  |  |  |  |
|  | B1.3/B1.4 | Install and grease tail rotor drive shaft flexible coupling |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect tail gearbox for damage and leaks |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect tail rotor drive shaft and flexible coupling for damage |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install tail rotor drive shaft bearing support |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / refit tail gear box chip detector and functional check |  |  |  |  |  |
|  | B1.3/B1.4 | Service main /tail gearbox with oil |  |  |  |  |  |
|  | B1.3/B1.4 | Identify helicopter transmission systems MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot helicopter transmission system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Demonstrate the procedure for a structural inspection (from nose to tail) |  |  |  |  |  |
|  | B1.3/B1.4 | Locate Aircraft Doors/Panels using zonal and station Identification method |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install heating shutoff valve and check operation of heating system |  |  |  |  |  |
|  | B1.3/B1.4 | Check operation of ventilation system |  |  |  |  |  |
|  | B1.3/B1.4 | Remove and install air cycle machine (ACM) |  |  |  |  |  |
|  | B1.3/B1.4 | Remove and install pack flow control valve |  |  |  |  |  |
|  | B1.3/B1.4 | Remove/ install ACM primary heat exchanger |  |  |  |  |  |
|  | B1.3/B1.4 | Remove /install temperature control valve |  |  |  |  |  |
|  | B1.3/B1.4 | Functionally test air cycle air conditioning system IAW AMM for evidence of serviceability or malfunction |  |  |  |  |  |
|  | B1.3/B1.4 | Remove and install combustion heaters and inspection checks in accordance with maintenance manual |  |  |  |  |  |
|  | B1.3/B1.4 | Remove/ install Vapor Cycle system blower motor assembly |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Perform compressor drive belt inspection and tension check in accordance with maintenance manual |  |  |  |  |  |
|  | B1.3/B1.4 | Test vapour cycle air conditioning system for correct function and freedom from refrigerant leaks |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install various temperature sensors of air-conditioning system and check operation |  |  |  |  |  |
|  | B1.3/B1.4 | Identify air-conditioning system MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot air-conditioning system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect cockpit / cabin emergency equipment for serviceability |  |  |  |  |  |
|  | B1.3/B1.4 | Check crew/pax seat belts for serviceability |  |  |  |  |  |
|  | B1.3/B1.4 | Replace crew seat |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install engine fire detector element IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform inspection of engine fire detecting elements IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform engine fire switch operation test IAW AMM |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Replace engine fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install cargo smoke detector unit IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform test of cargo smoke detection and warning system IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Identify fire and smoke detection system MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot fire and smoke detection system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Perform weight check on fire extinguisher bottles and replace, if necessary |  |  |  |  |  |
|  | B1.3/B1.4 | Identify hazards and discuss precaution and safety measures required during refuelling/defueling |  |  |  |  |  |
|  | B1.3/B1.4 | Perform aircraft daily fuel drain and fuel check |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install fuel tank access panels |  |  |  |  |  |
|  | B1.3/B1.4 | Replace and test fuel booster pump IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform refuelling and defueling |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Perform a fuel quantity indicating system test |  |  |  |  |  |
|  | B1.3/B1.4 | Removal / installation of fuel flow transmitter and test |  |  |  |  |  |
|  | B1.3/B1.4 | Removal / installation of fuel low pressure warning switch and test |  |  |  |  |  |
|  | B1.3/B1.4 | Identify fuel system MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot fuel system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Identify hazards and discuss precaution and safety measures required before pressurizing the hydraulic system |  |  |  |  |  |
|  | B1.3/B1.4 | Depressurize hydraulic reservoir, replenish hydraulic fluid, and recharge reservoir if required |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect hydraulic filters for foreign particles IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Remove/ install hydraulic pressure filter assembly |  |  |  |  |  |
|  | B1.3/B1.4 | Remove / install hydraulic return filter assembly |  |  |  |  |  |
|  | B1.3/B1.4 | Perform visual inspection of hydraulic filter bypass indicator |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Check/replenish hydraulic and/or pneumatic accumulator pressure. |  |  |  |  |  |
|  | B1.3/B1.4 | Perform operational test of hydraulic supply shutoff valve IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Removal / installation of hydraulic pressure switch / transmitter and test IAW AMM. |  |  |  |  |  |
|  | B1.3/B1.4 | Perform operational test of hydraulic system reservoir quantity indication IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Perform operational test of hydraulic system temperature indication IAW AMM |  |  |  |  |  |
|  | B1.3/B1.4 | Identify hydraulic system MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot hydraulic system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Remove and refit windshield wiper blades and check tension |  |  |  |  |  |
|  | B1.3/B1.4 | Identify anti-ice system MMEL Requirements |  |  |  |  |  |
|  | B1.3/B1.4 | Troubleshoot anti-ice system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.3/B1.4 | Service landing gear shock strut and perform dimension check |  |  |  |  |  |

E2 – Excluding (rotorcraft) mechanical or structural systems

| Index No. | Exclusion Category | E2 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.3/B1.4 | Perform landing gear retraction and extension operation check |  |  |  |  |  |
|  | B1.3/B1.4 | Replace aircraft wheels and brakes |  |  |  |  |  |
|  | B1.3/B1.4 | Inspect / replace landing gear skid shoes |  |  |  |  |  |
|  | B1.3/B1.4 | Perform operation check of landing gear indication system |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E3 – Category B1.1 / B1.2 Powerplant (Turbine)

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Fan blades and /or compressor inspection IAW AMM |  |  |  |  |  |
|  | B1.1 | Fan blades removal / installation IAW AMM |  |  |  |  |  |
|  | B1.1/ | Fan/ propellor spinner removal / installation IAW AMM |  |  |  |  |  |
|  | B1.1/ | Perform Fan /propellor spinner balance IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Perform zonal visual inspection engine case area (i.e. fan / compressor / turbine) |  |  |  |  |  |
|  | B1.1/B1.3 | Turbine blades inspection / check IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Inspect engine exhaust zone heat blankets |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/ install engine exhaust zone heat blankets |  |  |  |  |  |
|  | B1.1/B1.3 | Perform inspection engine exhaust nozzle |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Perform inspection engine exhaust nozzle |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine exhaust nozzle assembly |  |  |  |  |  |
|  | B1.1/ | Perform functional test engine thrust reverser assembly |  |  |  |  |  |
|  | B1.1/ | Perform engine thrust reverser rigging check |  |  |  |  |  |
|  | B1.1/ | Perform engine thrust reverser lock-out procedure |  |  |  |  |  |
|  | B1.1/ | Trouble shoot engine thrust reverser system fault. |  |  |  |  |  |
|  | B1.1/B1.3 | Discuss/review engine oil system layout and locate components |  |  |  |  |  |
|  | B1.1/B1.3 | Physical check of engine oil quantity level, refill to correct level, if necessary |  |  |  |  |  |
|  | B1.1/B1.3 | Carry out replacement of engine oil pressure filter IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Carry out replacement of engine oil scavenge filter IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Carry out inspection of engine oil filter IAW AMM |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Carry out visual inspection of engine chip detector IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Perform visual inspection of flexible engine oil hoses |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine oil pump IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Drain and replenish oil system. |  |  |  |  |  |
|  | B1.1/B1.3 | Remove and install fuel /oil coiler or air/oil cooler |  |  |  |  |  |
|  | B1.1/B1.3 | Perform Inspection of Oil Filter Bypass Indicator |  |  |  |  |  |
|  | B1.1/B1.3 | Discuss/review engine fuel system layout and locate components |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine fuel control unit/ hydro-mechanical unit IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Rig engine power lever / throttle control |  |  |  |  |  |
|  | B1.1/B1.3 | Perform a FADEC /EEC system BITE test |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine fuel flow transmitter |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Remove / install engine fuel pump IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Carry out inspection / replacement of engine fuel filter element |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine fuel spray nozzles |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine fuel manifolds |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install engine fuel flow divider |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / Install engine fuel heater |  |  |  |  |  |
|  | B1.1/B1.3 | Perform Inspection of Fuel Filter Bypass Indicator |  |  |  |  |  |
|  | B1.1/B1.3 | Perform functional test of engine fuel feed shut-off valve |  |  |  |  |  |
|  | B1.1/B1.3 | Discuss / review the bleed air system layout and locate the components |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / Install Nacelle Inlet Cowl Anti-Icing Shut-off Valve |  |  |  |  |  |
|  | B1.1/B1.3 | Perform functional test Nacelle Inlet Cowl Anti-Icing Shut-off Valve |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Remove / Install Bleed Air Shut Off Valve |  |  |  |  |  |
|  | B1.1/B1.3 | Troubleshoot bleed air system fault |  |  |  |  |  |
|  | B1.1/B1.3 | Removal / installation of Starter / Generator |  |  |  |  |  |
|  | B1.1/B1.3 | Starter / Generator – Functional Test |  |  |  |  |  |
|  | B1.1/B1.3 | Perform functional test engine starter relay |  |  |  |  |  |
|  | B1.1/B1.3 | Remove and install start control valve |  |  |  |  |  |
|  | B1.1/B1.3 | Remove and install air turbine starter |  |  |  |  |  |
|  | B1.1/B1.3 | Perform Troubleshooting Engine Start System Fault |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install ignition exciter box |  |  |  |  |  |
|  | B1.1/B1.3 | Perform ignitor plug inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install ignitor plug |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Perform Ignition lead inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install ignition lead |  |  |  |  |  |
|  | B1.1/B1.3 | Demonstrate replacement of engine exhaust gas temperature thermocouple / temperature sensors and test |  |  |  |  |  |
|  | B1.1/B1.3 | Demonstrate replacement of engine N1 / N2 tachometer and test |  |  |  |  |  |
|  | B1.1/B1.3 | Demonstrate replacement of engine fuel pressure or flow indication components and test |  |  |  |  |  |
|  | B1.1/B1.3 | Demonstrate replacement of engine oil pressure and temperature sensors and test |  |  |  |  |  |
|  | B1.1/B1.3 | Perform functional test on engine indication systems |  |  |  |  |  |
|  | B1.1/B1.3 | APU Powerplant – Remove/Install |  |  |  |  |  |
|  | B1.1/B1.3 | APU Electronic Control Unit – Remove/Install |  |  |  |  |  |
|  | B1.1/B1.3 | APU oil filter / fuel filter – Remove/Install |  |  |  |  |  |
|  | B1.1/B1.3 | APU magnetic chip detector visual inspection |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Perform APU compressor inlet visual inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU turbine exhaust outlet visual inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Trouble shoot APU system fault |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU EEC BITE test |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU trend monitoring procedure |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU operational test. |  |  |  |  |  |
|  | B1.1/B1.3 | Engine Mounts – Inspect/Check |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine drain line inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Engine Cowl – Remove/Install |  |  |  |  |  |
|  | B1.1/B1.3 | Engine – Remove/Install |  |  |  |  |  |
|  | B1.1/B1.3 | Remove and install engine fire detector elements IAW AMM |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Inspect engine fire detecting element for correct installation and damage |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine fire detection system functional test |  |  |  |  |  |
|  | B1.1/B1.3 | Perform weight check on fire extinguisher bottles and replace, if necessary |  |  |  |  |  |
|  | B1.1/B1.3 | Replace fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Inspect APU fire detecting elements for correct installation and damage IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU fire detecting systems test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Perform APU fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Trouble shoot APU system faults. |  |  |  |  |  |
|  | B1.1/B1.3 | Perform / discuss engine ground run preparation procedure |  |  |  |  |  |
|  | B1.1/B1.3 | Engine Ground Running – Dry Motoring Cycle |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Engine Ground Running – Wet Motoring Cycle |  |  |  |  |  |
|  | B1.1/B1.3 | Engine Ground Running – Low Power Test |  |  |  |  |  |
|  | B1.1/B1.3 | Engine Ground Running – High Power Test |  |  |  |  |  |
|  | B1.1/B1.3 | Trouble shoot various engine system fault. |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine oil sample collection for analysis |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine trend monitoring |  |  |  |  |  |
|  | B1.1/B1.3 | Perform engine hot section borescope inspection |  |  |  |  |  |
|  | B1.1/B1.3 | Carryout compressor washing and cleaning IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Review / discuss engine storage and back to service procedure |  |  |  |  |  |

E3 – Excluding (turbine) powerplant systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E3 – Category B1.2 / B1.4 Powerplant (Piston)

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Carry out annual/100-hour inspection of engine |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out cylinder differential compression checks, identify and rectify faults |  |  |  |  |  |
|  | B1.2/B1.4 | Remove and replace cylinder & piston assembly |  |  |  |  |  |
|  | B1.2/B1.4 | Perform piston ring end gap measure and adjust |  |  |  |  |  |
|  | B1.2/B1.4 | Replace cylinder base seals |  |  |  |  |  |
|  | B1.2/B1.4 | Replace push rod housing seals |  |  |  |  |  |
|  | B1.2/B1.4 | Replacement of rocker cover seals |  |  |  |  |  |
|  | B1.2/B1.4 | Remove, install, and replace induction manifold gaskets |  |  |  |  |  |
|  | B1.2/B1.4 | Perform engine inlet/outlet valve adjustment |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | For non-adjustable valves perform pushrod length change to achieve required valve lifter (hydraulic tappet) adjustment as per AMM |  |  |  |  |  |
|  | B1.2/B1.4 | Perform vacuum pump check |  |  |  |  |  |
|  | B1.2/B1.4 | Perform vacuum pump change |  |  |  |  |  |
|  | B1.2/B1.4 | Replace vacuum system inlet filter |  |  |  |  |  |
|  | B1.2/B1.4 | Perform a fuel injection system check |  |  |  |  |  |
|  | B1.2/B1.4 | Replace fuel system filter |  |  |  |  |  |
|  | B1.2/B1.4 | Remove, clean, and inspect fuel injectors |  |  |  |  |  |
|  | B1.2/B1.4 | Remove, inspect, and install fuel manifold |  |  |  |  |  |
|  | B1.2/B1.4 | Check and adjust carburettor float level |  |  |  |  |  |
|  | B1.2/B1.4 | Perform carburettor fuel mixer and idle RPM adjustment |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect carburettor heat controls for free operation and wear of the air box valve spindle and bushes |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Inspect engine control cables |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out removal, replacement and rigging of engine control cables |  |  |  |  |  |
|  | B1.2/B1.4 | Rig engine power lever |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine drive belts for wear, damage, and correct tension, carry out rectifications |  |  |  |  |  |
|  | B1.2/B1.4 | Perform magnetos adjustment (timing adjustments), switch and earth leads |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect flexible induction air hoses for condition |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine cooling fan drive for condition |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine cooling fan assembly for security, check for bearing condition, oil leaks |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine cooling fan shroud and baffles for security |  |  |  |  |  |
|  | B1.2/B1.4 | Remove and replace magneto |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out impulse coupling inspection check |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Carry out internal magneto timing and adjustment |  |  |  |  |  |
|  | B1.2/B1.4 | Conduct points gap inspection and adjustment |  |  |  |  |  |
|  | B1.2/B1.4 | Remove spark plugs, clean, test and reinstall. Check function |  |  |  |  |  |
|  | B1.2/B1.4 | Replace spark plug and check function (ground run) |  |  |  |  |  |
|  | B1.2/B1.4 | Perform ignition system test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove and replace ignition harness assembly |  |  |  |  |  |
|  | B1.2/B1.4 | Remove replace/repair HT leads |  |  |  |  |  |
|  | B1.2/B1.4 | Demonstrate cold cylinder test procedure |  |  |  |  |  |
|  | B1.2/B1.4 | Perform replacement of starter motor, inspect starter drive gear and ring gear teeth for wear/damage |  |  |  |  |  |
|  | B1.2/B1.4 | Check baffles for condition |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out replacement, repair, and installation of cooling baffles |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Inspect dipstick tube, oil filler neck and crankcase breather for signs of oil frothing paying attention to crankcase breather vent for blockages |  |  |  |  |  |
|  | B1.2/B1.4 | Check oil quantity level, refill to correct level, if necessary |  |  |  |  |  |
|  | B1.2/B1.4 | Perform engine oil change |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect screen type oil filter for metal particles, contaminants etc. |  |  |  |  |  |
|  | B1.2/B1.4 | Remove spin-on cartridge type oil filter, cut open, and inspect for metallic particles and signs of contamination |  |  |  |  |  |
|  | B1.2/B1.4 | Demonstrate checking and adjustment of oil pressure |  |  |  |  |  |
|  | B1.2/B1.4 | Demonstrate replacement of oil temperature sensor |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect oil cooler for damage. |  |  |  |  |  |
|  | B1.2/B1.4 | Check oil cooler vernatherm (oil cooler thermostat controller) for correct operation |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out oil cooler shutter and cowl flap adjustment |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect exhaust system for cracks or signs of leakage |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove and install exhaust manifold and gaskets. |  |  |  |  |  |
|  | B1.2/B1.4 | Remove cabin heat shroud, inspect for signs of exhaust leakage from muffler assembly |  |  |  |  |  |
|  | B1.2/B1.4 | Perform functional test on engine indication system |  |  |  |  |  |
|  | B1.2/B1.4 | Demonstrate replacement of exhaust temperature sensor |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine using borescope |  |  |  |  |  |
|  | B1.2/B1.4 | Assist in an engine removal & installation |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect, install all fluid and air carrying lines. |  |  |  |  |  |
|  | B1.2/B1.4 | Perform preservation procedures Prepare engine for shipment. |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine isolation mounts for condition and replace as required |  |  |  |  |  |
|  | B1.2/B1.4 | Inspect engine earthing cable for condition |  |  |  |  |  |
|  | B1.2/B1.4 | Assist in an engine test run-up, performance checks, and adjustments |  |  |  |  |  |

E3 – Excluding (piston) powerplant systems

| Index No. | Exclusion Category | E3 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Demonstrate test for Carbon monoxide |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out inspect damage to firewalls/ auxiliary firewalls and repair as required. |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E5 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Locate flight data recorder and cockpit voice recorder and perform tests |  |  |  |  |  |
|  | B1.1/B1.2 | Perform horizontal stabilizer electric trim system functional tests |  |  |  |  |  |
|  | B1.1/B1.2 | Perform functional test of stall warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform a fuel quantity indicating system test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Removal / installation of fuel flow transmitter and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Removal / installation of fuel low pressure warning switch and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot fuel system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify fuel system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear position indication proximity sensors / switches IAW AMM |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of landing gear position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear doors position indication proximity sensors / switches IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of landing gear doors position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install air-ground sensing proximity sensors / switches IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of air-Ground sensing and warnings |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify landing gear and doors position indication system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot landing gear and doors position indication system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test waste tank qty indication |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test potable water qty indication |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Trouble shoot water/waste system fault |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Use download data from onboard maintenance system for fault monitoring and troubleshooting |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Perform software load procedure on central maintenance system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Locate and identify Integrated Modular avionics systems controls and indications (if installed) |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / Install Bleed Air Pressure/temperature switch |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of engine exhaust gas temperature thermocouple / temperature sensors and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of engine N1 / N2 tachometer and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of engine fuel pressure or flow indication components and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of engine oil pressure and temperature sensors and test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test on engine indication systems |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform APU EEC BITE test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform engine fire detection system functional test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform APU fire detecting systems test IAW AMM |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Perform APU fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform engine fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of oil temperature sensor |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test on engine indication system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Demonstrate replacement of exhaust temperature sensor |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E6 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E6 – Excluding avionics LRUs

| Index No. | Exclusion Category | E6 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Isolate aircraft electrical system and fitted safety tags prior to maintenance in accordance with the AMM. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an independent Yaw Damper control module and then perform an operational interlock test. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a cockpit Audio Control Panel (ACP) and perform an audio loop and microphone selection test. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a VHF Communications transceiver LRU and perform a radio transmission and reception check. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a Passenger Address (PA) amplifier LRU and then perform a cabin chime priority override test. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove, install, and perform a system status download check on a crash-protected Flight Data Recorder (FDR) chassis. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove, install, and perform test on a Primary Flight Display (PFD) unit. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a VOR/ILS receiver unit and perform a localiser and glideslope test. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a Weather Radar receiver/transmitter, and perform a radiation safety zone cleared self-test |  |  |  |  |  |

E6 – Excluding avionics LRUs

| Index No. | Exclusion Category | E6 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a TCAS computer processor unit and perform a cockpit Resolution Advisory (RA) display test. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a MCDU/CDU and perform a keystroke entry and operation check. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a Central Maintenance Computer (CMC) and perform a configuration protocol load and fault-clear test. |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E9 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E9 – Excluding fabric surfaces

| Index No. | Exclusion Category | E9 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Inspect fabric skin visually for tautness, ripples, sagging, tears, and localized fabric rotting |  |  |  |  |  |
|  | B1.1/B1.2 | Assess UV degradation and environmental damage on upper wing surfaces using a magnifying glass. |  |  |  |  |  |
|  | B1.1/B1.2 | Perform fabric strength tests utilizing a calibrated Maule mechanical tester on designated test zones. |  |  |  |  |  |
|  | B1.1/B1.2 | Conduct ring-testing (tap-testing) across fabric surfaces to verify internal structural bonding integrity. |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect drainage grommets and ventilation holes for blockages, debris, and secure attachment. |  |  |  |  |  |
|  | B1.1/B1.2 | Evaluate internal airframe structures through inspection panels for signs of corrosion, wood rot, or structural deformation. |  |  |  |  |  |
|  | B1.1/B1.2 | Mark and remove defective sections of aircraft fabric without inflicting damage to the underlying structure. |  |  |  |  |  |
|  | B1.1/B1.2 | Clean and prepare metal and wooden structural members, applying approved anti-corrosive or sealing treatments. |  |  |  |  |  |
|  | B1.1/B1.2 | Apply anti-chafing tape over sharp edges, structural rivets, and prominent rib assemblies. |  |  |  |  |  |

E9 – Excluding fabric surfaces

| Index No. | Exclusion Category | E9 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Lay out and cut un-shrunk synthetic fabric, ensuring correct warp and weft orientation according to the manual |  |  |  |  |  |
|  | B1.1/B1.2 | Appy fabric cement and dope adhesive to the structural framework to secure the initial fabric layout |  |  |  |  |  |
|  | B1.1/B1.2 | Install a pre-sewn fabric envelope over a control surface, aligning seams exactly with structural edges |  |  |  |  |  |
|  | B1.1/B1.2 | Apply reinforcing tape over rib profiles along the full chord length prior to mechanical stitching |  |  |  |  |  |
|  | B1.1/B1.2 | Perform rib lacing (stitching) utilizing an approved modified Seine knot at standard, regulated spacing intervals. |  |  |  |  |  |
|  | B1.1/B1.2 | Secure fabric mechanically to the wing ribs using approved structural clips, screws, or specialized rivets |  |  |  |  |  |
|  | B1.1/B1.2 | Conduct thermal tautening of synthetic fabric utilizing a calibrated heat iron at designated temperature steps |  |  |  |  |  |
|  | B1.1/B1.2 | Brush the first matrix coat of nitrate dope into the fabric to guarantee total thread penetration |  |  |  |  |  |
|  | B1.1/B1.2 | Execute a doped-on patch repair over a simulated puncture using approved pinked-edge fabric patches |  |  |  |  |  |
|  | B1.1/B1.2 | Apply surface finishing tapes over structural seams, leading edges, and rib-laced paths |  |  |  |  |  |
|  | B1.1/B1.2 | Spray an ultraviolet-blocking coat (aluminium dope) followed by the final glossy protective topcoat |  |  |  |  |  |

E9 – Excluding fabric surfaces

| Index No. | Exclusion Category | E9 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Identify common tools for fabric covering structure repair |  |  |  |  |  |
|  | B1.1/B1.2 | Perform various inspections of fabric structure for damage IAW AMM. |  |  |  |  |  |
|  | B1.1/B1.2 | Evaluate different levels of damage classes of fabric structure IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform various repair and/or recover fabric on wing/ fuselage and/or control surface elements IAW AMM |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E10 – Category B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E10 – Excluding wooden structures

| Index No. | Exclusion Category | E10 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2 | Review the aircraft's logbook and Structural Repair Manual (SRM) to identify specific inspection zones and wear limits for a wood-structured wing or fuselage. |  |  |  |  |  |
|  | B1.2 | Perform a visual inspection of external fabric-covered plywood surfaces to detect signs of surface cracks, swelling, or underlying wood rot. |  |  |  |  |  |
|  | B1.2 | Use an inspection mirror and flashlight to check internal wing ribs and spars through internal access panels for compression fractures or splits. |  |  |  |  |  |
|  | B1.2 | Conduct a physical inspection of wood glue lines using a feeler gauge to detect delamination or joint separation. |  |  |  |  |  |
|  | B1.2 | Inspect internal drainage and water collection points in the lower fuselage or wings to ensure they are clear of dirt and blockages. |  |  |  |  |  |
|  | B1.2 | Mix a batch of approved structural wood adhesive (e.g., T-88 epoxy or Aerodux) according to the precise manufacturer-specified ratios. |  |  |  |  |  |
|  | B1.2 | Fabricate a test sample joint and perform a destruct sample test to confirm structural glue line strength and adhesion quality before applying it to the aircraft. |  |  |  |  |  |

E10 – Excluding wooden structures

| Index No. | Exclusion Category | E10 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2 | Cut out a damaged section of a wooden spar and prepare a standard 10:1 or 12:1 taper scarf joint according to approved maintenance data |  |  |  |  |  |
|  | B1.2 | Fabricate a new wooden replacement rib or gusset plate using template drawings and specialized aviation woodworking hand tools |  |  |  |  |  |
|  | B1.2 | Clamp a newly glued scarf repair member to a spar using protective wood blocks, ensuring correct and even clamping pressure across the entire glue line |  |  |  |  |  |
|  | B1.2 | Repair a punctured plywood skin surface by executing a flush patch or a splayed patch repair in accordance with the structural manual |  |  |  |  |  |
|  | B1.2 | Apply an approved protective sealer coating (e.g., marine-grade clear varnish or epoxy sealer) over a repaired internal wooden member to prevent future moisture ingress |  |  |  |  |  |
|  | B1.2 | Restore the exterior finishing scheme over a repaired plywood skin surface to match the surrounding fuselage or wing section |  |  |  |  |  |
|  | B1.2 | Perform various inspections of wooden structure for damage IAW AMM |  |  |  |  |  |
|  | B1.2 | Evaluate different levels of damage classes of wooden structure IAW AMM |  |  |  |  |  |
|  | B1.2 | Perform different wooden structure repairs IAW AMM |  |  |  |  |  |

E10 – Excluding wooden structures

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E12 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E12 – Excluding propellers

| Index No. | Exclusion Category | E12 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Isolate the aircraft electrical system, pull the propeller de-ice circuit breakers, and tag the engine control quadrant "Do Not Operate" |  |  |  |  |  |
|  | B1.1/B1.2 | Research the Aircraft Maintenance Manual (AMM) to locate the exact multi-stage torque values and indexing requirements for the hub |  |  |  |  |  |
|  | B1.1/B1.2 | Remove the spinner dome fasteners, unseat the dome, and inspect the internal structure for grease tracking or cracking |  |  |  |  |  |
|  | B1.1/B1.2 | Remove the propeller assembly off the engine shaft IAW the procedure in AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Clean the RGB output shaft flange and inspect the splines, threads, and sealing surfaces for fretting or corrosion |  |  |  |  |  |
|  | B1.1/B1.2 | Install the propellor assembly onto the engine shaft IAW the procedure in AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform a tactile and visual inspection on composite blade faces to locate delamination, matrix cracking, or lightning strip erosion |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect aluminium alloy blades/wooden propellor for stone bruises, nicks, or chemical pitting along the critical inboard trailing edge |  |  |  |  |  |

E12 – Excluding propellers

| Index No. | Exclusion Category | E12 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Conduct a dye penetrant inspection on a suspected surface indication near the hub blade retention area to verify structural integrity |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect the pitch-change cylinder yoke and blade retention bearings for evidence of grease leakage or seal displacement |  |  |  |  |  |
|  | B1.1/B1.2 | Blend out a minor trailing-edge nick within allowable AMM limits using a fine file and emery cloth, maintaining the original foil radius |  |  |  |  |  |
|  | B1.1/B1.2 | Treat a reworked aluminium blade section with chemical conversion coating (Alodine) and apply matching touch-up epoxy paint. |  |  |  |  |  |
|  | B1.1/B1.2 | Clean and re-torque loose spinner bulkhead balance weights, ensuring no alteration to the pre-existing balance profile occurs |  |  |  |  |  |
|  | B1.1/B1.2 | Measure the blade-to-blade tracking radius, logging values against the AMM limit |  |  |  |  |  |
|  | B1.1/B1.2 | Measure and record the blade feathering, reverse (beta), and fine-pitch angles using a digital universal propeller protractor |  |  |  |  |  |
|  | B1.1/B1.2 | Execute a dynamic balance ground run, calculate the required correction formula, and install precision weights to reduce IPS vibration levels |  |  |  |  |  |

E12 – Excluding propellers

| Index No. | Exclusion Category | E12 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Inspect the de-ice slip rings and carbon brush blocks for tracking alignment, cleanliness, and minimum allowable brush length |  |  |  |  |  |
|  | B1.1/B1.2 | Measure the electrical resistance and insulation continuity of individual blade de-ice heating elements using a megohmmeter |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect propeller de-icing system and components including propeller de-ice slip ring and brushes |  |  |  |  |  |
|  | B1.1/B1.2 | Replace propeller de-ice boot |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot a "sluggish feathering" report by checking the fluid levels and testing the electrical circuit of the auxiliary feathering pump |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot an uncommanded blade pitch hunt by checking the mechanical governor linkage rods for excessive play or worn rod-end bearings |  |  |  |  |  |
|  | B1.1/B1.2 | Perform an operational engine ground run to verify target maximum RPM governing limits and check the overspeed governor function |  |  |  |  |  |
|  | B1.1/B1.2 | Check and adjust the mechanical beta-ring clearance and feedback slip-ring gap to prevent unsafe pitch excursions in the beta range |  |  |  |  |  |
|  | B1.1/B1.2 | Diagnose a propeller oil leak by tracing the oil trail back to either the shaft seal, pitch change piston, or individual blade shank seals |  |  |  |  |  |

E12 – Excluding propellers

| Index No. | Exclusion Category | E12 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Troubleshoot a failure in the propeller synchrophasing system by measuring electrical signal outputs from the pulse generator probes |  |  |  |  |  |
|  | B1.1/B1.2 | Perform operational test of propeller electric / pneumatic de-icing system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform removal/installation and adjustment of governor system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform synchronisation adjustment of engine parameters on a multi- engine driven airplane |  |  |  |  |  |
|  | B1.1/B1.2 | Perform various inspections of propeller mounting bolts and mounting flange on engine |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect propeller counterweights for condition and security |  |  |  |  |  |
|  | B1.1/B1.2 | Perform propeller feathering check |  |  |  |  |  |
|  | B1.1/B1.2 | Inspect propeller blade clamps |  |  |  |  |  |
|  | B1.1/B1.2 | Perform propeller lubrication |  |  |  |  |  |
|  | B1.1/B1.2 | Perform propeller static and dynamic balancing |  |  |  |  |  |
|  | B1.1/B1.2 | Check propeller de-icing system and components |  |  |  |  |  |

E12 – Excluding propellers

| Index No. | Exclusion Category | E12 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Inspect propeller de-ice slip ring and brushes |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E13 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E13 – Excluding hydraulics – ATA29

| Index No. | Exclusion Category | E13 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Discuss/review replacement procedure for a hydraulic flight control actuator |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform inspection and operational test of speed brake system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify hazards and discuss precaution and safety measures required before pressurizing the hydraulic system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Depressurize hydraulic reservoir, replenish hydraulic fluid, and recharge reservoir if required |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace engine driven / electrical hydraulic pump |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect hydraulic filters for foreign particles IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove/ install hydraulic pressure filter assembly |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install hydraulic return filter assembly |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of hydraulic supply shutoff valve IAW AMM |  |  |  |  |  |

E13 – Excluding hydraulics – ATA29

| Index No. | Exclusion Category | E13 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Removal / installation of hydraulic pressure switch / transmitter and test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of hydraulic system reservoir quantity indication IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operational test of hydraulic system temperature indication IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Check/replenish hydraulic and/or pneumatic accumulator pressure |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform visual inspection of hydraulic filter bypass indicator |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify hydraulic system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot hydraulic system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E14 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E14 – Excluding vapour cycle air conditioning aspects of ATA21

| Index No. | Exclusion Category | E14 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a system receiver-dryer (filter-dehydrator) assembly and perform a system leak test and operational check to confirm moisture indicator status |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Drain and replenish the compressor lubricating oil to specified levels and perform a functional run to check for oil level stability and seal integrity |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a belt-driven or electrically driven vapour cycle compressor LRU and perform a compressor clutch engagement test and system pressure check |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a condenser heat exchanger assembly and perform an ambient air-flow check and system pressure differential test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an evaporator core unit within the cabin air distribution ducting and perform an operational check verifying duct temperature reduction |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a thermostatic expansion valve (TXV) and perform a system operational test to monitor superheat temperatures and frost line location |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an electric condenser cooling fan assembly and perform a functional test confirming correct rotation direction and airflow volume |  |  |  |  |  |

E14 – Excluding vapour cycle air conditioning aspects of ATA21

| Index No. | Exclusion Category | E14 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an air-mix temperature control valve or bypass flap actuator and perform a functional stroke test from the cockpit climate control panel |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a pressure transducer or transmitter system and then perform a calibration check against physical manifold gauge readings |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a damaged rigid or flexible refrigerant line/hose assembly and perform an electronic sniff test or fluorescent dye leak-detection test on all disturbed joints |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a compressor drive belt and perform a belt tension alignment check and structural inspection of the pulleys |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install the system recirculation or intake air filters and perform a cabin air delivery functional check to ensure unrestricted airflow. |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify air-conditioning system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot air-conditioning system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |

E14 – Excluding vapour cycle air conditioning aspects of ATA21

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E15 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E15 – Excluding air conditioning aspects of ATA21

| Index No. | Exclusion Category | E15 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an Air Cycle Machine (ACM) pack core unit, and then perform a system leak check and compressor/turbine operational check |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a primary air-to-air heat exchanger unit and then perform a structural inspection of the cooling air ducting and a system pressure test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a secondary heat exchanger unit within the cooling pack bay and perform a cooling pack functional airflow check |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a cooling pack water separator assembly and perform an operational check of the water spray nozzle system and a coalescer bag inspection |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a pneumatic Pack Cooling Control Valve (FCV) and perform an electrical command and dual-actuator stroke functional test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a turbine bypass / Temperature Control Valve (TCV) LRU and perform a cockpit temperature modulation functional test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an air conditioning pack bleed air PRV and perform a high-pressure pneumatic system operational test |  |  |  |  |  |

E15 – Excluding air conditioning aspects of ATA21

| Index No. | Exclusion Category | E15 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a ram air inlet/outlet flap actuator assembly and then perform a complete stroke-limit calibration check via the air conditioning controller |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a cabin air recirculation fan motor assembly, and perform an electrical load test and check for correct blade rotation direction |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a flight deck or cabin zone trim air valve and perform an individual climate zone temperature response test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install an avionics equipment cooling overboard exhaust valve and perform an automatic actuation and safety interlock test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a compressor discharge overheat temperature sensor switch and perform an electrical continuity test and loop system check |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install a cabin supply duct temperature sensor probe and verify accuracy against real-time central maintenance data displays |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Functionally test air cycle air conditioning system IAW AMM for evidence of serviceability or malfunction |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify air-conditioning system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot air-conditioning system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |

E15 – Excluding air conditioning aspects of ATA21

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E16 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E16 – Excluding pressurisation aspects of ATA21

| Index No. | Exclusion Category | E16 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove/install and Cabin Pressure Controller (CPC) LRU, and perform a system power-up check and BITE test via the flight deck panel |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install the main cabin pressure outflow valve assembly and perform an electrical manual-override actuator stroke check |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install cabin positive/negative pressure safety relief valve and perform a pneumatic override seal test |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install cabin altimeter and differential pressure (Delta P) gauge and perform a system static leak check |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install cabin altitude warning pressure switch and perform test to verify the 10,000ft cabin altitude aural warning trigger circuit |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install cockpit cabin pressure selector panel and perform test |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a forward or aft fuselage negative pressure relief flap valve and perform a physical hinge-freedom and seal alignment check |  |  |  |  |  |

E16 – Excluding pressurisation aspects of ATA21

| Index No. | Exclusion Category | E16 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove and install an electronics equipment bay cooling skin vent valve and perform an automatic pressurisation-interlock closure test |  |  |  |  |  |
|  | B1.1/B1.2 | Remove and install a ground exhaust ventilation valve and perform a ground/flight configuration changeover functional test. |  |  |  |  |  |
|  | B1.1/B1.2 | Check operation of pressurisation system in accordance with maintenance manual |  |  |  |  |  |
|  | B1.1/B1.2 | Identify pressurisation system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 | Troubleshoot faulty pressurisation Systems and rectify defects |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E32 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E32 – Excluding electrical systems in aircraft equipped with multi-generator powered systems

For E32 exclusion removal from B1 category licences use the task list for exclusion removal for E1 & E4 at the beginning of Part 2 of this logbook.

###### Exclusion E33 – Category B1.2 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E33 – Excluding all supercharging systems

| Index No. | Exclusion Category | E33 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install a supercharger impeller casing/housing assembly and perform an induction system static pressure test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install supercharger high/low speed gear-change clutch assembly and perform an engine ground run to verify positive clutch engagement |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install dedicated internal supercharger oil scavenge pump and run the engine to verify proper drainage and check for oil accumulation in the induction system |  |  |  |  |  |
|  | B1.2/B1.4 | Remove and install a splined supercharger main accessory drive shaft and then perform a backlash measurement check against engine overhaul manual limits |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install spring-loaded manifold over boost safety relief valve and perform a physical check to verify its pressure cracking point |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install high-pressure compressor discharge induction pipe and perform a localized joint inspection and seal check |  |  |  |  |  |

E33 – Excluding all supercharging systems

| Index No. | Exclusion Category | E33 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install charge-air intercooler heat exchanger core and perform an air-side restriction check and pressure leak test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install manual or automatic intercooler bypass flap valve and perform a cockpit control linkage rigging and stroke check |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install aneroid-controlled Automatic Boost Control unit and perform an operational ground run to check max static boost regulation limits |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the mechanical linkages connecting the Automatic Boost Control bellows to the carburettor/throttle butterfly valve and adjust and lock all rod-end clearances |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an induction alternate air door assembly and perform a manual override function check to verify unrestricted cockpit control lever movement |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/induction intake ducting to inspect a supercharger impeller wheel and use feeler gauges to document axial and radial blade-to-casing clearances against engine manual limits |  |  |  |  |  |
|  | B1.2/B1.4 | Perform a Full Supercharger Operational Ground Run, cycling the engine across its power ranges to verify stable manifold pressure regulation and ensure freedom from engine surging or boost fluctuations. |  |  |  |  |  |

E33 – Excluding all supercharging systems

| Index No. | Exclusion Category | E33 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Demonstrate turbocharger and waste gate test and adjustment |  |  |  |  |  |
|  | B1.2/B1.4 | Carry out removal and installation of turbocharger, conduct waste gate test and adjustment post install |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E35 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E35 – Excluding pressurised structures

| Index No. | Exclusion Category | E35 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform inspection; identify faulty rivet settings (aircraft structure) |  |  |  |  |  |
|  | B1.1/B1.2 | Remove defective rivets without causing further damage to skin |  |  |  |  |  |
|  | B1.1/B1.2 | Select and install oversize rivets as instructed by Structure Repair Manual (SRM) |  |  |  |  |  |
|  | B1.1/B1.2 | Set a range of different fasteners in aluminium sheet |  |  |  |  |  |
|  | B1.1/B1.2 | Discuss structure bonding methods, adhesive and sealant used, and inspection of bonded joints |  |  |  |  |  |
|  | B1.1/B1.2 | Perform visual inspection of aircraft structure and recognise for signs of deformation defects or damage like impact damage, fatigue cracking, and corrosion, in accordance with maintenance documentation |  |  |  |  |  |
|  | B1.1/B1.2 | Perform corrosion removal, assessment and reprotection on an aircraft part |  |  |  |  |  |
|  | B1.1/B1.2 | Perform an assessment of aircraft structure damage using the aircraft structural repair manual (SRM) and determine the repair scheme in accordance with SRM |  |  |  |  |  |

E35 – Excluding pressurised structures

| Index No. | Exclusion Category | E35 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Perform blending of damaged aircraft skin or structure within dimensional accuracy of± 0.010 in/ 0.25 mm |  |  |  |  |  |
|  | B1.1/B1.2 | Perform Zonal visual inspection of fuselage skin/ structure |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install main entry door |  |  |  |  |  |
|  | B1.1/B1.2 | Perform rigging procedure of main entry door |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install emergency exit door/hatch |  |  |  |  |  |
|  | B1.1/B1.2 | Check main doors and emergency exit doors seal and replace, if required |  |  |  |  |  |
|  | B1.1/B1.2 | Remove / install cockpit windshield |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/ install cabin window |  |  |  |  |  |

E35 – Excluding pressurised structures

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E36 – Category B1.2 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E36 – Excluding carburettor systems

| Index No. | Exclusion Category | E36 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install a float-type carburettor assembly and perform an engine ground run to verify standard power output and check for fuel leaks |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a pressure-injection carburettor core unit and run the engine to verify stable fuel-metering pressure across all power settings |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a carburettor fuel inlet screen/strainer assembly and perform a physical leak check under active booster pump pressure |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an internal carburettor mixture control needle or valve assembly and test idle cut-off operation during an engine shutdown |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the mechanical push-pull rods and spherical rod ends connecting the cockpit lever to the throttle butterfly shaft and check for positive full-travel stop contact |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the cockpit mixture control cable assembly to the carburettor input lever and verify clear cushion-to-stop clearances at both lean and rich limits |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a carburettor air intake heat box assembly and perform a cockpit control functional check to verify unhindered valve door travel |  |  |  |  |  |

E36 – Excluding carburettor systems

| Index No. | Exclusion Category | E36 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Rig and adjust a manual carburettor heat selector cable and perform an engine ground run to confirm a distinct RPM drop when hot air is selected |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a mechanical cylinder primer valve or distribution line and perform a fuel-tight pressure check |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a flexible or rigid fuel delivery hose connecting the engine-driven pump to the carburettor inlet and check for leaks during an engine ground run |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a fuel pressure sensor or transmitter line at the carburettor inlet and perform a system calibration verification against a master test gauge |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an inline fuel flow indicating transmitter module and verify cockpit flow readings during an operational engine test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a main induction air intake element or foam filter and check for unhindered induction airflow and correct intake sealing |  |  |  |  |  |
|  | B1.2/B1.4 | Access the interior of a carburettor float bowl to check the float level geometry and needle valve clearance against overhaul manual criteria, and then perform a static leak test upon reassembly |  |  |  |  |  |
|  | B1.2/B1.4 | Perform a Full Carburettor Engine Ground Run to adjust the idle speed screw and idle mixture needle, verifying a proper 25 to 50 RPM rise at idle cut-off |  |  |  |  |  |

E36 – Excluding carburettor systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E37 – Category B1.2 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E37 – Excluding fuel injection systems

| Index No. | Exclusion Category | E37 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install an engine-driven fuel injection pump and perform an engine ground run to check the high/low unmetered fuel pressure limits |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a fuel-air control assembly and run the engine to verify throttle response and check for casing fuel leaks |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a top-deck fuel manifold flow divider and perform a system pressurisation check to ensure uniform fuel distribution cutoff. |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a cylinder fuel discharge nozzle, and perform an induction-side fuel leak check and spray atomisation inspection |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the mechanical push-pull rods connecting the cockpit throttle lever to the FACU butterfly shaft, and verify positive full-travel stop contact |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the cockpit mixture control cable to the fuel injection unit lever, and verify clear cushion-to-stop clearances at both lean and rich limits |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an induction system alternate air door assembly, and perform a manual override function check to verify unrestricted cockpit control lever movement |  |  |  |  |  |

E37 – Excluding fuel injection systems

| Index No. | Exclusion Category | E37 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install a set of rigid, high-pressure fuel injector lines, and check for system leaks during an operational engine ground run |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a fuel-air control unit inlet screen/strainer assembly, and perform a physical leak check under active fuel boost pump pressure |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an inline fuel flow indicating transmitter module, and verify cockpit flow readings during an operational engine test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a fuel pressure sensor or transmitter line at the fuel manifold, and perform a system calibration verification against a master test gauge |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an electric auxiliary/boost fuel pump, and verify proper fuel pressure output and check for casing electrical circuit continuity |  |  |  |  |  |
|  | B1.2/B1.4 | Disconnect the fuel line terminations to perform a systematic Fuel System Flushing and Purging Procedure, and verify the removal of all trapped air pockets or contaminants from the metering paths |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a fuel return check valve or vapor separator line, and perform a system operational test to ensure stable fuel delivery pressures without vapor locking |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a set of fuel injector air-shrouds or clean the bleed air ambient vents, and perform an operational check to verify unrestricted atmospheric pressure reference |  |  |  |  |  |

E37 – Excluding fuel injection systems

| Index No. | Exclusion Category | E37 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install the induction gasket interface on a fuel-air control unit mounting flange, and perform a ground run using an induction leak-detection method to verify an absolute airtight seal |  |  |  |  |  |
|  | B1.2/B1.4 | Connect a master test gauge set to perform a Full Fuel Injection System Pressure and Flow Check, adjusting the pump internal relief valve to set precise unmetered fuel pressures at idle and full static RPM |  |  |  |  |  |
|  | B1.2/B1.4 | Perform a Fuel Injection Engine Ground Run to adjust the mechanical idle speed stop screw and the idle mixture knurled wheel, verifying a proper 25 to 50 RPM rise at idle cut-off |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E38 – Category B1.2 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E38 – Excluding turbo supercharging systems

| Index No. | Exclusion Category | E38 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install an exhaust-driven turbocharger core unit, and perform an engine ground run to check for casing oil leaks and turbine rotational binding |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a complete exhaust wastegate valve housing, and perform an operational check to verify smooth butterfly valve movement |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a hot-section exhaust transition pipe leading into the turbocharger turbine inlet, and perform a gas-tight seal inspection during an engine run |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a structural V-band clamp on the turbine exhaust exit housing, and torque the coupling to engine manual limits followed by a hot-retorque check |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a pneumatic density controller unit, and perform an operational engine run to verify max full-throttle deck pressure limits |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a differential pressure controller, and run the engine to check bootstrap regulation during rapid throttle transitions |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an absolute pressure controller unit, and perform a ground flight-power calibration test |  |  |  |  |  |

E38 – Excluding turbo supercharging systems

| Index No. | Exclusion Category | E38 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.2/B1.4 | Remove/install a hydraulic wastegate actuator piston cylinder, and bleed the system and check for smooth stroke movement |  |  |  |  |  |
|  | B1.2/B1.4 | Remov/ install a charge-air intercooler heat exchanger core, and perform an induction system pressure leak test |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a high-pressure induction discharge pipe (deck-pressure pipe), and perform an engine run to check all flexible hose couplers under boost conditions |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a mechanical compressor deck pop-off over boost safety valve, and perform a physical manual cracking-pressure check |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a rigid/braided turbocharger bearing oil feed line, and perform a static pressure leak check during an engine ground run |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install an engine-driven turbocharger oil scavenge pump, and run the engine to verify unrestricted oil drainage from the turbocharger centre housing |  |  |  |  |  |
|  | B1.2/B1.4 | Remove/install a high-temperature TIT thermocouple probe in the exhaust header, and perform an electrical loop test to verify instrument cockpit indication |  |  |  |  |  |
|  | B1.2/B1.4 | Rig and align the mechanical linkages between the wastegate actuator and butterfly valve, and then execute a Full Turbo-Supercharger Operational Ground Run to adjust the oil controller orifices and eliminate engine bootstrapping |  |  |  |  |  |

E38 – Excluding turbo supercharging systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E39 – Category B1.1 / B1.3

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E39 – Excluding airframe ice protection systems

| Index No. | Exclusion Category | E39 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1 | Remove/install a Wing Anti-Ice (WAI) pressure regulation and shut-off valve (PRSOV), and perform an engine bleed air ground test to verify pneumatic regulation |  |  |  |  |  |
|  | B1.1 | Inspect wing leading-edge double-walled telescopic bleed air ducting, and "piccolo tube" distribution duct, and perform pneumatic system leak check |  |  |  |  |  |
|  | B1.1 | Perform operational test of wing anti-ice system IAW AMM |  |  |  |  |  |
|  | B1.3 | Remove/install helicopter Main Rotor Blade (MRB) electro-thermal de-ice mat assembly, and perform a structural bonding vacuum bag check |  |  |  |  |  |
|  | B1.3 | Remove/install a main rotor head high-current slip ring assembly, and perform a brush-block contact alignment and spring tension measurement |  |  |  |  |  |
|  | B1.3 | Remove/install a rotary-wing Blade De-Ice Controller (BDIC) / distributor unit, and perform a cockpit test to verify correct asymmetrical symmetrical dwell times |  |  |  |  |  |
|  | B1.3 | Remove/install a rotor head droop stop or flap restrainer heater mat, and perform a circuit continuity and heating profile check |  |  |  |  |  |

E39 – Excluding airframe ice protection systems

| Index No. | Exclusion Category | E39 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Remove/install Windshield Temperature Control Unit (WTCU), and initiate a system BITE check to verify automatic trip parameters |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/install a cockpit side-window anti-fogging power relay or terminal junction block, and verify electrical circuit loop stability |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/install Pitot-Static probe, and perform an electrical current-draw amperage check via the cockpit indications |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/install Angle of Attack (AoA) alpha-vane assembly, and perform the operational check |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/install a Total Air Temperature (TAT) aspirated probe, and perform the operational check |  |  |  |  |  |
|  | B1.1/B1.3 | Remove/install engine anti-ice valve and perform operation test of engine thermal anti-ice system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Remove / install windshield wiper motor assembly IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Remove and install windshield wiper arm assembly IAW AMM |  |  |  |  |  |
|  | B1.1/B1.3 | Perform windshield rain repellent application |  |  |  |  |  |
|  | B1.1/B1.3 | Identify anti ice/de-ice system MMEL Requirements |  |  |  |  |  |

E39 – Excluding airframe ice protection systems

| Index No. | Exclusion Category | E39 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.3 | Troubleshoot anti ice/de-ice system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.3 | Troubleshoot anti-ice system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E40 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E40 – Excluding airframe fire protection systems

| Index No. | Exclusion Category | E40 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install engine fire detector element IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform inspection of engine fire detecting elements IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform engine fire switch operation test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace engine fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot engine fire detection system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install cargo smoke detector unit IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform test of cargo smoke detection and warning system IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify fire and smoke detection system MMEL Requirements |  |  |  |  |  |

E40 – Excluding airframe fire protection systems

| Index No. | Exclusion Category | E40 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot cargo fire and smoke detection system faults, rectify, and record the causes of the faults in maintenance documentation |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform weight check on fire extinguisher bottles and replace, if necessary |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Replace APU fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Inspect APU fire detecting elements for correct installation and damage IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform APU fire detecting systems test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform APU fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform engine fire extinguishing system test IAW AMM |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E41 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E41 – Excluding oxygen systems

| Index No. | Exclusion Category | E41 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Review oxygen systems warnings and safety precautions |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a high-pressure crew oxygen storage cylinder and regulator valve assembly, and perform a hydrostatic test date verification and system pressure test |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a cockpit quick-donning crew oxygen mask and regulator assembly, and perform an intra-cockpit microphone communication and positive-pressure breathing test |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a multi-mask Passenger Service Unit (PSU) chemical oxygen generator assembly, and perform a mechanical latch security and thermal insulation check |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install drop-down passenger service module oxygen mask and lanyard assembly, and verify correct deployment linkage and flow indicator operation |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a portable flight deck/cabin emergency oxygen bottle and mask assembly, and perform a physical pressure gauge check |  |  |  |  |  |

E41 – Excluding oxygen systems

| Index No. | Exclusion Category | E41 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Remove/install a cabin crew Protective Breathing Equipment (PBE) / smoke hood container assembly, and perform a visual moisture indicator check and expiration date verification |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a cabin first-aid oxygen therapeutic mask and flow selector valve, and verify multi-stage flow rate delivery limits |  |  |  |  |  |
|  | B1.1/B1.2 | Remove/install a fuselage-mounted high-pressure green oxygen overpressure blowout disc assembly, and check the structural seal and external discharge indicator pipe |  |  |  |  |  |
|  | B1.1/B1.2 | Connect an approved, oil-free high-pressure oxygen cart to execute a complete Aircraft Gaseous Oxygen System Recharging and Purging Procedure, monitoring temperature, and pressure curves to eliminate internal system moisture |  |  |  |  |  |
|  | B1.1/B1.2 | Perform a structural inspection on an oxygen cylinder storage cradle and structural retention straps, and check for proper torque limits |  |  |  |  |  |

E41 – Excluding oxygen systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E42 – Category B1.1 / B1.2 / B1.3 / B1.4

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E42 – Excluding landing gear retraction systems

| Index No. | Exclusion Category | E42 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear door assembly |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear extension retraction actuator |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove and install nose and main -landing-gear downclock sensor |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Prepare aircraft for landing gear retraction / extension operation |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform normal operation landing gear extension retraction procedure |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test emergency landing gear extension |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear position indication proximity sensors / switches IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of landing gear position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install landing gear doors position indication proximity sensors / switches IAW AMM |  |  |  |  |  |

E42 – Excluding landing gear retraction systems

| Index No. | Exclusion Category | E42 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of landing gear doors position indication and warning system |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Remove / install air-ground sensing proximity sensors / switches IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform functional test of air-Ground sensing and warnings |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Identify landing gear and doors position indication system MMEL Requirements |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Troubleshoot landing gear and doors position indication system and rectify defects |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform landing gear retraction and extension operation check |  |  |  |  |  |
|  | B1.1/B1.2 B1.3/B1.4 | Perform operation check of landing gear indication system |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E43 – Category B1.1 / B1.2

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E43 – Excluding fabric other than flight controls

| Index No. | Exclusion Category | E43 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/B1.2 | Demonstrate correct lacing in a section of fabric |  |  |  |  |  |
|  | B1.1/B1.2 | Construct a fabric patch including correct stitching |  |  |  |  |  |
|  | B1.1/B1.2 | Carry out doping in a suitable fabric area |  |  |  |  |  |
|  | B1.1/B1.2 | Identify common tools for fabric covering structure repair |  |  |  |  |  |
|  | B1.1/B1.2 | Perform various inspections of fabric structure for damage IAW AMM. |  |  |  |  |  |
|  | B1.1/B1.2 | Evaluate different levels of damage classes of fabric structure IAW AMM |  |  |  |  |  |
|  | B1.1/B1.2 | Perform various repair and/or recover fabric on wing/ fuselage and/or control surface elements IAW AMM |  |  |  |  |  |

E43 – Excluding fabric other than flight controls

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E44 – Category B1.1

Note:	Complete at least 80% of the tasks listed under the relevant subcategory exclusion to support removal of the exclusion applied for.

E44 – Excluding wiring repairs

| Index No. | Exclusion Category | E44 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/ | Identify broken airframe wire, strip insulation, crimp an environmental splice sleeve using calibrated tooling, and heat-shrunk the sleeve |  |  |  |  |  |
|  | B1.1/ | Strip wire insulation, crimp male/female contacts using a positioning turret tool and use an insertion tool to populate an electrical connector plug |  |  |  |  |  |
|  | B1.1/ | Strip high-frequency coaxial cable layers, position a connector sleeve, crimp the centre pin, and secure the outer shield grounding ring. |  |  |  |  |  |
|  | B1.1/ | Measure wire gauge, select a matching ring terminal lug, crimp the lug onto the wire conductor, and apply identification heat-shrink sleeve wire markers |  |  |  |  |  |
|  | B1.1/ | Prepare a shielded cable run, install a solder sleeve/shield terminator ring using a heat gun, and terminate the ground pigtail to the aircraft chassis |  |  |  |  |  |
|  | B1.1/ | Inspect main electrical junction box routing, identify wires chafing against structural brackets, and wrap affected looms with anti-chafing tape/expandable sleeving |  |  |  |  |  |
|  | B1.1/ | Use a plastic extraction tool to remove a damaged or corroded pin from a high-density rectangular connector block, and insert a newly terminated wire pin |  |  |  |  |  |

E44 – Excluding wiring repairs

| Index No. | Exclusion Category | E44 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1.1/ | Clean an airframe structural earth point, attach a component bonding strap, and use a milli-ohmmeter to verify electrical resistance was under AMM limits |  |  |  |  |  |
|  | B1.1/ | Conduct a detailed visual inspection of an engine nacelle wire harness loom, check for heat degradation or fluid ingress, and replace affected individual wires |  |  |  |  |  |
|  | B1.1/ | Re-route a repaired wire bundle through an airframe bulkhead, bind the new wires using specified nylon lacing cord (clove hitches/lock stitches), and secure stand-off clamps |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

Part 3
Exclusion(s) Removal
Maintenance Practical Experience

## Category B2 Avionics Licence

### Part 3 – Exclusion(s) Removal Maintenance Practical Experience

#### Category B2 Avionics Licence exclusion(s) removal task list

###### Exclusions E1, E4 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Use test meters to measure volts, amps, and resistance in practical task circumstances |  |  |  |  |  |
|  | B2 | Using a multimeter and wiring diagram, check an aircraft electrical circuit for continuity and resistance |  |  |  |  |  |
|  | B2 | Using a megger and wiring diagram, check an aircraft electrical circuit for insulation resistance |  |  |  |  |  |
|  | B2 | Read and interpret electrical wiring diagrams, including symbols used |  |  |  |  |  |
|  | B2 | Carry out bonding tests for equipment bonding, metallic surface bonding, and static bonds |  |  |  |  |  |
|  | B2 | Select and use appropriate wire stripping tools to strip various gauges of aircraft copper and aluminium wires |  |  |  |  |  |
|  | B2 | Select, crimp, and install terminal lugs on different gauges of copper and aluminium wires |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Select and splice pre- and post-insulated splices on different gauges of aircraft wires |  |  |  |  |  |
|  | B2 | Using approved documents, identify aircraft electrical connectors (class, contact style, insert arrangement, and polarisation) |  |  |  |  |  |
|  | B2 | Using approved documents, identify aircraft electrical terminal blocks (class, contact style, insert arrangement) |  |  |  |  |  |
|  | B2 | Insert and extract electrical contacts (pins and sockets) in a variety of electrical connectors using the correct insertion/extraction tools |  |  |  |  |  |
|  | B2 | Replace different types of aircraft electrical connectors |  |  |  |  |  |
|  | B2 | Select appropriate crimping tools and terminate pins/sockets in aircraft electrical connectors |  |  |  |  |  |
|  | B2 | Inspect coaxial cable installations for damage; correct cause if necessary |  |  |  |  |  |
|  | B2 | Check the integrity of coaxial cables using an ohmmeter |  |  |  |  |  |
|  | B2 | Inspect electrical looms/harnesses to ensure minimum bend radius is maintained, cables are not in tension, plugs are aligned, and insulation is not chafed |  |  |  |  |  |
|  | B2 | Demonstrate familiarity with aircraft wiring specifications and standards |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Replace aircraft wires by identifying the part number (or alternate) and wire type/class |  |  |  |  |  |
|  | B2 | Inspect wiring for damage tolerance and repair wiring damage per the aircraft wiring specification manual |  |  |  |  |  |
|  | B2 | Interpret and apply wire/cable identification and marking using direct and indirect marking methods |  |  |  |  |  |
|  | B2 | Prepare and terminate a wire loom per the standard wiring practices manual using crimping and/or soldering techniques |  |  |  |  |  |
|  | B2 | Inspect cables and wire bundles as installed through bulkhead holes |  |  |  |  |  |
|  | B2 | Identify, inspect, and replace wiring clamps used in high-vibration, low-vibration, and hot areas |  |  |  |  |  |
|  | B2 | Apply wiring protection techniques including sleeving and heat-shrink |  |  |  |  |  |
|  | B2 | Perform GVI of wiring installations at wheel bays, wings, engines, pylon, and nacelle areas |  |  |  |  |  |
|  | B2 | Perform wiring repairs at wheel bays and wing areas |  |  |  |  |  |
|  | B2 | Perform wiring repairs at engine and pylon areas |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform simple soldering tasks |  |  |  |  |  |
|  | B2 | Solder cables to single- and multi-pin connectors and inspect solder joints |  |  |  |  |  |
|  | B2 | Inspect the main aircraft battery installation for any cracks, leaks, swelling, or loose electrical plugs |  |  |  |  |  |
|  | B2 | Remove/refit main battery and perform system test per AMM |  |  |  |  |  |
|  | B2 | Perform aircraft battery capacity check |  |  |  |  |  |
|  | B2 | Inspect engine generator housing, electrical plugs, and wiring harnesses for loose connections, wire rubbing |  |  |  |  |  |
|  | B2 | Remove/install generator, alternator, or IDG and perform functional test per AMM |  |  |  |  |  |
|  | B2 | Perform operational test of the electrical load shedding system per AMM |  |  |  |  |  |
|  | B2 | Perform operational test of generator priority to AC bus per AMM |  |  |  |  |  |
|  | B2 | Remove/install generator control unit and perform functional tests per AMM |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove/install voltage regulators and perform functional tests per AMM |  |  |  |  |  |
|  | B2 | Remove/install transformer rectifier units (TRUs) and perform functional tests per AMM |  |  |  |  |  |
|  | B2 | Remove/install rotary inverter and/or static inverter and perform operational test per AMM |  |  |  |  |  |
|  | B2 | Remove/install electrical control panel and perform operational test per AMM |  |  |  |  |  |
|  | B2 | Replace cockpit circuit breakers using the Electrical Standard Wiring Practices Manual |  |  |  |  |  |
|  | B2 | Identify Electrical Power System MMEL requirements |  |  |  |  |  |
|  | B2 | Troubleshoot Electrical Power System faults and rectify defects |  |  |  |  |  |
|  | B2 | Replace external lights and check operation |  |  |  |  |  |
|  | B2 | Replace internal cabin lights and check operation |  |  |  |  |  |
|  | B2 | Perform operational test of passenger “No Smoking” and “Fasten Seat Belt” signs |  |  |  |  |  |
|  | B2 | Replace emergency lights power pack and test the system |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Replace and check operation of internal and external emergency lights |  |  |  |  |  |
|  | B2 | Identify MMEL requirements for internal/external and emergency lighting systems |  |  |  |  |  |
|  | B2 | Remove/install flight deck map/grimes light and check operation |  |  |  |  |  |
|  | B2 | Perform replacement of ovens, boilers, and beverage makers in the galley and test |  |  |  |  |  |
|  | B2 | Remove/install engine fire detection system components per AMM |  |  |  |  |  |
|  | B2 | Inspect engine fire detecting elements per AMM |  |  |  |  |  |
|  | B2 | Perform engine fire switch operational test per AMM |  |  |  |  |  |
|  | B2 | Replace engine fire extinguisher bottle squib and perform test |  |  |  |  |  |
|  | B2 | Remove/install cargo smoke detector unit per AMM |  |  |  |  |  |
|  | B2 | Test cargo smoke detection and warning system per AMM |  |  |  |  |  |
|  | B2 | Test cargo fire extinguishing system per AMM |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Test lavatory smoke detection and warning system |  |  |  |  |  |
|  | B2 | Troubleshoot engine fire warning and cargo/lavatory smoke detection systems and rectify defects |  |  |  |  |  |
|  | B2 | Replace and test a fuel boost pump per AMM |  |  |  |  |  |
|  | B2 | Remove/install propeller electrical de-ice boot |  |  |  |  |  |
|  | B2 | Perform operational test of propeller electric/pneumatic de-icing system per AMM |  |  |  |  |  |
|  | B2 | Perform operational test of electric/pneumatic de-icing boots on wing and horizontal stabiliser per AMM |  |  |  |  |  |
|  | B2 | Remove/install window heat control unit and test per AMM |  |  |  |  |  |
|  | B2 | Perform operational test of pitot/static/AOA sensor heating per AMM |  |  |  |  |  |
|  | B2 | Troubleshoot anti-ice/de-ice system faults and rectify defects |  |  |  |  |  |
|  | B2 | Remove/install nose and main landing gear downlock sensor |  |  |  |  |  |
|  | B2 | Troubleshoot nose wheel steering system fault |  |  |  |  |  |

E1, E4 – Excluding electrical systems

| Index No. | Exclusion Category | E1, E4 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove/install main wheel speed sensor per AMM |  |  |  |  |  |
|  | B2 | Perform functional test of anti-skid system per AMM |  |  |  |  |  |
|  | B2 | Perform functional test of autobrake system per AMM |  |  |  |  |  |
|  | B2 | Remove/install toilet flush control unit and test |  |  |  |  |  |
|  | B2 | Inspect engine ignition leads and starter connections for frayed metal braiding, and loose mounting clamps |  |  |  |  |  |
|  | B2 | Remove/install starter/generator and perform a functional test |  |  |  |  |  |
|  | B2 | Remove/install engine starter relay and perform functional test |  |  |  |  |  |
|  | B2 | Remove/install engine start switch |  |  |  |  |  |

E1, E4 – Excluding electrical systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E5 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate flight data recorder and cockpit voice recorder and perform tests |  |  |  |  |  |
|  | B2 | Perform horizontal stabiliser electric trim system functional tests |  |  |  |  |  |
|  | B2 | Perform functional test of stall warning system |  |  |  |  |  |
|  | B2 | Perform a fuel quantity indicating system test. |  |  |  |  |  |
|  | B2 | Remove/install fuel flow transmitter and perform system test |  |  |  |  |  |
|  | B2 | Remove/install fuel low pressure warning switch and perform system test |  |  |  |  |  |
|  | B2 | Troubleshoot fuel system and rectify defects |  |  |  |  |  |
|  | B2 | Identify fuel system MMEL requirements |  |  |  |  |  |
|  | B2 | Remove / install landing gear position indication proximity sensors / switches IAW AMM |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform functional test of landing gear position indication and warning system |  |  |  |  |  |
|  | B2 | Remove/install landing gear doors position indication proximity sensors/switches IAW AMM |  |  |  |  |  |
|  | B2 | Perform functional test of landing gear doors position indication and warning system |  |  |  |  |  |
|  | B2 | Remove/install air-ground sensing proximity sensors/switches IAW AMM |  |  |  |  |  |
|  | B2 | Perform functional test of air-ground sensing and warning system |  |  |  |  |  |
|  | B2 | Identify landing gear and doors position indication system MMEL requirements |  |  |  |  |  |
|  | B2 | Troubleshoot landing gear and doors position indication system and rectify defects |  |  |  |  |  |
|  | B2 | Perform a functional test of the waste tank quantity indication system |  |  |  |  |  |
|  | B2 | Perform a functional test of the potable water quantity indication system |  |  |  |  |  |
|  | B2 | Troubleshoot a water/waste system fault and rectify defects |  |  |  |  |  |
|  | B2 | Use download data from the onboard maintenance system for fault monitoring and troubleshooting |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform software load procedure on the central maintenance system |  |  |  |  |  |
|  | B2 | Locate and identify integrated modular avionics systems controls and indications (if installed) |  |  |  |  |  |
|  | B2 | Remove/install bleed air pressure/temperature switch |  |  |  |  |  |
|  | B2 | Demonstrate replacement of engine exhaust gas temperature thermocouple/temperature sensors and test |  |  |  |  |  |
|  | B2 | Demonstrate replacement of engine N1/N2 tachometer and test |  |  |  |  |  |
|  | B2 | Demonstrate replacement of engine fuel pressure or flow indication components and test |  |  |  |  |  |
|  | B2 | Demonstrate replacement of engine oil pressure and temperature sensors and test |  |  |  |  |  |
|  | B2 | Perform functional test on engine indication systems |  |  |  |  |  |
|  | B2 | Perform APU EEC BITE test |  |  |  |  |  |
|  | B2 | Perform engine fire detection system functional test |  |  |  |  |  |
|  | B2 | Perform APU fire detection system test IAW AMM |  |  |  |  |  |

E5 – Excluding instrument subsystems of mechanical, powerplant or structural systems

| Index No. | Exclusion Category | E5 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform APU fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B2 | Perform engine fire extinguishing system test IAW AMM |  |  |  |  |  |
|  | B2 | Demonstrate replacement of oil temperature sensor |  |  |  |  |  |
|  | B2 | Perform functional test on engine indication system |  |  |  |  |  |
|  | B2 | Demonstrate replacement of exhaust temperature sensor |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E7 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect autopilot servo installations by checking mounting security, attachment hardware, structural condition |  |  |  |  |  |
|  | B2 | Inspect autopilot and autothrottle wiring looms for chafing, insulation damage, and proper bonding |  |  |  |  |  |
|  | B2 | Inspect yaw damper system components by checking connectors, pins, mounting security, and condition for corrosion or contamination |  |  |  |  |  |
|  | B2 | Remove and install autopilot servo units, reconnect all interfaces, and perform an operational test to verify correct control surface response |  |  |  |  |  |
|  | B2 | Remove and install yaw damper computer, reconnect system wiring, and perform a functional test to confirm system operation |  |  |  |  |  |
|  | B2 | Remove and install flight guidance computer and perform BITE test to confirm correct operation and absence of faults |  |  |  |  |  |
|  | B2 | Remove and install autopilot mode control panel and perform functional checks on all switches, knobs, and annunciators |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install autothrottle servo, rig linkage as required, and perform thrust command tracking test to verify correct operation |  |  |  |  |  |
|  | B2 | Troubleshoot autopilot or autothrottle system faults using TSM or onboard diagnostics to identify and rectify defective components |  |  |  |  |  |
|  | B2 | Rectify intermittent autopilot or autothrottle faults by inspecting connectors and wiring and repairing or replacing as required |  |  |  |  |  |
|  | B2 | Perform a full autopilot functional test by engaging all modes and verifying correct aircraft response and system operation |  |  |  |  |  |
|  | B2 | Carry out autopilot engagement and disengagement checks and confirm proper disengagement warning indications |  |  |  |  |  |
|  | B2 | Verify autopilot servo clutch operation by checking smooth engagement, disengagement, and slip limits |  |  |  |  |  |
|  | B2 | Perform flight director command checks and verify correct display response to system inputs |  |  |  |  |  |
|  | B2 | Perform autopilot disconnect warning test and confirm correct audio and visual alerts |  |  |  |  |  |
|  | B2 | Verify control wheel steering mode operation by engaging system and confirming proper aircraft response |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform autothrottle system test and verify engine thrust follows commanded settings accurately |  |  |  |  |  |
|  | B2 | Perform stabiliser trim system test and verify correct movement, direction, and operational limits |  |  |  |  |  |
|  | B2 | Perform Mach trim and rudder limiter system functional tests and verify correct system response |  |  |  |  |  |
|  | B2 | Perform stability augmentation system functional testing and verify inputs and outputs operate correctly |  |  |  |  |  |
|  | B2 | Perform autopilot BITE test using maintenance system and confirm system status and fault reports |  |  |  |  |  |
|  | B2 | Inspect autopilot wiring bonding and grounding points and verify proper electrical continuity |  |  |  |  |  |
|  | B2 | Verify autopilot trim runaway protection by confirming system interrupts trim in abnormal conditions |  |  |  |  |  |
|  | B2 | Assess dispatch limitations for autopilot, autothrottle, and yaw damper systems using MEL |  |  |  |  |  |
|  | B2 | Identify applicable auto flight system MMEL requirements and confirm compliance for dispatch |  |  |  |  |  |
|  | B2 | Inspect flight control sensors, transmitters, and wiring by checking installation, connectors, security, and condition |  |  |  |  |  |
|  | B2 | Inspect fly by wire computers and associated wiring for correct installation, cooling, and connector condition |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect trim system components including servo motors, clutches, and shear pins for condition and security |  |  |  |  |  |
|  | B2 | Inspect LVDTs and associated wiring looms for damage, wear, and correct installation |  |  |  |  |  |
|  | B2 | Remove and install flight control computers and perform system functional test to verify operation |  |  |  |  |  |
|  | B2 | Remove and install control surface position sensors and verify correct cockpit indications |  |  |  |  |  |
|  | B2 | Remove and install flap and slat position transmitters and perform indication system test |  |  |  |  |  |
|  | B2 | Remove and install angle of attack sensors and perform system functional test |  |  |  |  |  |
|  | B2 | Remove and install stall warning stick shaker and perform operational test to confirm functionality |  |  |  |  |  |
|  | B2 | Remove and install control column and rudder pedal position sensors and perform functional checks |  |  |  |  |  |
|  | B2 | Troubleshoot flight control and fly by wire system faults using maintenance diagnostics to identify issues |  |  |  |  |  |
|  | B2 | Analyse intermittent faults in flight control systems and verify system restoration after rectification |  |  |  |  |  |
|  | B2 | Perform stabiliser trim functional check and verify full range of operation and limits |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform flight control indication system test and verify accuracy of cockpit indications |  |  |  |  |  |
|  | B2 | Identify flight control system MMEL requirements and confirm compliance |  |  |  |  |  |
|  | B2 | Assess dispatch limitations for flight control systems in accordance with MEL |  |  |  |  |  |
|  | B2 | Remove and install fly-by-wire system components and perform functional tests |  |  |  |  |  |
|  | B2 | Troubleshoot faulty fly-by-wire systems and rectify identified defects |  |  |  |  |  |
|  | B2 | Verify control surface position indication accuracy by comparing actual and indicated positions |  |  |  |  |  |
|  | B2 | Perform system reversion mode testing to confirm correct operation between normal and alternate modes |  |  |  |  |  |
|  | B2 | Inspect cockpit display units for condition, secure mounting, and adequate cooling airflow |  |  |  |  |  |
|  | B2 | Inspect instrument panel wiring and connections for damage, security, and correct routing |  |  |  |  |  |
|  | B2 | Inspect warning and caution annunciators and verify correct operation of all indications |  |  |  |  |  |
|  | B2 | Remove and install EFIS display units and perform operational checks across all modes |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install flight data recorder and perform data recording integrity checks |  |  |  |  |  |
|  | B2 | Remove and install cockpit voice recorder and perform internal self-test to confirm operation |  |  |  |  |  |
|  | B2 | Inspect flight data recorder and verify underwater locator beacon battery condition and expiry date |  |  |  |  |  |
|  | B2 | Troubleshoot EFIS, EICAS, or ECAM faults using maintenance diagnostics to isolate and rectify faults |  |  |  |  |  |
|  | B2 | Analyse intermittent display or indication faults and verify system restoration after corrective action |  |  |  |  |  |
|  | B2 | Identify indicating and recording system MMEL requirements and confirm compliance |  |  |  |  |  |
|  | B2 | Perform full EFIS system functional check and verify all display indications are correct |  |  |  |  |  |
|  | B2 | Verify brightness and dimming control operation across full range |  |  |  |  |  |
|  | B2 | Perform display reversionary mode check and verify correct switching between systems |  |  |  |  |  |
|  | B2 | Perform crew alerting system checks and verify warnings and cautions are correctly displayed |  |  |  |  |  |
|  | B2 | Verify display system input signals from sensors and ensure consistency of indications |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform cockpit voice recorder microphone checks and verify audio quality |  |  |  |  |  |
|  | B2 | Perform flight data recorder parameter validation and confirm correct data capture |  |  |  |  |  |
|  | B2 | Verify aircraft data bus inputs such as ARINC or AFDX to display systems |  |  |  |  |  |
|  | B2 | Inspect display cooling fans and ensure airflow is unobstructed and functional |  |  |  |  |  |
|  | B2 | Perform comparison checks between standby and primary instruments to confirm accuracy |  |  |  |  |  |
|  | B2 | Remove and install air data computer and perform system functional test to confirm operation |  |  |  |  |  |
|  | B2 | Inspect pitot static probes and fuselage ports for blockage, damage, and condition |  |  |  |  |  |
|  | B2 | Remove and install pitot static probe assembly and perform leak test to verify system integrity |  |  |  |  |  |
|  | B2 | Remove and install ADIRU and perform alignment and operational checks |  |  |  |  |  |
|  | B2 | Inspect ADIRS installation including mounting, alignment, and cooling provisions |  |  |  |  |  |
|  | B2 | Troubleshoot ADIRU alignment or drift faults using onboard diagnostics |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Diagnose altimeter discrepancies using pitot static test equipment |  |  |  |  |  |
|  | B2 | Troubleshoot air data and navigation system faults and rectify identified issues |  |  |  |  |  |
|  | B2 | Identify navigation system MMEL requirements and confirm compliance |  |  |  |  |  |
|  | B2 | Remove and install temperature and angle of attack sensors and perform a system test |  |  |  |  |  |
|  | B2 | Remove and install compass and navigation display components and verify functionality |  |  |  |  |  |
|  | B2 | Perform pitot static system leak test and confirm no pressure loss within limits |  |  |  |  |  |
|  | B2 | Perform air data system calibration using appropriate test equipment |  |  |  |  |  |
|  | B2 | Verify altitude encoder accuracy against known reference values |  |  |  |  |  |
|  | B2 | Perform airspeed indication cross check against test equipment or standby systems |  |  |  |  |  |
|  | B2 | Test pitot heater operation and verify correct current draw |  |  |  |  |  |
|  | B2 | Perform ADIRU drift monitoring test and verify acceptable limits |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform inertial reference system alignment verification and confirm correct alignment time |  |  |  |  |  |
|  | B2 | Verify heading reference accuracy against known reference |  |  |  |  |  |
|  | B2 | Perform comparison between air data computer and inertial reference inputs |  |  |  |  |  |
|  | B2 | Perform altitude alerting system test and verify correct warning operation |  |  |  |  |  |
|  | B2 | Inspect static system lines and drain valves for condition and blockage |  |  |  |  |  |
|  | B2 | Perform barometric correction checks and verify correct system response |  |  |  |  |  |
|  | B2 | Verify total air temperature sensor operation and accuracy |  |  |  |  |  |
|  | B2 | Perform navigation data integrity checks across system interfaces |  |  |  |  |  |
|  | B2 | Inspect IMA cabinet structure, racks, and cooling systems for security and condition |  |  |  |  |  |
|  | B2 | Inspect IMA backplanes and connectors for bent pins, contamination, and damage |  |  |  |  |  |
|  | B2 | Remove and install CPIOM module and perform boot validation and system checks |  |  |  |  |  |

E7 – Excluding instrument aspects of avionic systems – ATA 22, 271 31, 34, and 42

| Index No. | Exclusion Category | E7 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install IMA power supply unit and verify redundancy operation |  |  |  |  |  |
|  | B2 | Remove and install IMA configuration module and perform software validation |  |  |  |  |  |
|  | B2 | Troubleshoot CPIOM module faults and rectify system issues |  |  |  |  |  |
|  | B2 | Evaluate MEL dispatch capability for IMA system failures |  |  |  |  |  |
|  | B2 | Apply MEL procedures for degraded avionics network operation |  |  |  |  |  |
|  | B2 | Locate and identify IMA system controls and cockpit indications |  |  |  |  |  |
|  | B2 | Perform integrated modular avionics system functional test and verify system operation |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E8 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect VHF/HF antenna base bonding joints to ensure continuous High Intensity Radiated Field (HIRF) shielding |  |  |  |  |  |
|  | B2 | Perform detailed visual inspection of electronic bay racking ground straps for HIRF protection integrity |  |  |  |  |  |
|  | B2 | Inspect radio coaxial cable outer braided shields for physical wear, breakage, or compromise of HIRF protection |  |  |  |  |  |
|  | B2 | Inspect flight deck control column Push-To-Talk (PTT) wiring paths for shielding degradation |  |  |  |  |  |
|  | B2 | Inspect Cabin Interphone and Passenger Address (PA) speaker wiring lines for proximity separation from high-power cables |  |  |  |  |  |
|  | B2 | Inspect Flight/Service Interphone jack points for physical moisture ingress and terminal corrosion |  |  |  |  |  |
|  | B2 | Check HF antenna coupler pressure seals and external bonding jumper connections for secure attachment |  |  |  |  |  |
|  | B2 | Locate and perform the physical removal and installation procedure for a VHF blade antenna |  |  |  |  |  |
|  | B2 | Remove and install a VHF communications transceiver in the main avionics bay |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a VHF Audio Control Panel (ACP) into the cockpit centre pedestal |  |  |  |  |  |
|  | B2 | Remove and install an HF antenna coupler in the unpressurised aircraft empennage compartment |  |  |  |  |  |
|  | B2 | Remove and install an HF high-power transceiver unit according to Aircraft Maintenance Manual (AMM) steps |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation profile for a long-wire or shunt HF antenna system |  |  |  |  |  |
|  | B2 | Remove and install a flight deck control column Push-To-Talk (PTT) or HOT MIC switch assembly |  |  |  |  |  |
|  | B2 | Remove and install a Passenger Address (PA) Amplifier assembly unit from its equipment rack position |  |  |  |  |  |
|  | B2 | Perform an operational test of the VHF communication system using local tower frequencies |  |  |  |  |  |
|  | B2 | Perform radio Vertical Standing Wave Ratio (VSWR) measurement testing on a VHF communication antenna line |  |  |  |  |  |
|  | B2 | Complete a radio VSWR measurement test on an HF antenna transmission line using a calibrated inline wattmeter |  |  |  |  |  |
|  | B2 | Perform a functional test of the VHF communication system from the observer's audio station |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform operational functional testing of the long-range HF communication system across multiple bands |  |  |  |  |  |
|  | B2 | Perform post-installation system level calibration adjustments and audio level tests on a PA Amplifier |  |  |  |  |  |
|  | B2 | Execute formal operational testing procedures of the Flight Interphone system between captain and first officer |  |  |  |  |  |
|  | B2 | Complete functional testing parameters of the Service Interphone network from external ground power panels |  |  |  |  |  |
|  | B2 | Execute an operational test profile of the Cabin Interphone system across all flight attendant crew stations |  |  |  |  |  |
|  | B2 | Perform an operational functional clarity test of the aircraft Passenger Address (PA) loudspeaker system |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty VHF communications system exhibiting high background noise levels and rectify defects |  |  |  |  |  |
|  | B2 | Isolate an HF communication "fail to tune" error symptom down to a component fault and rectify the problem |  |  |  |  |  |
|  | B2 | Troubleshoot an inoperative flight deck PTT switch line using digital multimeter wiring checks |  |  |  |  |  |
|  | B2 | Isolate a persistent background hum fault inside the Flight/Service Interphone network loop and correct the defect |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Diagnose an inoperative audio output sector on the Cabin Interphone/Passenger Address system and restore functionality |  |  |  |  |  |
|  | B2 | Identify Master Minimum Equipment List (MMEL) dispatch allowances for a partially inoperative VHF communications system |  |  |  |  |  |
|  | B2 | Determine MMEL operational relief parameters and long-range route limitations for an unserviceable HF transceiver system |  |  |  |  |  |
|  | B2 | Identify flight crew operational restrictions and MMEL mandates for an inoperative Flight Interphone circuit |  |  |  |  |  |
|  | B2 | Review MMEL document requirements for a failed aircraft Cabin Interphone system network link |  |  |  |  |  |
|  | B2 | Evaluate Passenger Address (PA) system failure criteria against MMEL requirements to draft correct cockpit deferral tags |  |  |  |  |  |
|  | B2 | Inspect Emergency Locator Transmitter (ELT) remote switch shield paths to ensure protection from external HIRF interference |  |  |  |  |  |
|  | B2 | Inspect Cockpit Voice Recorder (CVR) structural housing, acoustic beacon attachment, and thermal insulation shields |  |  |  |  |  |
|  | B2 | Inspect high-sensitivity VOR antenna elements for symmetry, tip erosion, and structural grounding path security |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Check ADF combined loop/sense antenna outer surface for structural delamination and perimeter seal cracks |  |  |  |  |  |
|  | B2 | Inspect Marker Beacon antenna external low-profile casing for impact debris pitting or moisture entry points |  |  |  |  |  |
|  | B2 | Check navigation radio racking structural bonding points to ensure low electrical bond resistance values |  |  |  |  |  |
|  | B2 | Remove and install an Emergency Locator Transmitter (ELT) main processing unit in the fuselage |  |  |  |  |  |
|  | B2 | Remove and install a Cockpit Voice Recorder (CVR) assembly unit using designated safety lock pins |  |  |  |  |  |
|  | B2 | Remove and install a VOR navigation receiver module inside the primary electronics bay rack |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation procedure for an aircraft VOR navigation system antenna |  |  |  |  |  |
|  | B2 | Remove and install an Automatic Direction Finder (ADF) navigation receiver module unit |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation technical procedure for an ADF system antenna assembly |  |  |  |  |  |
|  | B2 | Remove and install a Marker Beacon receiver module inside the standard avionics’ equipment shelf |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for a Marker Beacon system antenna |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform required operational and safety self-test procedures on an ELT system using the remote cabin toggle |  |  |  |  |  |
|  | B2 | Execute a comprehensive operational functional test on a CVR system via the flight deck test panel interface |  |  |  |  |  |
|  | B2 | Perform formal testing of a VOR navigation receiver system using specialised ramp test equipment (e.g., Nav 401/402) |  |  |  |  |  |
|  | B2 | Verify VOR bearing track deviation accuracy limits across 90-degree points using an RF signal simulator test set |  |  |  |  |  |
|  | B2 | Perform functional testing of an ADF navigation receiver system using an avionics ramp test set generator |  |  |  |  |  |
|  | B2 | Complete functional testing on a Marker Beacon navigation receiver using an appropriate RF test equipment set |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty ELT unit displaying an early digital self-test failure indication and replace internal batteries |  |  |  |  |  |
|  | B2 | Isolate a CVR system showing an unserviceable bias monitor indication flag and rectify the underlying defect |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty VOR receiver navigation system showing excessive course deviation error flags and fix the issue |  |  |  |  |  |
|  | B2 | Diagnose an ADF navigation system showing a sluggish pointer response or weak station identification audio tracking |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a faulty Marker Beacon system showing zero light indications over a known inner or outer test signal |  |  |  |  |  |
|  | B2 | Identify flight dispatch limitations and specific MMEL parameters for an unserviceable ELT system |  |  |  |  |  |
|  | B2 | Review regulatory MMEL rules regarding operations with an inoperative or deactivated CVR assembly |  |  |  |  |  |
|  | B2 | Identify operational VOR navigation system capability limits against active MMEL route requirements |  |  |  |  |  |
|  | B2 | Review MMEL guidance documentation options for operating an aircraft with an inoperative ADF navigation unit |  |  |  |  |  |
|  | B2 | Identify Marker Beacon navigation system deferral allowances according to current MMEL manual requirements |  |  |  |  |  |
|  | B2 | Inspect Localiser (LOC) and Glide Slope (GS) antenna installations for structural fasteners and HIRF shield continuity |  |  |  |  |  |
|  | B2 | Inspect Distance Measuring Equipment (DME) blade antennas for structural security, skin cracking, and clean bonding |  |  |  |  |  |
|  | B2 | Check Global Positioning System (GPS) antenna low-profile radome skin surfaces for microscopic cracks |  |  |  |  |  |
|  | B2 | Verify ILS coaxial line routing layouts inside unpressurised bays to ensure separation from high-current power cables |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect DME transceiver tray alignment slots for structural security and locking latch wear points |  |  |  |  |  |
|  | B2 | Check GPS internal antenna grounding interface structures for corrosion or degradation of HIRF protection |  |  |  |  |  |
|  | B2 | Remove and install an Instrument Landing System (ILS) navigation receiver module unit |  |  |  |  |  |
|  | B2 | Locate and perform the removal and installation procedure for Localizer and Glide Slope system antennas |  |  |  |  |  |
|  | B2 | Remove and install a Distance Measuring Equipment (DME) interrogator transceiver unit |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation technical procedure for a DME system antenna |  |  |  |  |  |
|  | B2 | Remove and install an aircraft Global Positioning System (GPS) antenna in accordance with the Maintenance Manual (MM) |  |  |  |  |  |
|  | B2 | Test an ILS navigation receiver system for proper centre-line tracking using appropriate test sets (e.g., Nav 401/402) |  |  |  |  |  |
|  | B2 | Verify accurate cross-side ILS data transmission onto the Electronic Flight Instrument System (EFIS) flight deck displays |  |  |  |  |  |
|  | B2 | Perform testing of a DME interrogator transceiver unit using an appropriate calibrated avionics ramp test set |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Demonstrate familiarity with the GPS system interface with the Flight Management System (FMS) control panel |  |  |  |  |  |
|  | B2 | Access and view raw satellite tracking data, signal-to-noise ratios, and figure-of-merit values via the FMS/GPS page |  |  |  |  |  |
|  | B2 | Perform complete GPS system functional verification testing and cross-check primary position parameters |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty ILS (LOC & GS) navigation system showing persistent flag drops and rectify the defect |  |  |  |  |  |
|  | B2 | Isolate an intermittent Glide Slope signal loss problem down to a faulty RF diplexer component and replace it |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty DME interrogator system that fails to lock onto runway transponders and fix the unit |  |  |  |  |  |
|  | B2 | Diagnose a DME tracking calculation error issue using system self-test isolation checks |  |  |  |  |  |
|  | B2 | Perform GPS functional tests to identify faults related to slow acquisition or missing satellite position data locks |  |  |  |  |  |
|  | B2 | Identify precision approach category limitations and MMEL requirements for an inoperative ILS navigation system receiver |  |  |  |  |  |
|  | B2 | Review active MMEL compliance requirements for flying with an inoperative DME interrogator transceiver unit |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Identify Required Navigation Performance (RNP) flight operational restrictions under MMEL parameters for a failed GPS sensor |  |  |  |  |  |
|  | B2 | Inspect Air Traffic Control (ATC) transponder blade antennas for lightning strike damage and skin ground bonding |  |  |  |  |  |
|  | B2 | Inspect Traffic Collision Avoidance System (TCAS) directional antenna structural mounting surfaces for secure fasteners |  |  |  |  |  |
|  | B2 | Inspect Weather Radar transceiver wave-guide connections for physical warping, alignment, and internal cleanliness |  |  |  |  |  |
|  | B2 | Check Weather Radar nose radome internal composite skin for structural moisture ingress or HIRF mesh grid damage |  |  |  |  |  |
|  | B2 | Inspect Radio Altimeter (RadAlt) antenna low-profile faces for structural debris pitting or paint contamination |  |  |  |  |  |
|  | B2 | Verify surveillance and altimetry system cable routing paths maintain appropriate separation distances from high-power paths |  |  |  |  |  |
|  | B2 | Inspect TCAS processing unit tray locking mechanism devices for physical cracks and structural security |  |  |  |  |  |
|  | B2 | Remove and install an Air Traffic Control (ATC) transponder processing unit inside the main electronics bay |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and perform the removal and installation procedure for an ATC transponder system antenna |  |  |  |  |  |
|  | B2 | Remove and install a Traffic Collision Avoidance System (TCAS) computer processing unit module |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation technical procedure for a TCAS directional antenna |  |  |  |  |  |
|  | B2 | Remove and install an aircraft Weather Radar transceiver processing unit inside the nose bay compartment |  |  |  |  |  |
|  | B2 | Remove and install a directional Weather Radar antenna drive array plate assembly |  |  |  |  |  |
|  | B2 | Remove and install a Radio Altimeter transceiver unit inside the lower avionics bay tracking shelves |  |  |  |  |  |
|  | B2 | Locate and execute the removal and installation procedure for a Radio Altimeter transmit or receive antenna |  |  |  |  |  |
|  | B2 | Perform testing on an ATC transponder system across Mode S address fields using an appropriate flightline test set |  |  |  |  |  |
|  | B2 | Complete operational functional testing on a TCAS processing system using an appropriate transponder/TCAS test set |  |  |  |  |  |
|  | B2 | Execute a full operational functional test profile on the Weather Radar transceiver system using safe-distance profiles |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform testing on a Radio Altimeter system across variable altitude steps using an appropriate test set (e.g., 555 altimeter test set) |  |  |  |  |  |
|  | B2 | Perform flight deck target presentation verification checks on the TCAS traffic display screen interface |  |  |  |  |  |
|  | B2 | Verify functional operation of the Weather Radar antenna stabilisation system across pitch and roll changes |  |  |  |  |  |
|  | B2 | Check Radio Altimeter zero-altitude system calibration factors on the primary flight displays |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty ATC transponder system displaying altimetry data encoding mismatches and rectify the problem |  |  |  |  |  |
|  | B2 | Isolate a TCAS system showing an internal directional antenna connection error fault code and rectify the wire fault |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty Weather Radar system exhibiting sweeping sector display dropouts and repair the defect |  |  |  |  |  |
|  | B2 | Diagnose an unserviceable Weather Radar antenna tilt drive motor mechanism using systemic circuit analysis |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty Radio Altimeter system displaying an intermittent flag drop at low altitudes and correct the fault |  |  |  |  |  |
|  | B2 | Identify airspace operational limits and MMEL requirements for an unserviceable ATC transponder system |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Review mandatory flight route restrictions and MMEL requirements for an inoperative TCAS computer processor |  |  |  |  |  |
|  | B2 | Identify daytime and nighttime operational relief limits and MMEL parameters for an inoperative Weather Radar system |  |  |  |  |  |
|  | B2 | Review auto-land category drop status and MMEL requirements for a failed Radio Altimeter transceiver system |  |  |  |  |  |
|  | B2 | Determine RVSM flight tracking operational limits when dispatching an aircraft under surveillance system MMEL deferral |  |  |  |  |  |
|  | B2 | Inspect Integrated Modular Avionics (IMA) main cabinet chassis structures for physical damage and cooling duct security |  |  |  |  |  |
|  | B2 | Perform detailed inspections of IMA fibre-optic connection lines for routing curve radius and clean interfaces |  |  |  |  |  |
|  | B2 | Inspect Inflight Entertainment System (IFE) seat electronic box installations for secure structural mounts |  |  |  |  |  |
|  | B2 | Check cabin area high-speed network distributions for shielding ground integrity to maintain HIRF protection |  |  |  |  |  |
|  | B2 | Inspect IMA core processing module outer grounding path structures to verify low resistance bonding limits |  |  |  |  |  |
|  | B2 | Locate, identify, and trace Integrated Modular Avionics (IMA) core network switches and hardware controls |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a Core Processing Input/Output Module (CPM) inside an IMA cabinet network assembly |  |  |  |  |  |
|  | B2 | Locate and swap an inoperative Inflight Entertainment (IFE) system seat-back display electronic unit |  |  |  |  |  |
|  | B2 | Remove and install a main cabin zone area network control manager box unit |  |  |  |  |  |
|  | B2 | Replace a damaged fibre-optic data bus patch line inside an IMA main distribution junction centre |  |  |  |  |  |
|  | B2 | Access flight deck maintenance panels to isolate IMA controls, diagnostic indices, and system fault pages |  |  |  |  |  |
|  | B2 | Run a comprehensive IMA system central processing unit verification test via the onboard maintenance computer |  |  |  |  |  |
|  | B2 | Perform a complete operational functional test of the passenger address speaker system across all seating zones |  |  |  |  |  |
|  | B2 | Perform a functional operational test of the cabin interphone system linking forward, mid, and aft galley units |  |  |  |  |  |
|  | B2 | Perform a comprehensive operational test of the central Inflight Entertainment system (IFE) distribution server |  |  |  |  |  |
|  | B2 | Verify operational system data throughput on the aircraft cabin passenger Wi-Fi connectivity distribution network |  |  |  |  |  |
|  | B2 | Complete system data link loop verification testing on an active IMA processing channel lane |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a cabin system software distribution error anomaly and rectify network routing defects |  |  |  |  |  |
|  | B2 | Isolate a local zone audio dropout fault on the passenger address system and repair the branch line |  |  |  |  |  |
|  | B2 | Diagnose an isolated power supply failure condition inside an IFE passenger seat block distribution module |  |  |  |  |  |
|  | B2 | Isolate an IMA data bus channel transmission communication error flag down to a dirty fibre-optic connection face |  |  |  |  |  |
|  | B2 | Troubleshoot an inoperative attendant handset unit inside the main cabin interphone grid network loop |  |  |  |  |  |
|  | B2 | Identify core system redundancy impact levels and MMEL allowances for an unserviceable IMA processor card module |  |  |  |  |  |
|  | B2 | Review emergency advisory announcement requirements and cabin system MMEL relief limits for an unserviceable PA sector |  |  |  |  |  |
|  | B2 | Identify specific crew operating procedures and MMEL constraints when dispatching with a failed cabin interphone station |  |  |  |  |  |
|  | B2 | Review commercial aircraft passenger configuration relief allowances inside the MMEL manual for an inoperative IFE setup |  |  |  |  |  |

E8 – Excluding radio aspects of avionics systems – ATA23, 34, 42 and 44

| Index No. | Exclusion Category | E8 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Complete official documentation entries and test cycles to clear an open cabin network system MMEL configuration hold |  |  |  |  |  |
|  | B2 | Perform flight deck target presentation verification checks on the TCAS traffic display screen interface |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E11 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E11 – Excluding audio CVR systems

| Index No. | Exclusion Category | E11 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect flight deck Audio Control Panel (ACP) toggle knobs, selector switches, and backlighting matrices for physical degradation or fluid contamination |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of internal cabin and flight deck Interphone jack assemblies for contact oxidation and debris ingress |  |  |  |  |  |
|  | B2 | Inspect Passenger Address (PA) speaker wiring loops and distribution amplifier chassis units for proper structural security and connection integrity |  |  |  |  |  |
|  | B2 | Inspect Cockpit Voice Recorder (CVR) structural housing, underwater acoustic beacon attachment, and thermal insulation shields |  |  |  |  |  |
|  | B2 | Remove and install a flight deck centre pedestal VHF Audio Control Panel (ACP) according to Aircraft Maintenance Manual (AMM) steps |  |  |  |  |  |
|  | B2 | Remove and install a passenger cabin layout Passenger Address (PA) Amplifier module from its designated avionic shelf location |  |  |  |  |  |
|  | B2 | Remove and install a Cockpit Voice Recorder (CVR) assembly unit using designated safety lock pins |  |  |  |  |  |

E11 – Excluding audio CVR systems

| Index No. | Exclusion Category | E11 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a flight deck control column Push-To-Talk (PTT) or HOT MIC switch assembly |  |  |  |  |  |
|  | B2 | Execute formal functional and audio clarity testing profiles on the Flight Interphone network linking captain and first officer stations |  |  |  |  |  |
|  | B2 | Complete an operational testing cycle of the Service Interphone network loop via external aircraft ground maintenance plug panels |  |  |  |  |  |
|  | B2 | Perform a post-installation system adjustment and comprehensive operational test of the aircraft Passenger Address (PA) system zones |  |  |  |  |  |
|  | B2 | Execute a comprehensive operational functional test on a CVR system via the flight deck test panel interface and monitor audio playback |  |  |  |  |  |
|  | B2 | Troubleshoot a faulty flight deck ACP channel showing intermittent mic audio transmission drops and rectify the wiring defect |  |  |  |  |  |
|  | B2 | Isolate a persistent background hum or electrical whine fault inside the Flight/Service Interphone network loop and correct the grounding issue |  |  |  |  |  |
|  | B2 | Diagnose an inoperative audio output zone sector on the Cabin Interphone/Passenger Address network loop and restore normal system functionality |  |  |  |  |  |
|  | B2 | Isolate a CVR system showing an unserviceable bias monitor indication flag and rectify the underlying defect |  |  |  |  |  |

E11 – Excluding audio CVR systems

| Index No. | Exclusion Category | E11 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Identify Master Minimum Equipment List (MMEL) flight crew operational restrictions and mandates for an inoperative Flight Interphone circuit |  |  |  |  |  |
|  | B2 | Review MMEL document requirements for a failed aircraft Cabin Interphone system network link to determine cabin crew communication alternatives |  |  |  |  |  |
|  | B2 | Evaluate Passenger Address (PA) system failure criteria against MMEL requirements to draft correct cockpit deferral tags and placards |  |  |  |  |  |
|  | B2 | Review regulatory MMEL rules regarding operations with an inoperative or deactivated CVR assembly before aircraft dispatch |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E18 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E18 – Excluding ADF systems

| Index No. | Exclusion Category | E18 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the lower fuselage ADF flush-mounted loop antenna outer radome shell for physical cracks, composite delamination, and impact erosion pitting |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the ADF loop antenna perimeter aerodynamic sealing compound to identify potential water ingress pathways |  |  |  |  |  |
|  | B2 | Examine the ADF receiver unit main handles, racking locking hooks, and rear backplane electrical pin connectors for physical damage or distortion |  |  |  |  |  |
|  | B2 | Inspect ADF system structural bonding jumper straps and antenna mounting plate attachment points for oxidation and fraying |  |  |  |  |  |
|  | B2 | Check ADF coaxial transmission lines for evidence of structural chafing, sharp bend radius violations, or insulation wearing near fuselage bulkheads |  |  |  |  |  |
|  | B2 | Remove and install an Automatic Direction Finder (ADF) navigation receiver module unit inside the primary avionics bay equipment rack |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft flush-mounted ADF loop antenna assembly |  |  |  |  |  |

E18 – Excluding ADF systems

| Index No. | Exclusion Category | E18 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Replace a damaged or contaminated coaxial RF connector terminating at the ADF sense or loop antenna interface cable |  |  |  |  |  |
|  | B2 | Perform required operational system self-test procedures on the ADF control head interface panel and verify immediate bearing needle response behaviour |  |  |  |  |  |
|  | B2 | Execute a comprehensive ADF system ramp test using an IFR 4000 or equivalent avionics test generator set to verify signal capture and track accuracy |  |  |  |  |  |
|  | B2 | Verify ADF audio output clarity and volume range capability for positive Morse code radio navigation beacon identifier recognition |  |  |  |  |  |
|  | B2 | Test ADF bearing pointer response speed and tracking behaviour during a simulated 180-degree station passage using an RF signal simulator |  |  |  |  |  |
|  | B2 | Troubleshoot an ADF navigation system exhibiting a sluggish, wandering, or frozen bearing pointer response, and isolate the underlying component defect |  |  |  |  |  |
|  | B2 | Isolate a persistent 180-degree ADF tracking ambiguity error or high-noise audio fault condition down to an unbonded component or open loop line |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) dispatch allowances for an inoperative ADF system to execute proper physical circuit deactivation, warning tag attachment, and cockpit placarding |  |  |  |  |  |

E18 – Excluding ADF systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E19 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E19 – Excluding VOR systems

| Index No. | Exclusion Category | E19 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect tail-mounted VOR/LOC "cat-whisker" antenna blades for physical symmetry, structural bending, and leading-edge tip erosion |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the VOR antenna structural mounting base and internal stabiliser rib attach points for stress cracking or corrosion |  |  |  |  |  |
|  | B2 | Examine the VOR navigation receiver tray alignment slots, lock-down handles, and rear connector pins for bending or physical distortion |  |  |  |  |  |
|  | B2 | Inspect VOR RF signal splitter/diplexer chassis boxes and structural bonding jumper straps for secure attachment and low electrical bond resistance |  |  |  |  |  |
|  | B2 | Check VOR coaxial cable transmission lines for secure clamp layout, correct bend radius, and clear separation from high-power wiring |  |  |  |  |  |
|  | B2 | Remove and install a VOR navigation receiver module unit from its standard avionics bay equipment shelf position |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft VOR navigation system antenna assembly |  |  |  |  |  |

E19 – Excluding VOR systems

| Index No. | Exclusion Category | E19 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Replace a defective or corroded RF signal splitter/diplexer unit located in the VOR antenna line in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Perform an internal VOR system self-test via the flight deck panel and verify proper TO/FROM flag transitions on the navigation display |  |  |  |  |  |
|  | B2 | Execute a VOR system accuracy alignment test across all four tracking quadrants using a specialised avionics ramp test set (e.g., Nav 401/402) |  |  |  |  |  |
|  | B2 | Verify VOR audio identification clarity and volume output levels to ensure positive Morse code station recognitionhttps://www.scribd.com/document/208557238/Very-High-Frequency-Omnidirectional-Rangehttps://studentpilot.com/2003/11/01/lost-arts-1-fss-communications/https://forums.flightsimulator.com/t/vor-range-often-incorrect/406277 |  |  |  |  |  |
|  | B2 | Verify cross-side VOR navigation data transfer integrity onto the Electronic Flight Instrument System (EFIS) co-pilot display panel |  |  |  |  |  |
|  | B2 | Troubleshoot a VOR navigation system displaying an excessive course deviation error beyond the allowable +/- 4 degrees limit and rectify the fault |  |  |  |  |  |
|  | B2 | Isolate an intermittent VOR "NAV warning flag" drop symptom down to a loose coaxial cable shield or faulty RF diplexer component and replace it |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) flight dispatch criteria under Instrument Flight Rules (IFR) for an unserviceable VOR system, and execute circuit breaker isolation, warning tags, and cockpit placarding |  |  |  |  |  |

E19 – Excluding VOR systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E20 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E20 – Excluding ILS systems

| Index No. | Exclusion Category | E20 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect Localiser and Glide Slope antenna external structures for physical damage, hardware security, leading-edge erosion, and lightning strike evidence |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the Localiser and Glide Slope antenna base installations for corrosion, skin cracking, and correct aerodynamic sealant application |  |  |  |  |  |
|  | B2 | Examine Instrument Landing System (ILS) receiver mounting tray alignment racks, lock-down devices, and rear backplane electrical pin connectors for damage or contamination |  |  |  |  |  |
|  | B2 | Inspect ILS coaxial transmission line paths for proper security clamping, correct minimum bend radius compliance, and clear separation from high-current power cables |  |  |  |  |  |
|  | B2 | Verify that the ILS structural bonding jumper straps and antenna mounting interfaces maintain low electrical bond resistance to ensure continuous High Intensity Radiated Field (HIRF) shielding |  |  |  |  |  |
|  | B2 | Remove and install an Instrument Landing System (ILS) or Multi-Mode Receiver (MMR) navigation module within the primary avionics bay equipment rack |  |  |  |  |  |

E20 – Excluding ILS systems

| Index No. | Exclusion Category | E20 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft Localiser antenna assembly in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft Glide Slope antenna assembly according to the AMM steps |  |  |  |  |  |
|  | B2 | Replace a damaged or defective RF signal splitter, diplexer, or directional coupler unit located within the ILS coaxial antenna transmission lines |  |  |  |  |  |
|  | B2 | Fabricate or replace a damaged RF coaxial connector terminating at the ILS receiver or antenna interface cable |  |  |  |  |  |
|  | B2 | Perform an internal ILS system self-test via the flight deck control panel or onboard maintenance computer and verify display warning flags clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive ILS Localiser centre-line tracking and course deviation validation test using a specialised avionics ramp test set (e.g., Nav 401/402) |  |  |  |  |  |
|  | B2 | Execute a comprehensive ILS Glide Slope slope-line tracking and null-balance verification test using an approved RF flightline signal simulator |  |  |  |  |  |
|  | B2 | Verify accurate cross-side ILS navigation data transfer and flight director command bar display synchronisation onto both pilot and co-pilot primary flight instruments |  |  |  |  |  |

E20 – Excluding ILS systems

| Index No. | Exclusion Category | E20 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform an operational audio verification check to confirm the clarity and volume level of the ILS runway identifier station Morse code signalhttps://omb.report/icr/201910-2120-003/doc/96829601 |  |  |  |  |  |
|  | B2 | Troubleshoot an ILS navigation system displaying persistent or intermittent Localiser warning flags on the flight deck and rectify the underlying wiring or component defect |  |  |  |  |  |
|  | B2 | Isolate an erratic or fluctuating Glide Slope course deviation problem down to a faulty RF diplexer assembly or a loose coaxial cable shield connection and replace it |  |  |  |  |  |
|  | B2 | Diagnose a total loss of ILS audio identification with normal display tracking bars using systemic digital multimeter continuity and signal tracing checks |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) flight dispatch allowances and precision approach category drop status (e.g., CAT III to CAT I) for an unserviceable ILS receiverhttps://www.casa.gov.au/licences-and-certificates/air-operators/gaining-part-91-approvalhttps://www.tarmacview.com/glossary/cat-iii/ |  |  |  |  |  |
|  | B2 | Execute the mandatory MMEL operational deactivation procedures for an inoperative ILS channel, including circuit breaker mechanical isolation, warning tag attachment, and cockpit logbook placarding |  |  |  |  |  |
|  | B2 | Inspect Localiser and Glide Slope antenna external structures for physical damage, hardware security, leading-edge erosion, and lightning strike evidence |  |  |  |  |  |

E20 – Excluding ILS systems

| Index No. | Exclusion Category | E20 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Perform a detailed visual inspection of the Localiser and Glide Slope antenna base installations for corrosion, skin cracking, and correct aerodynamic sealant application |  |  |  |  |  |
|  | B2 | Examine Instrument Landing System (ILS) receiver mounting tray alignment racks, lock-down devices, and rear backplane electrical pin connectors for damage or contamination |  |  |  |  |  |
|  | B2 | Inspect ILS coaxial transmission line paths for proper security clamping, correct minimum bend radius compliance, and clear separation from high-current power cable |  |  |  |  |  |
|  | B2 | Verify that the ILS structural bonding jumper straps and antenna mounting interfaces maintain low electrical bond resistance to ensure continuous High Intensity Radiated Field (HIRF) shielding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E21 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E21 – Excluding weather radar systems

| Index No. | Exclusion Category | E21 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the aircraft weather radar (WXR) nose radome external composite skin for impact erosion pitting, lightning strike attachment points, or physical structural cracks |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the internal radome wall to check for moisture ingress, composite delamination, and damage to the High Intensity Radiated Field (HIRF) mesh or lightning diverter strips |  |  |  |  |  |
|  | B2 | Examine the weather radar antenna drive array gimbal mechanisms, stabilization springs, and primary structural mounting fasteners for secure attachment or mechanical wear |  |  |  |  |  |
|  | B2 | Inspect weather radar transceiver and receiver-transmitter (R/T) tray alignment racks, lock-down handles, and rear backplane electrical plug connectors for contamination or pin damage |  |  |  |  |  |
|  | B2 | Check weather radar high-frequency waveguide paths for physical dents, internal contamination, security of coupling hardware, or misalignment at structural joints |  |  |  |  |  |
|  | B2 | Remove and install a Weather Radar Receiver-Transmitter (R/T) processing unit within the primary nose bay avionic compartment in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |

E21 – Excluding weather radar systems

| Index No. | Exclusion Category | E21 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for a directional weather radar flat-plate antenna array drive assembly |  |  |  |  |  |
|  | B2 | Remove and install a dedicated flight deck weather radar control panel head unit or multi-function display select panel |  |  |  |  |  |
|  | B2 | Replace a damaged or defective high-frequency waveguide section, sealing gasket, or coaxial RF connector terminating at the weather radar R/T interface lines |  |  |  |  |  |
|  | B2 | Perform a weather radar internal system self-test via the flight deck maintenance interface and verify that display colour test pattern fields grid out cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive flight deck operational test of the weather radar system using safe-distance ground profiles to verify proper ground mapping and tilt range behaviour |  |  |  |  |  |
|  | B2 | Verify proper functional operation of the weather radar antenna stabilisation tracking loops across active pitch and roll changes using simulated flight instrument inputs |  |  |  |  |  |
|  | B2 | Troubleshoot a weather radar system displaying sweeping sector target presentation dropouts or line-blanking faults, and rectify the underlying component or antenna wiring defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable weather radar antenna tilt or scan drive motor malfunction down to a specific servo controller unit or broken drive wire and repair the assembly |  |  |  |  |  |

E21 – Excluding weather radar systems

| Index No. | Exclusion Category | E21 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Review Master Minimum Equipment List (MMEL) flight dispatch criteria for daytime or nighttime operations with an unserviceable weather radar system to execute mandatory circuit deactivation, warning tag placement, and cockpit placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E22 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E22 – Excluding ATC transponder systems

| Index No. | Exclusion Category | E22 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect Air Traffic Control (ATC) transponder lower and upper L-band blade antennas for lightning strike evidence, structural bonding degradation, and physical skin attachment security |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the transponder antenna base structural perimeters for corrosion, surface oxidation, or failure of the aerodynamic environmental sealant |  |  |  |  |  |
|  | B2 | Examine ATC transponder receiver-transmitter processing unit tray alignment racks, lock-down mechanical devices, and rear backplane pin connectors for physical distortion or pin contamination |  |  |  |  |  |
|  | B2 | Inspect transponder coaxial transmission lines for secure clamping layouts, structural bulkhead chafing protection, and proper minimum bend radius compliance |  |  |  |  |  |
|  | B2 | Verify that the transponder structural bonding jumper straps maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding boundarieshttps://lectromec.com/ica-guidance-for-hirf-and-lightning-protection/ |  |  |  |  |  |
|  | B2 | Remove and install an Air Traffic Control (ATC) transponder processing unit module within the primary electronics equipment bay rack |  |  |  |  |  |

E22 – Excluding ATC transponder systems

| Index No. | Exclusion Category | E22 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft transponder system L-band blade antenna assembly |  |  |  |  |  |
|  | B2 | Remove and install a flight deck centre pedestal ATC transponder mode selector control head or Multi-Function Control Display Unit (MCDU)https://www.scribd.com/document/473677165/34-12-00-at |  |  |  |  |  |
|  | B2 | Replace a damaged or defective coaxial RF connector terminating at the transponder processing unit or antenna interface cable |  |  |  |  |  |
|  | B2 | Perform an internal transponder system self-test via the flight deck panel and verify that all system warning flags or failure messages clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive ATC transponder ramp test across Mode A, Mode C, and Mode S address fields using a calibrated flightline test set (e.g., IFR 6000) |  |  |  |  |  |
|  | B2 | Verify the accuracy of the transmitted pressure altitude data link parameters against the primary flight deck altimetry instruments during ramp interrogation testing |  |  |  |  |  |
|  | B2 | Troubleshoot an ATC transponder system displaying persistent altitude reporting errors or data encoding mismatches, and rectify the underlying wire or sensor defect |  |  |  |  |  |
|  | B2 | Isolate an intermittent transponder signal dropout fault condition down to a faulty coaxial cable shield, a loose antenna connection, or a failing processing unit, and replace the component |  |  |  |  |  |

E22 – Excluding ATC transponder systems

| Index No. | Exclusion Category | E22 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Review Master Minimum Equipment List (MMEL) airspace operational limits and flight routing parameters for an unserviceable ATC transponder system to execute proper physical circuit deactivation, warning tag installation, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E23 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E23 – Excluding radio altimeter systems

| Index No. | Exclusion Category | E23 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the Radio Altimeter (RadAlt) transmit and receive antenna low-profile faces for external debris pitting, structural cracks, paint contamination, or lightning strike damage |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the RadAlt antenna mounting base perimeters for physical skin corrosion, moisture ingress, or degradation of the aerodynamic environmental sealant |  |  |  |  |  |
|  | B2 | Examine the radio altimeter transceiver tray alignment shelves, mechanical lock-down handles, and rear backplane multi-pin electrical connectors for distortion, wear, or pin contamination |  |  |  |  |  |
|  | B2 | Inspect the RadAlt coaxial transmission line installations for secure clamping layouts, proper structural bulkhead protection, and strict adherence to specific system cable length matching rules |  |  |  |  |  |
|  | B2 | Verify that the radio altimeter processing unit structural bonding straps and antenna base interfaces maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding bounds |  |  |  |  |  |

E23 – Excluding radio altimeter systems

| Index No. | Exclusion Category | E23 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a Radio Altimeter transceiver unit within the lower avionics bay tracking equipment shelves in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft Radio Altimeter transmit antenna assembly |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft Radio Altimeter receive antenna assembly |  |  |  |  |  |
|  | B2 | Replace a damaged or defective high-frequency coaxial RF connector terminating at the radio altimeter transceiver or antenna interface cable |  |  |  |  |  |
|  | B2 | Perform an internal radio altimeter system self-test via the flight deck maintenance interface or primary flight instruments and verify that all system failure flags clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive Radio Altimeter system validation test across variable altitude simulation steps using a specialised flightline test set (e.g., Aeroflex ALT-555 or ALT-8000) |  |  |  |  |  |
|  | B2 | Verify the accuracy of the radio altimeter zero-altitude (ground calibration) factor on the primary flight displays when the aircraft is in a normal ground-resting profile |  |  |  |  |  |

E23 – Excluding radio altimeter systems

| Index No. | Exclusion Category | E23 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a radio altimeter system displaying an intermittent flag drop or erratic altitude reading at low altitudes, and rectify the underlying component or antenna wiring defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable Rad Alt altitude transmission fault down to an internal transceiver failure, crosstalk interference, or a damaged coaxial cable shield line, and replace the componenthttps://paraorg.dfv.aero/uploads/manual_alti_2_englisch.pdf |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) automatic landing capability downgrades and flight operational restrictions for a failed radio altimeter system to execute mandatory circuit deactivation, warning tag placement, and cockpit placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E24 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E24 – Excluding DME systems

| Index No. | Exclusion Category | E24 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect Distance Measuring Equipment (DME) L-band blade antennas for structural security, lightning strike evidence, leading-edge erosion, and physical skin attachment security |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the DME antenna mounting base perimeters for physical skin corrosion, moisture ingress, or degradation of the aerodynamic environmental sealant |  |  |  |  |  |
|  | B2 | Examine DME interrogator transceiver tray alignment slots, lock-down mechanical handles, and rear backplane multi-pin electrical connectors for distortion, wear, or pin contamination |  |  |  |  |  |
|  | B2 | Inspect DME coaxial cable transmission line installations for secure clamping layouts, proper structural bulkhead protection, and correct minimum bend radius compliance |  |  |  |  |  |
|  | B2 | Verify that the DME processing unit structural bonding straps and antenna base interfaces maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding boundaries |  |  |  |  |  |
|  | B2 | Remove and install a Distance Measuring Equipment (DME) interrogator transceiver unit from the main avionics bay equipment rack |  |  |  |  |  |

E24 – Excluding DME systems

| Index No. | Exclusion Category | E24 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft DME system L-band blade antenna assembly |  |  |  |  |  |
|  | B2 | Remove and install a flight deck DME tuning selector head, control unit, or Multi-Function Control Display Unit (MCDU) interface |  |  |  |  |  |
|  | B2 | Replace a damaged or defective high-frequency coaxial RF connector terminating at the DME interrogator processing unit or antenna interface cable. |  |  |  |  |  |
|  | B2 | Perform an internal DME system self-test via the flight deck maintenance panel or primary flight instruments and verify that all system warning flags or failure messages clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive DME system validation test across variable distance and groundspeed simulation steps using a calibrated flightline test set (e.g., IFR 6000) |  |  |  |  |  |
|  | B2 | Verify accurate cross-side DME navigation data transfer and distance readout synchronisation onto both pilot and co-pilot primary flight displays |  |  |  |  |  |
|  | B2 | Troubleshoot a DME navigation system displaying a persistent distance calculation error, slow station lock-on, or intermittent warning flags, and rectify the underlying defect. |  |  |  |  |  |

E24 – Excluding DME systems

| Index No. | Exclusion Category | E24 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Isolate an intermittent DME signal dropout fault condition down to a faulty coaxial cable shield, a loose antenna connection, or a failing interrogator processing unit, and replace the component |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) flight dispatch compliance requirements and route restrictions for flying with an unserviceable DME interrogator system to execute proper physical circuit deactivation, warning tag installation, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E25 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E25 – Excluding Doppler systems

| Index No. | Exclusion Category | E25 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the fuselage lower-skin Doppler antenna radome assembly for physical cracks, environmental composite erosion, and structural fastener secureness |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the internal Doppler radar compartment to check for moisture ingress, structure pitting, or damage to the grounding path network |  |  |  |  |  |
|  | B2 | Examine the Doppler transceiver and transmitter-receiver (R/T) tray alignment tracking shelves, lock-down mechanical mechanisms, and rear backplane pin plugs for damage or contact contamination |  |  |  |  |  |
|  | B2 | Inspect high-frequency Doppler system coaxial transmission lines for physical wear, secure support clamping, and clear physical separation from high-current power cables |  |  |  |  |  |
|  | B2 | Verify that the Doppler processing hardware structural bonding jumper straps maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding boundaries |  |  |  |  |  |
|  | B2 | Remove and install a Doppler radar receiver-transmitter (R/T) processing unit within the lower electronics bay tracking rack according to the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |

E25 – Excluding Doppler systems

| Index No. | Exclusion Category | E25 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft flush-mounted Doppler radar antenna array assembly |  |  |  |  |  |
|  | B2 | Remove and install a flight deck centre pedestal Doppler system control indicator panel head unit or Multi-Function Control Display Unit (MCDU) interface |  |  |  |  |  |
|  | B2 | Replace a damaged or defective high-frequency coaxial RF connector terminating at the Doppler processing unit or antenna interface lines |  |  |  |  |  |
|  | B2 | Perform a Doppler radar internal system self-test via the flight deck maintenance display panel and verify that all system warning flags or failure messages clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive operational functional ground-test of the Doppler system using simulated flight environment data overrides to verify ground speed and drift calculation output |  |  |  |  |  |
|  | B2 | Verify proper structural tracking data loop transmission and data sharing from the Doppler processor onto the main Flight Management System (FMS) navigation pages |  |  |  |  |  |
|  | B2 | Troubleshoot a Doppler radar navigation system displaying persistent track acquisition memory hold locks or data display dropouts, and rectify the underlying component or antenna defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable Doppler system heading data bus communication error down to an open wiring fault line or a corrupt digital signal node, and repair the path |  |  |  |  |  |

E25 – Excluding Doppler systems

| Index No. | Exclusion Category | E25 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Review Master Minimum Equipment List (MMEL) dispatch allowances, route restrictions, and oceanic flight limitation rules for flying with an unserviceable Doppler navigation setup to execute proper physical circuit breaker deactivation, warning tag installation, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E26 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E26 – Excluding satellite navigation systems

| Index No. | Exclusion Category | E26 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the aircraft Global Positioning System (GPS) or Satellite Navigation (Sat Nav) low-profile antenna external radome skin surface for microscopic impact cracks, lightning strike attachment pathways, structural erosion, or mounting screw degradation. |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the internal Sat Nav antenna fuselage mounting location and doubling plates for structural corrosion, composite skin delamination, or hardware fastening looseness. |  |  |  |  |  |
|  | B2 | Examine Sat Nav and GPS receiver processing unit tray alignment structures, mechanical clamping levers, and rear backplane multi-pin electrical connectors for pin bending, physical distortion, or surface contamination. |  |  |  |  |  |
|  | B2 | Inspect high-frequency Sat Nav coaxial transmission lines for secure support clamp placement, strict adherence to minimum bend radius specifications, and clear physical separation from high-current aircraft power cables. |  |  |  |  |  |
|  | B2 | Verify that the satellite navigation receiver structural bonding jumper straps and antenna grounding interfaces maintain low electrical resistance values to preserve continuous High Intensity Radiated Field (HIRF) shielding bounds. |  |  |  |  |  |

E26 – Excluding satellite navigation systems

| Index No. | Exclusion Category | E26 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a Global Positioning System (GPS) or Global Navigation Satellite System (GNSS) receiver processing module inside the main electronics bay equipment rack in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft upper-fuselage satellite navigation or GPS antenna assembly |  |  |  |  |  |
|  | B2 | Remove and install a dedicated flight deck GPS control head unit, or replace a Multi-Function Control Display Unit (MCDU) interface linked to the Sat Nav system tracking data bus |  |  |  |  |  |
|  | B2 | Replace a damaged or defective high-frequency coaxial RF connector terminating at the satellite navigation processing unit or antenna interface transmission lines |  |  |  |  |  |
|  | B2 | Perform an internal satellite navigation system self-test via the cockpit display maintenance panel or Flight Management System (FMS) page, verifying all initial system failure flags clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive flight deck operational test of the Sat Nav system to view raw active satellite constellation tracking data, signal-to-noise ratios, and actual figure-of-merit position data metrics |  |  |  |  |  |
|  | B2 | Perform a complete functional validation check of the GPS/GNSS sensor system interface with the main Flight Management System (FMS) and multi-function navigation displays to verify tracking crosstalk alignment |  |  |  |  |  |

E26 – Excluding satellite navigation systems

| Index No. | Exclusion Category | E26 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a satellite navigation system displaying slow satellite position acquisition, intermittent position dropouts, or permanent navigation accuracy warning alerts, and rectify the underlying component or wire shielding defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable Sat Nav data bus communication failure or digital loop fault down to a corrupt ARINC 429 signal node, an open wiring path, or a dirty interface plug, and repair the path connection |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) Required Navigation Performance (RNP) flight operational restrictions and dispatch boundaries for an unserviceable Sat Nav sensor channel to execute proper physical circuit breaker isolation, warning tag attachment, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E27 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E27 – Excluding autopilot systems

| Index No. | Exclusion Category | E27 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect autopilot control panel switches, mode selectors, dual-channel engagement levers, and display lighting arrays for physical cracking, switch stickiness, or fluid contamination |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of automatic flight control computer racking assemblies, checking for secure frame grounding, tray structure integrity, and pin alignment |  |  |  |  |  |
|  | B2 | Examine autopilot primary elevator, aileron, and rudder servo actuator assemblies for housing security, mechanical fluid leaks, structural bracket cracking, and cable attach pin wear |  |  |  |  |  |
|  | B2 | Inspect autopilot automatic pitch trim bridle cables, capstan drive drums, and pulleys for proper structural tension limits, cable fraying, or wear points |  |  |  |  |  |
|  | B2 | Verify that automatic flight control system processing unit structural bonding straps maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding bounds |  |  |  |  |  |
|  | B2 | Remove and install an Autopilot Flight Director Computer (AFDC) or Flight Guidance Computer (FGC) unit within the primary electronics bay racking structure |  |  |  |  |  |

E27 – Excluding autopilot systems

| Index No. | Exclusion Category | E27 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for an autopilot primary flight control surface servo actuator assembly in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Remove and install a dedicated flight deck autopilot glare-shield control panel module or Flight Control Panel (FCP) assembly interface |  |  |  |  |  |
|  | B2 | Replace an unserviceable autopilot automatic disconnect switch or touch-control steering (TCS) button located on the pilot or co-pilot flight control column |  |  |  |  |  |
|  | B2 | Perform an autopilot system initialization test and automated Built-In Test Equipment (BITE) check via the flight deck maintenance display panel, verifying all fault codes clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive autopilot ground operational functional test to verify dual-channel command engagement, flight director bar alignment, and interlock loop safety protections |  |  |  |  |  |
|  | B2 | Verify proper automatic system axis disconnect response behaviour when tripping the control wheel manual interrupt switch or applying overriding control force to the columns |  |  |  |  |  |
|  | B2 | Troubleshoot an automatic flight control system displaying uncommanded pitch axis drifts, altitude hold tracking errors, or premature channel drops, and rectify the underlying wiring or component defect |  |  |  |  |  |

E27 – Excluding autopilot systems

| Index No. | Exclusion Category | E27 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Isolate an unserviceable autopilot heading select command tracking failure down to a corrupt digital bus line, a missing inertial reference unit input, or a faulty computer internal channel, and repair the path |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) automatic landing capability downgrades, Reduced Vertical Separation Minimum (RVSM) route restrictions, and dispatch boundaries for an unserviceable autopilot channel to execute proper physical circuit deactivation, warning tag attachment, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E28 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E28 – Excluding multi-axis autopilot systems

| Index No. | Exclusion Category | E28 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect multi-axis autopilot mode control panels, flight guidance selectors, and touchscreen display units for structural bezel cracks, tactile switch stickiness, or screen delamination |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the multi-axis flight control computer mainframes to verify structural racking security, clear cooling fan ducting, and proper pin alignment |  |  |  |  |  |
|  | B2 | Examine multi-axis elevator, roll, and yaw servo actuator motor assemblies for casing damage, physical structural bracket cracking, and linkage attachment security |  |  |  |  |  |
|  | B2 | Inspect automatic pitch-trim and yaw-damper cable systems, checking drive drums, pulleys, and bridle paths for proper structural tension limits, cable fraying, or wear points |  |  |  |  |  |
|  | B2 | Verify that the multi-axis flight guidance system structural bonding straps maintain low electrical resistance values to preserve High Intensity Radiated Field (HIRF) shielding boundaries |  |  |  |  |  |
|  | B2 | Remove and install a multi-axis Flight Guidance Computer (FGC) or Automatic Flight Control Computer (AFCC) unit within the primary electronic bay tracking racks |  |  |  |  |  |

E28 – Excluding multi-axis autopilot systems

| Index No. | Exclusion Category | E28 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Locate and execute the physical removal and installation procedure for a multi-axis primary flight control servo motor actuator or yaw damper assembly according to the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Remove and install a flight deck glare-shield Mode Control Panel (MCP) or Flight Control Panel (FCP) assembly unit |  |  |  |  |  |
|  | B2 | Replace an unserviceable autopilot manual disconnect bar switch, auto-throttle disconnect button, or touch-control steering (TCS) sensor switch located on the flight control column |  |  |  |  |  |
|  | B2 | Perform a multi-axis autopilot system initialisation test and automated Built-In Test Equipment (BITE) check, verifying all cross-channel fault tracking logs clear cleanlyhttps://www.scribd.com/document/362427655/287035970-B767-ATA-32-Student-Book-pdf |  |  |  |  |  |
|  | B2 | Execute a comprehensive ground operational functional test of the multi-axis autopilot system to verify pitch, roll, and yaw axis command engagement, cross-side flight director bar synchronisation, and interlock loop safety protections |  |  |  |  |  |
|  | B2 | Verify proper multi-axis system safety disconnect response profiles when activating the control wheel manual interrupt switch, triggering the stick shaker, or applying manual override breakout forces |  |  |  |  |  |
|  | B2 | Verify proper automatic interlock safety cut-out operations to ensure the multi-axis autopilot system is physically inhibited from engaging while the aircraft is in a manual go-around flight phase or when landing gear unsafe configurations are present |  |  |  |  |  |

E28 – Excluding multi-axis autopilot systems

| Index No. | Exclusion Category | E28 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a multi-axis flight control system displaying uncommented lateral deviations, roll axis oscillations, or premature dual-channel engagement drops, and rectify the underlying component or wiring defecthttps://intellaparts.com/blog/maintenance-service-insights-for-daewoo-doosan-2-0t-3-0t-series-3-lift-trucks/?srsltid=AfmBOooSsbB4s7t__E8--ST4-Jie0U9jLFBPP_0eyHpc06lrPG768w-W |  |  |  |  |  |
|  | B2 | Isolate an unserviceable multi-axis autopilot heading or altitude capture tracking failure down to a corrupt ARINC 429 digital bus line, a missing Air Data Inertial Reference Unit (ADIRU) input, or an internal processor channel fault, and repair the path |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) automatic landing capability downgrades, CAT III vs CAT I approach restrictions, and RVSM flight routing constraints for an unserviceable multi-axis autopilot channel to execute proper physical circuit deactivation, warning tag installation, and cockpit logbook placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E29 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E29 – Excluding remote indicating compass systems

| Index No. | Exclusion Category | E29 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect remote indicating compass flux valve wing-tip or tail-cone mounting cavities for physical structural damage, moisture accumulation, skin corrosion, and secure hardware fastening |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the remote directional gyro unit racking foundations, checking for frame mount cracking, anti-vibration shock absorber deterioration, and electrical grounding bond security |  |  |  |  |  |
|  | B2 | Examine flight deck compass indicator instruments, horizontal situation indicators (HSIs), and magnetic slave control panel faces for bezel cracking, glass delamination, and control knob mechanical stickiness |  |  |  |  |  |
|  | B2 | Inspect remote indicating compass system wiring interconnect lines and coaxial cable paths for secure attachment clamps, structural bulkhead protection grommets, and proper minimum bend radius compliance |  |  |  |  |  |
|  | B2 | Verify that the remote compass flux valve and directional gyro housing structural bonding straps maintain low electrical resistance values to preserve continuous High Intensity Radiated Field (HIRF) shielding bounds |  |  |  |  |  |

E29 – Excluding remote indicating compass systems

| Index No. | Exclusion Category | E29 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a remote indicating compass magnetic flux valve sensor assembly unit from its designated wingtip or tail-cone stabiliser mounting location |  |  |  |  |  |
|  | B2 | Remove and install a remote directional gyroscope processing unit from the main avionics bay equipment tracking racks in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Remove and install a flight deck centre pedestal compass slaving control accessory panel or multi-function display compass select panel interface |  |  |  |  |  |
|  | B2 | Replace an unserviceable or oxidized electrical plug terminal connector at the flux valve wire loom interface according to standard wire electrical repair instructions |  |  |  |  |  |
|  | B2 | Perform an internal system initialisation test and automated Built-In Test Equipment (BITE) check via the magnetic slaving panel display, verifying all error flags clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive ground operational functional test of the remote indicating compass system using a portable flux valve simulator to verify accurate indicator card tracking and heading alignment response |  |  |  |  |  |
|  | B2 | Perform a formal compass swing alignment procedure on a certified airport compass rose grid to calculate residual deviation values and update the flight deck compass correction card metrics |  |  |  |  |  |

E29 – Excluding remote indicating compass systems

| Index No. | Exclusion Category | E29 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot a remote indicating compass system displaying persistent heading card drift, excessive precession errors, or a frozen gyro warning flag, and rectify the underlying wiring or component defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable compass slaving loop failure condition down to an unbonded flux valve housing, a defective slaving amplifier channel, or an open synchronization wiring path, and repair the circuit fault |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) flight dispatch boundaries, night IFR navigation restrictions, and primary heading reference redundancy rules for an unserviceable remote indicating compass channel to execute proper physical circuit deactivation, warning tag placement, and cockpit placarding |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E30 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E30 – Excluding inertial navigation and reference systems

| Index No. | Exclusion Category | E30 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect Inertial Reference Unit (IRU) or Air Data Inertial Reference Unit (ADIRU) main tray chassis installations, structural mounting latch frames, and cooling fan exhaust ducting pathways for physical damage, cracking, or blockage |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of internal electronics bay racking infrastructure to ensure all thermal cooling air sensors, tray alignment indexing pins, and structural attachment points are free from contamination or physical wear |  |  |  |  |  |
|  | B2 | Examine flight deck Inertial Reference System (IRS) mode select panels, alignment display modules, and keypad buttons for structural bezel cracking, switch stickiness, and digital segment display illumination clarity |  |  |  |  |  |
|  | B2 | Inspect high-speed digital data bus communication cable routes (e.g., ARINC 429 paths) linking the ADIRU to primary flight displays for secure clamping, structural bulkhead grommet protection, and clear separation from high-current aircraft power lines |  |  |  |  |  |

E30 – Excluding inertial navigation and reference systems

| Index No. | Exclusion Category | E30 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Verify that the inertial navigation processing unit structural bonding straps and racking attachment ground joints maintain low electrical resistance values to preserve continuous High Intensity Radiated Field (HIRF) shielding bounds |  |  |  |  |  |
|  | B2 | Remove and install an Air Data Inertial Reference Unit (ADIRU) or Inertial Reference Unit (IRU) assembly module from the main avionics bay equipment rack in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for a flight deck overhead pedestal IRS Mode Select Panel (MSP) or navigation display select panel interface |  |  |  |  |  |
|  | B2 | Remove and install a dedicated backup battery pack assembly unit linked directly to the inertial navigation system's emergency power supply bus line |  |  |  |  |  |
|  | B2 | Replace a damaged or defective multi-pin electrical plug connector terminating at the main ADIRU backplane interface wiring bundle using approved avionics wiring repair techniques |  |  |  |  |  |
|  | B2 | Perform an inertial navigation system initialisation test and automated Built-In Test Equipment (BITE) check via the onboard maintenance computer panel, verifying all historical fault codes clear cleanly |  |  |  |  |  |

E30 – Excluding inertial navigation and reference systems

| Index No. | Exclusion Category | E30 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Execute a comprehensive flight deck operational test of the IRS system to complete a full stationary ground alignment cycle, verifying normal transition from alignment status to full navigation mode parameters |  |  |  |  |  |
|  | B2 | Verify proper structural tracking data loop transmission and synchronisation of pitch, roll, heading, and acceleration metrics from the ADIRU onto both primary flight displays and the Flight Management System (FMS) |  |  |  |  |  |
|  | B2 | Troubleshoot an inertial navigation system displaying a persistent alignment failure warning flag, excessive drift rates, or premature alignment dropouts during ground cycles, and rectify the underlying hardware or sensor defecthttps://www.dymenzions.com/en/atex-in-practice/ |  |  |  |  |  |
|  | B2 | Isolate an unserviceable ADIRU heading or attitude data bus communication failure down to an open wiring path, a corrupt digital signal node, or a dirty interface plug, and repair the path connection |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) dispatch allowances, Required Navigation Performance (RNP) operational constraints, and oceanic flight tracking restriction rules for an unserviceable inertial reference channel to execute proper physical circuit breaker deactivation, warning tag installation, and cockpit logbook placarding. |  |  |  |  |  |

E30 – Excluding inertial navigation and reference systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E31 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E31 – Excluding pressurisation systems

| Index No. | Exclusion Category | E31 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect the cabin pressure outflow valve and safety relief valves for mechanical binding, physical obstruction, skin corrosion, and deterioration of the peripheral environmental seals |  |  |  |  |  |
|  | B2 | Perform a detailed visual inspection of the cockpit cabin pressure control panel, monitoring instruments, and landing elevation selectors for glass delamination, bezel cracking, or selector knob stickiness |  |  |  |  |  |
|  | B2 | Examine the forward and aft fuselage pressure bulkhead areas around pressure control components for structural cracking, loose rivets, or evidence of cabin air sealant degradation |  |  |  |  |  |
|  | B2 | Inspect cabin pressure sensor static port lines, intake filters, and pneumatic sensing plumbing loops for moisture accumulation, debris blockage, or physical line chafing |  |  |  |  |  |
|  | B2 | Verify that the digital cabin pressure controller chassis structural bonding jumper straps maintain low electrical resistance values to preserve continuous High Intensity Radiated Field (HIRF) shielding bounds |  |  |  |  |  |

E31 – Excluding pressurisation systems

| Index No. | Exclusion Category | E31 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Remove and install a digital Cabin Pressure Controller (CPC) module unit from the main avionics bay equipment rack or centre pedestal compartment in accordance with the Aircraft Maintenance Manual (AMM) |  |  |  |  |  |
|  | B2 | Locate and execute the physical removal and installation procedure for an aircraft main cabin pressure outflow valve assembly |  |  |  |  |  |
|  | B2 | Remove and install a flight deck cabin pressurisation control panel assembly or dual-channel manual override controller interface |  |  |  |  |  |
|  | B2 | Replace an unserviceable or contaminated cabin pressure altitude warning switch assembly or cockpit differential pressure sensing transducer unit |  |  |  |  |  |
|  | B2 | Perform a cabin pressurisation system initialisation test and automated Built-In Test Equipment (BITE) check via the flight deck maintenance display panel, verifying all fault codes clear cleanly |  |  |  |  |  |
|  | B2 | Execute a comprehensive ground operational functional test of the cabin pressurisation system using external static test rigs or internal system test modes to verify outflow valve modulation and manual gate control tracking |  |  |  |  |  |
|  | B2 | Perform a formal cabin pressure decay and structural leakage rate verification check following outflow valve replacement to confirm fuselage sealing integrity matches airworthiness manual tolerances |  |  |  |  |  |

E31 – Excluding pressurisation systems

| Index No. | Exclusion Category | E31 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Execute a functional calibration check of the flight deck cabin altitude warning horn and passenger oxygen mask deployment switch to ensure automatic activation occurs exactly when the pressurisation system detects a simulated cabin altitude threshold breach |  |  |  |  |  |
|  | B2 | Troubleshoot a cabin pressurisation system displaying erratic pressure fluctuations, high differential pressure warnings, or a complete failure to dump pressure on landing, and rectify the underlying component or sensor defect |  |  |  |  |  |
|  | B2 | Isolate an unserviceable cabin pressure auto-mode switching failure down to a corrupt digital data bus loop, a missing throttle position microswitch input, or a faulty controller lane, and repair the circuit fault |  |  |  |  |  |
|  | B2 | Review Master Minimum Equipment List (MMEL) maximum flight altitude downgrades, unpressurised flight routing constraints, and dispatch boundaries for an unserviceable auto-pressurisation channel to execute proper physical circuit deactivation, warning tag placement, and cockpit placarding |  |  |  |  |  |

E31 – Excluding pressurisation systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E32 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E32 – Excluding electrical systems in aircraft equipped with multi-generator powered systems

| Index No. | Exclusion Category | E32 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot faulty Electrical Power System and rectify defects |  |  |  |  |  |
|  | B2 | Perform operational test of electrical load shedding system according to AMM |  |  |  |  |  |
|  | B2 | Perform operation test of generator priority to AC bus according to AMM |  |  |  |  |  |
|  | B2 | Remove/install voltage regulators and functional test according to AMM |  |  |  |  |  |
|  | B2 | Remove / install transformer rectifier units (TRUs) and functional test according to AMM |  |  |  |  |  |
|  | B2 | Remove / install generator / alternator / IDG and run a functional test according to AMM |  |  |  |  |  |
|  | B2 | Remove/install Emergency Static Inverter and functional test according to AMM |  |  |  |  |  |
|  | B2 | Remove / install generator control unit and functional test according to AMM |  |  |  |  |  |
|  | B2 | Inspect generator phase feeder cables for chafing, heat damage, and security according to AMM |  |  |  |  |  |

E32 – Excluding electrical systems in aircraft equipped with multi-generator powered systems

| Index No. | Exclusion Category | E32 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Inspect APU generator electrical connector pins for distortion and security according to AMM |  |  |  |  |  |
|  | B2 | Perform an insulation resistance (Megger) test on generator phase feeders according to AMM |  |  |  |  |  |
|  | B2 | Inspect the external power receptacle pins for pitting, arcing, or physical wear according to AMM |  |  |  |  |  |
|  | B2 | Remove, install, and functionally test an Auxiliary Power Unit (APU) Generator according to AMM |  |  |  |  |  |
|  | B2 | Perform a functional test of the APU generator auto-start and bus tie logic according to AMM |  |  |  |  |  |
|  | B2 | Perform an external power priority and auto-changeover test using a ground power unit according to AMM |  |  |  |  |  |
|  | B2 | Apply Minimum Equipment List (MEL) restrictions for an inoperative IDG and placard in the cockpit according to MMEL |  |  |  |  |  |
|  | B2 | Apply MEL restrictions for an inoperative APU Generator and configure the system for dispatch according to MMEL |  |  |  |  |  |
|  | B2 | De-activate, secure, and collar the circuit breakers for a faulty generator channel to satisfy MEL operational dispatch requirements according to AMM |  |  |  |  |  |

E32 – Excluding electrical systems in aircraft equipped with multi-generator powered systems

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

###### Exclusion E34 – Category B2

Note:	Complete at least 80% of the tasks listed under the relevant category exclusion to support removal of the exclusion applied for.

E34 – Excluding digital systems

| Index No. | Exclusion Category | E34 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Detailed inspection of cockpit Electronic Flight Instrument System (EFIS) display units for screen delamination, matrix degradation, and structural mounting security |  |  |  |  |  |
|  | B2 | Detailed inspection of digital data bus routing (ARINC 429/717 or Ethernet cables) for minimum bend radius violations, proper shielding termination, and segregation from high-voltage wire bundles |  |  |  |  |  |
|  | B2 | Visual inspection of the primary Air Data Computer (ADC) pitot-static line connections, electrical harness strain reliefs, and chassis bonding straps |  |  |  |  |  |
|  | B2 | Detailed visual inspection of the Flight Control Computer (FCC) multi-pin rear connectors for recessed, bent, or corroded pins |  |  |  |  |  |
|  | B2 | Visual inspection of the weather radar waveguide connections and coaxial data lines for mechanical distortion or moisture ingress |  |  |  |  |  |
|  | B2 | Removal and installation of a cockpit Multi-Function Display (MFD) unit using proper electrostatic discharge (ESD) wrist straps and grounding techniques |  |  |  |  |  |

E34 – Excluding digital systems

| Index No. | Exclusion Category | E34 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Removal and installation of a digital Air Data Computer (ADC) located in the main electronics bay, ensuring correct alignment of the quick-disconnect pneumatic fittings |  |  |  |  |  |
|  | B2 | Removal and installation of a Flight Management Computer (FMC) or Flight Management System (FMS) core processing unit from its avionic rack structure |  |  |  |  |  |
|  | B2 | Removal and installation of an Autopilot Flight Director Computer (AFDC) unit from the main electrical equipment bay |  |  |  |  |  |
|  | B2 | Functional check of the electronic flight instrument system crosstalk and data-reconfiguration capabilities using the cockpit display switching panel |  |  |  |  |  |
|  | B2 | Operational check of the pitot-static system inputs to the digital display units, verifying correct altimeter and airspeed digital readouts against test set parameters |  |  |  |  |  |
|  | B2 | Functional check of the automatic flight control system (AFCS) flight director command bars through their full range of travel via the mode control panel |  |  |  |  |  |
|  | B2 | Troubleshoot an intermittent digital data bus fault causing an "ADC 1 FAIL" flag on the captain's primary flight display; isolate the defect to a loose pin socket in the main terminal block |  |  |  |  |  |
|  | B2 | Diagnose a recurring configuration error on a newly installed multi-function display; isolate the issue to an incorrectly set rear-connector chassis parity programming pin |  |  |  |  |  |

E34 – Excluding digital systems

| Index No. | Exclusion Category | E34 Exclusion Removal – Description of Required Tasks | A/C Rego | Maint Doc Ref. (WO/TC/Logbook) | Applicant Initials | Supervisor Name, ARN and Signature | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B2 | Troubleshoot an automatic flight guidance system failure to engage in vertical navigation (VNAV) mode; trace the fault down to a corrupted navigation database file load |  |  |  |  |  |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

Part 4
Exclusion(s) Removal
Maintenance Practical Experience

## Category B1 / B2

### Additional Practical Maintenance Experience for Category Exclusion Removal – Category B1/B2

The additional practical maintenance experience relevant to the category exclusion must have been gained from a representative cross-section of maintenance index tasks on operating aircraft.

See AMC/GM for CASR Part 66 for information about what is a representative cross-section of maintenance tasks and the user guide for this logbook.

| Index No. | A/C Type and Registration Number | Maintenance Record Referenced and Date | Description of work carried out | Task Type | Category and Exclusion | Supervisor Name, Signature, and Licence Number | Date (dd/mm/yyyy) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

| Index No. | A/C Type and Registration Number | Maintenance Record Referenced and Date | Description of work carried out | Task Type | Category and Exclusion | Supervisor Name, Signature, and Licence Number | Date (dd/mm/yyyy) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Declaration

I hereby declare that the entries in this section of the logbook are a true and accurate record of my practical maintenance experience, and that each task was performed by me under the supervision of the supervisor(s) whose signature appears against the relevant entry. Name: ARN: Date:

____