ANNEX A to CAAP 104-01 v2.2

Sample CAO 104.0 Manual

Disclaimer

This manual template has been prepared to guide and assist Certificate of Approval (COA) holders who propose to train and authorise maintenance certification staff under the provisions of Civil Aviation Order (CAO) 104.0. The procedures and training plans that are set out in this manual represent the minimum acceptable standards that are required in order to satisfy the requirements set out in CAO 104.0.

It is not a prerequisite that this manual must be used in order to obtain an approval to authorise certification staff; however, the training outcomes specified in each section of this manual must be met or exceeded for each particular aircraft or system for which training is to be given.

An applicant for approval under CAO 104.0 may use any form of manual that suits the particular COA holder provided that these minimum requirements are incorporated into that manual.

How to use this manual

This manual is made up of six general and four training topics:

General

- Sections 1 to 5 – describe the general administration and management processes applicable to the training procedures

- Section 10 – provides practical performance assessment guidelines for use within each of sections 6, 7, 8 and 9.

Training

- Section 6 – describes the procedures for training and authorising warbirds, historic and replica aircraft (WHR) employees

- Section 7 – describes procedures for training and authorising certain excluded type employees

- Sections 8 and 9 – describe the procedures for training and authorising excluded system employees in wood aircraft structures and/or fabric coverings.

- If you do not wish to use the full scope of the training manual, you may delete one or more of sections 6, 7, 8 and 9. However, if you propose to use this manual in part or in its entirety, then you should read the manual carefully and ensure that you are able to incorporate the chosen sections into your organisational procedures. Once approved, you will be required by regulation 11.077 of the Civil Aviation Safety Regulations 1998 (CASR ) to comply with the manual. Therefore, you should ensure that it is specific to your operations, not just a generic manual with your name on the cover page.

Important note: When assessing an application for a delegation under subregulation 42ZC (6) of Civil Aviation Regulation 1988 (CAR) Civil Aviation Safety Authority (CASA) will refuse the application if the manual does not meet these minimum requirements.

Wherever a blank space appears, you will be required to ensure that you insert the appropriate information. Depending on context, the required information may be any of the following:

- your name or the name that is shown on your COA

- your Aviation Reference Number (ARN)

- the identifying details of an aircraft as shown in the Part 66 Manual of Standards (MOS)

- the name of the person responsible for managing the training program

- the name of an employee under training.

If a phrase, position name or title is shown with multiple options (e.g. Training/Quality Manager) you should remove all but the correct option for your organisation. If the position is held under a different title in your organisation - use that title.

# Applicability statement and responsible manager certification

This manual has been generated to meet CASA Civil Aviation Order (CAO) 104.0 requirements for the purpose of providing maintenance training and issuing certification authorisations.

This manual sets out the procedures and assessment guidelines for the issue of certification authorisations that will be followed by …………………. working under the COA number…………………………..

This manual forms part of the ………………….. Quality and Procedures Manual, as accepted by CASA. Amendments to this manual will only be made by the Training/Quality Manager, in accordance with the procedures set out in this manual.

Authorised by (signature): ……………………	Training/Quality Manager (delete whichever does not apply)

Date					Revision: 1.0

# Distribution List

| Copy number | Manual holder |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |

Contents

1	Applicability statement and responsible manager certification	4

2	Distribution List	5

3	List of Effective Pages	8

3.1	Amendment Record	8

4	Introduction	9

5	Training program management	10

5.1	Qualifications	10

5.2	Training/Quality Manager responsibilities	10

5.3	Trainee eligibility assessment guidelines	11

6	Training procedures	11

6.1	Approved training providers	12

7	Training for initial issue of a Part 1 WHR certification authorisation.	13

7.1	Assessment for initial issue of a Part 1 certification authorisation	13

7.2	Assessment for re-issue of a Part 1 certification authorisation	13

7.3	Part 1 WHR training plan outline	14

8	Excluded type training procedures	19

8.1	Background	19

8.2	Scope of this section	19

8.3	Training procedure	19

8.4	Assessment	19

8.5	Authorisation issue	19

8.6	Notification to CASA	19

9	Wood structures and fabric covered surfaces	24

9.1	Inspect, test and repair fabric surfaces	24

10	Inspect and repair aircraft wooden structures	27

10.1	Objectives	27

10.2	Reference material	27

10.3	Theoretical knowledge	27

10.4	Practical training wood structure inspection and repair.	28

11	Practical performance assessment	29

11.1	Activity	29

Appendix A	Aircraft for which excluded type training may be provided	31

Appendix B	Certification authorisation form	33

Appendix C	Manual holder’s acknowledgement of receipt of amendment	35

# List of Effective Pages

| Page | Issue date |
| --- | --- |
| Cover page |  |
| Page 1 |  |
| Page 2 |  |
| Page 3 |  |
| Page 4 |  |
| Page 5 |  |
| Page 6 |  |
| Page 7 |  |
| Page 8 |  |
| Page 9 |  |
| Page 10 |  |
| Page 11 |  |
| Page 12 |  |
| Page 13 |  |
| Page 14 |  |
| Page 15 |  |
| Page 16 |  |
| Page 17 |  |

## Amendment Record

| Amendment issue number | Date |
| --- | --- |
| Initial Issue |  |

# Introduction

This document sets out the procedures, processes and documentation that will be followed for the purpose of training maintenance personnel in the maintenance of excluded aircraft systems, excluded aircraft types or WHR Part 1 aircraft as described under Section 5, 6, 8 or 10 of CAO 104.0.

Following these procedures, maintenance training staff will be able to:

- provide training in accordance with these procedures

- assess an employee’s competencies in accordance with these procedures

- where appropriate, issue a certification authorisation in accordance with these procedures.

Note:	For the purpose of this manual, employee means any individual who is performing or certifying for maintenance on behalf of and under the control of the COA holder.

# Training program management

This program will be managed by a person appointed to act as the Training/Quality Manager.

## Qualifications

A Training/Quality Manager must have the following attributes:

- credentials in quality management, acceptable experience in a training or quality management role, or acceptable experience in a maintenance supervision role

- a high level of understanding of the maintenance regulations in Parts 4 and 4A of CAR

- an understanding of the Part 66 licence regulations

- an understanding of workplace training and assessment processes

- a high level of understanding of the provisions and application of CAO 104.0

- a high level of understanding of the procedures set out in this training and assessment manual

- a high level of understanding of the responsibilities of the person administering the CAR 42ZC (6) delegation on behalf of the COA holder.

## Training/Quality Manager responsibilities

The Training/Quality Manager will have the following responsibilities:

- assessment of training providers using the guidelines set out in Sections 5 and 7 of this manual

- if external training providers are to be engaged, assessment of the external provider to ensure that the provider is able to deliver the required training outcomes as set out in this manual

- ensuring that each training plan is configured to:

  - adequately meet the trainee’s identified training needs

  - provide training in all aspects of the aircraft/engine that are unique to the aircraft/engine particularly features or techniques that would be unlikely to be encountered during normal performance of the trainee’s licence privileges

- review training results for each trainee and determine whether the training objectives have been satisfactorily achieved

- issue certification authorisations upon satisfactory completion of training and assessment

- notify CASA within 5 months of issuing a certification authorisation for an excluded system or type, and again at the expiration of 6 months if in the opinion of the manager, the trainee has successfully completed the training and certification period

- monitor the training program, for effectiveness and amend the program if deficiencies are identified

- notify CASA of any amendments to the training programs set out in this manual and provide CASA with copies of the amended pages with the changes highlighted with change bars

- ensure that certification authorisations are only issued or re-issued while the COA holder holds a CASA delegation for the purpose of subregulation 42ZC (6) of CAR to issue certification authorisations to employees for the purpose of paragraph 42ZC (4) (e) of CAR.

## Trainee eligibility assessment guidelines

In order to meet the requirements specified in CAO 104.0 the following course entry requirements will be applied:

- the trainee must possess a valid CASA Part 66 licence in the applicable subcategory for the aircraft for which training is to be provided.

- If training is for an excluded type or a Part 1 WHR certification authorisation, the trainee’s licence must not be affected by an exclusion which would disallow maintenance of the particular aircraft (i.e. E9-fabric surfaces, E10 -wooden structures, E12-propellers).

- If training is for an excluded system the trainee must have the appropriate licence category and sub-category for the system as set out in the CASA Part 66 Manual of Standards.

Guidance note: The following training program will ensure that an individual is able to demonstrate the required competencies for the issue of a certification authorisation. The training program needs to include the following:

training in the use of relevant data and documentation

training in maintenance tasks that are unique to the aircraft, engine or system

training in the application of any specialised maintenance techniques that are unique to the aircraft, engine or system

training in the use of test equipment that is unique to the aircraft or engine.

# Training procedures

## Approved training providers

Instruction will be provided by personnel experienced and qualified in the type of maintenance training being given to an individual under this training plan.

Training will be given in an on-the-job format or a combination of on-the-job and theoretical training. The makeup of the training will be decided by the training manager, based on an assessment of the needs of the individual trainee and the complexity of the particular aircraft.

Training can be given by approved licence holders or task specialists with recognised skills and experience in the relevant discipline. In the case of ex-military aircraft, training may also be provided by non-licenced individuals who have provided or supervised maintenance for a particular Part 1 aircraft in the military environment or provided maintenance training for the particular aircraft in the military environment.

Approved training personnel will be listed in a register of trainers contained in each relevant section of this manual and be revised as instructors are added or removed.

The register will detail the names of approved trainers and their qualifications for the role.

# Training for initial issue of a Part 1 WHR certification authorisation.

Part 1 WHR training will be given in accordance with the training plan in this section, which will be varied as required to reflect the individual’s training needs and specific requirements of the aircraft for which training is to be given.

A record of training and assessment will be retained in the personnel file of the individual and maintained by the Training/Quality/Aircraft Maintenance Manager.

Training records will be made available to CASA for review, as required by CASA.

## Assessment for initial issue of a Part 1 certification authorisation

Upon successful completion of training, a trainee will be assessed in accordance with the assessment guidelines set out in Section 10 of this manual. Assessment will be made by the approved training provider conducting the training. The results will be reviewed by the Training/Quality Manager.

If the Training/Quality Manager is satisfied that the trainee has achieved an acceptable level of competency in all knowledge topics listed in Table 1 and the maintenance tasks listed in Table 2, the trainee will be issued with a certification authorisation for the Part 1 aircraft in which the training and assessment was given. The certification authorisation will be valid for not more than 2 years from the date of issue.

Personnel holding current Part 66 licences in the category and ratings that cover a particular Part 1 aircraft will be deemed to have met the training requirements as specified in Section 8 of CAO 104.0 and may be issued with a Part 1 certification authorisation.

Personnel who hold a CASA issued maintenance authorisation issued under regulation 33B or paragraph 42ZC (6) of CAR will be deemed to have met the training requirements as specified in section 8 of CAO 104.0; and may be issued a Part 1 certification authorisation.

## Assessment for re-issue of a Part 1 certification authorisation

At the end of a 2 year period since issue or reissue of a certification authorisation, the Training/Quality Manager must assess the trained WHR employee to re-issue the certification authorisation for a further 2 years.

If, in the view of the Training/Quality Manager, no additional or refresher training is required, the certification authorisation may be reissued accordingly.

If, in the view of the Training/Quality Manager, further training is required, a training plan will be drawn up based on the assessed training needs.

Training for re-issue may be given by an authorised trainer or an employee who holds an equivalent valid certification authorisation.

Certification authorisations held by employees on the basis of a CASA issued authorisation or type rating, may be reissued without further assessment at the Training/Quality Manager’s discretion.

## Part 1 WHR training plan outline

Maintenance training program

………………………………. Pty Ltd 		COA Number ………………

Aircraft Type……………	Model…………..	Engine……………………..

Trainee………………………………………………..

### Objectives and outcomes

The Training/Quality Manager will ensure that a person is properly trained in the knowledge elements (Table 1 below) and maintenance procedures (Table 2) that are specific to the type of aircraft.

On completion of the training, the trainee will have demonstrated the competencies required to maintain the type of aircraft to an acceptable standard of airworthiness in accordance with the relevant instructions for continuing airworthiness.

### Reference material

- CAO 104.0

- Part 4 and 4A of CAR

- Applicable aircraft or system maintenance data.

### Processes

This training plan is based on the use of on-the-job training that will be provided using the aircraft and maintenance facilities of ………………..

Training and assessment will be provided by an employee or contractor of ................................., who has the knowledge, qualifications and licence/authorisation required to perform and certify for completion of the maintenance for which the training is provided. Training in knowledge topics must verify that the trainee meets the appropriate knowledge level. If deemed appropriate, external training providers may be engaged to provide supplementary theory instruction or skills training in particular special processes or techniques.

### Explanation of knowledge levels

#### Level 1

A familiarity with the principal elements of the topic such that the following trainee objectives are met:

- familiar with the basic elements of the topic

- able to give a simple description of the topic, using common words and examples

- able to use typical terms.

#### Level 2

A general knowledge of the theoretical and practical aspects of the topic and an ability to apply that knowledge, such that the following trainee objectives are met:

- understand the theoretical fundamentals of the topic

- give a general description of the topic using, as appropriate, typical examples

- demonstrate awareness of practical applications of the topic.

#### Level 3

A detailed knowledge of the theoretical and practical aspects of the topic, and a capacity to combine and apply the separate elements of knowledge in a logical and comprehensive manner, such that the following trainee objectives are met:

- describe the underlying intent and implications of the topic

- give a detailed description of the topic using theoretical fundamentals and specific examples

- explain in detail the theoretical and practical application of the topic.

Table 1 – Knowledge elements Part 1 WHR

| Topic | Level | Achieved |  |
| --- | --- | --- | --- |
|  |  | Y/N | Date and Signature |
| Source appropriate data references for each maintenance task. | 3 |  |  |
| Demonstrate an understanding of where to find: inspection wear limits damage limits acceptable repair practices service life limitation maintenance instructions special inspections | 3 |  |  |

Table 2 – Maintenance procedures

| Ground handling | Competent |  |
| --- | --- | --- |
|  | Y/N | Date and Signature |
| Demonstrate familiarity with: mooring earth points towing and turning limits ground power starting ……………………… ……………………… (Insert additional specific items if required) |  |  |
| Inspections | Competent |  |
|  | Y/N | Date and Signature |
| Perform under supervision: daily Inspection 25 hour inspection and service 50 hour inspection and service Annual inspection …………………………………… (Insert additional specific items if required) |  |  |

| Airframe | Competent |  |
| --- | --- | --- |
|  | Y/N | Date and Signature |
| Jacking and levelling - Demonstrate knowledge of: Safety precautions during jacking Jacking points Levelling points |  |  |
| Control surface travel checks - Demonstrate an understanding of control surface travel limits, and competency in the use of any aircraft specific travel measuring equipment such as travel gauges etc (Delete if only standard tools and procedures required) |  |  |
| Undercarriage functional tests Strut charging - Demonstrate use of any aircraft specific strut charging equipment or techniques (Delete if only standard tools and procedures required) |  |  |
| Flight control system rigging Flight control system – if applicable, demonstrate competency in aircraft specific techniques required when removing, replacing or adjusting the ……………..….system (Delete if only standard tools and procedures required) |  |  |
| Pressurisation tests (if applicable) – Demonstrate competency in setting up and operating ground pressurisation unit and an understanding of any aircraft specific safety requirements (Delete if no specific differences to other pressurised aircraft) |  |  |
| Crew oxygen system (if applicable) – Demonstrate competence in any applicable aircraft specific techniques when servicing crew oxygen system (Delete if no specific differences to other pressurised aircraft) |  |  |
| Explosive devices - Demonstrate comprehensive understanding of safety requirements and maintenance procedures related to: ground safety-disarming safety requirements before entering cockpit flight arming removing and installing ejection seats (Delete if no explosive devices fitted) |  |  |
| Structure- Special repair techniques - Demonstrate competency in repairing any of the following if applicable: bonded wood sandwich construction (such as ply/balsa/ply) laminated plywood wood box beams composite structures fabric covering and repairs (For each aircraft, delete if not required or insert additional training headings as required) |  |  |

| Engine | Competent |  |
| --- | --- | --- |
|  | Y/N | Date and Signature |
| General - Demonstrate competency to carry out the following tasks to an acceptable standard of airworthiness: compressor/engine wash compressor/engine inspection oil sampling fuel system component changes and rigging fuel control unit installation and rigging |  |  |
| Engine ground runs-high powered aircraft Demonstrate situational awareness with regard to danger zones around intakes and propeller/jet exhaust. Select run-up area where noise and propeller/jet blast will not create hazards to others. Demonstrate proper aircraft anchoring procedures. Conduct engine runs observing any type-specific precautions and procedures |  |  |

At the successful completion of the training and assessment in each task, the quality manager will be notified that the trainee is eligible for issue of a certification authorisation.

Table 3– Training providers for Part 1 WHR Maintenance

| Name | Authorised to provide training on: | Qualifications |
| --- | --- | --- |

# Excluded type training procedures

## Background

This section describes the procedures that will be followed by ………………when training and assessing an employee for the purpose of issuing a certification authorisation for one or more excluded aircraft types.

## Scope of this section

Training and authorisation under this section will only be given for those excluded aircraft types described for the purpose in the Table 2 of Appendix IX of the Part 66 MOS, and will be confined to aircraft listed in section 10 of this manual

## Training procedure

A trainee will be trained in the theory and practical elements listed in Tables 4 and/or 5 and 6 of this manual. This training will be given as class-room training and/or on-the-job training.

Class-room training will only be provided for engines, or engine systems if in the opinion of the Training/Quality Manager, a particular aspect of an engine or engine system is sufficiently complex to require more than on-the-job training instruction.

## Assessment

A trainee will be assessed during the course of the training and a final assessment will be at completion, using the guidelines set out in Annex A of this manual.

Assessment will be made by the training provider and the results will be reviewed by the Training/Quality Manager.

## Authorisation issue

If a trainee is assessed as having met the required competency standards, the Training/Quality Manager will issue a certification authorisation for the aircraft (engine) type for which the training was given.

A certification authorisation for an excluded aircraft type will be issued for a period of 
6 months and will not be renewable.

## Notification to CASA

At the completion of 5 months from the date of issue of a certification authorisation to an employee, the Training/Quality Manager will send a preliminary notification to CASA Permissions Application Centre (PAC).

The notification needs to include the following details:

- name and COA number of the organisation

- name and ARN of the employee

- the excluded aircraft type for which the authorisation has been issued

- date at which the authorisation was issued

- name and position of the person making the notification.

At the completion of 6 months, the Training/Quality Manager will advise CASA that the Licensed Aircraft Maintenance Engineer (LAME) has successfully completed the excluded type training and has satisfactorily exercised the privileges of the certification authorisation for 6 months.

Table 4 – Instruction plan- Piston Engines

| ATA Chapter | Elements | Required? Y/N | Theory Assessment Person/Sign/ Date | Practical Assessment Person/Sign/ Date | Quality Assurance Person/Sign/Date |
| --- | --- | --- | --- | --- | --- |
| 70 | Standard practices-engines |  |  |  |  |
| 70A | Constructional arrangement and operation (carburettors, fuel injection systems, induction, exhaust and cooling systems, supercharging/ turbocharging, lubricating systems |  |  |  |  |
| 70B | Engine performance |  |  |  |  |
| 71 | Powerplant |  |  |  |  |
| 73 | Engine fuel and control |  |  |  |  |
| 73A | FADEC |  |  |  |  |
| 74 | Ignition |  |  |  |  |
| 76 | Engine control |  |  |  |  |
| 77 | Engine control systems |  |  |  |  |
| 79 | Oil |  |  |  |  |
| 80 | Starting |  |  |  |  |
| 81 | Turbines |  |  |  |  |
| 82 | Water injection |  |  |  |  |
| 83 | Accessory gearboxes |  |  |  |  |

Table 5 – Instruction plan - Turbine Engines

| ATA Chap | Elements | Required? Y/N | Theory Assessment Person/Sign/ Date | Practical Assessment Person/Sign/ Date | Quality Assurance Person/Sign/Date |
| --- | --- | --- | --- | --- | --- |
| 49 | Auxiliary power (APUs) |  |  |  |  |
| 70 | Standard practices-engines |  |  |  |  |
| 70A | Constructional arrangement and operation (installation, inlet, compressors, combustion section, turbine section, bearings and seals, lubrication systems) |  |  |  |  |
| 70B | Engine performance |  |  |  |  |
| 71 | Powerplant |  |  |  |  |
| 72 | Engine turbine and turboprop and ducted fan and unducted fan |  |  |  |  |
| 73 | Engine fuel and control |  |  |  |  |
| 73A | FADEC |  |  |  |  |
| 74 | Ignition |  |  |  |  |
| 75 | Air |  |  |  |  |
| 76 | Engine controls |  |  |  |  |
| 77 | Engine indicating systems |  |  |  |  |
| 78 | Exhaust |  |  |  |  |
| 79 | Oil |  |  |  |  |
| 80 | Starting |  |  |  |  |
| 82 | Water injections |  |  |  |  |
| 83 | Accessory gearboxes |  |  |  |  |
| 84 | Propulsion augmentation |  |  |  |  |

Table 6 – The employee will be trained and assessed in the following maintenance procedures

| Activity | Competent |  |
| --- | --- | --- |
|  | Y/N | Date and Signature |
| Inspections (delete non – applicable items): Perform under supervision: Daily Inspection 25 hour inspection and service 50 hour inspection and service Annual/100hourly inspection …………………………………… …………………………………… …………………………………… …………………………………… (Insert additional specific items if required) |  |  |
| Engine General- demonstrate competency to carry out the following tasks to an acceptable standard of airworthiness: servicing oil/coolant etc fuel system component changes and rigging magneto removal/installation and timing compressor wash |  |  |
| Engine ground runs-high powered aircraft/turbine powered aircraft: demonstrate situational awareness with regard to danger zones around propeller ensure that jet engine danger zones are clear of personnel and loose objects select run-up area where noise will not create hazards to others. demonstrate proper aircraft anchoring procedures conduct engine runs observing any type-specific precautions and procedures |  |  |
| Engine Inspection-demonstrate competency in using inspection aids (including borescope if applicable) to carry out: cylinder inspection crankcase inspection camshaft/cam ring inspection internal compressor/combustion chamber/turbine inspection interpret inspection results using appropriate data references |  |  |

At the successful  completion of the training and assessment in each task, the Quality Manager will be notified that the employee is eligible for issue of a certification authorisation.

Table 7 - Training providers for excluded type or system maintenance

| Name | Authorised to provide training in/on: | Qualifications |
| --- | --- | --- |

Prerequisites

Training personnel, for the purpose of excluded type or excluded system training, must have held a maintenance licence (CAR 30/Part 66/both) in the appropriate subcategory for a minimum of 5 years. The trainer must have held a rating in the particular aircraft or engine for which training is to be given for at least 18 months.

A person who holds, or has held, a CASA maintenance authorisation for the particular aircraft or engine for at least 5 years will be deemed to have met the requirements.

# Wood structures and fabric covered surfaces

## Inspect, test and repair fabric surfaces

### Objectives

On completion of this training, a trainee will be able to apply hand skills, knowledge of fabric surfaces and maintenance publications to inspect, test and repair aircraft fabric surfaces and the application of hand skills, knowledge of fabric surfaces and maintenance publications to re-cover fabric surfaces of aircraft and aircraft components that are covered with fabric.

### Reference material

- FAA Advisory Circular (AC) 43-13: current edition

- Poly-Fiber Procedure Manual

- Ceconite Procedure Manual

- Applicable aircraft maintenance data

### Knowledge requirements

A trainee will be required to demonstrate a comprehensive theoretical knowledge of the following subject matters: (Knowledge levels are explained in Section 10).

Table 8 - Knowledge requirements

| Topic | Level | Achieved |  |
| --- | --- | --- | --- |
|  |  | Y/N | Name /Signature /Date |
| WHS hazards associated with fabric covering processes and how to obtain relevant MSDS and PPE | 1 |  |  |
| regulatory requirements relating to the repair of fabric surfaces | 1 |  |  |
| industry publications relating to the repair of fabric surfaces | 1 |  |  |
| aircraft fabric surface covering and finishing materials and principles | 2 |  |  |
| fabric-covered component attachment methods | 2 |  |  |
| types of fabric and related characteristics | 2 |  |  |
| types of tape, cord and thread and their relative advantages and disadvantages | 2 |  |  |
| cements and finishing scheme materials, their characteristics, uses, storage requirements and related safety precautions | 2 |  |  |
| inspection requirements for new fabric prior to use | 3 |  |  |
| the criteria for fabric rejuvenation, repairs and repair methods, including the need for re-balancing of flight control surfaces after fabric rejuvenation or repair | 3 |  |  |
| the application of flight loads to fabric-covered surfaces, and related failure mechanisms | 3 |  |  |
| compatibility of differing fabric materials and adhesives, sealants and dopes | 2 |  |  |
| acceptable methods of testing fabric for deterioration | 3 |  |  |
| acceptable methods of applying adhesives, sealants and tautening dopes | 3 |  |  |
| acceptable methods for heat tautening of Dacron coverings | 3 |  |  |

If a trainee has passed the CASA Basic examination FE Fabric and Doping, the COA holder may deem the employee as having met the relevant required knowledge standards for this section.

### Practical training

Using aircraft when available, and/ or training aids as required a trainee will be trained in the following maintenance tasks:

identify the type of fabric covering (cotton/Dacron/fibreglass etc.)

using relevant data inspect for:

blockage of drainage holes at water and dirt collection points

deterioration of protective finishings

deterioration of fabric tensile strength

damage to inspection panels and zips

carry out at least two of the following types of fabric repairs in accordance with applicable repair instructions:

by surface rejuvenation

by stitching

by doped-on repair

apply tightening substrates (if applicable)

apply protective finishes

replace reinforcing rings

using an aircraft or a major subassembly of an aircraft, a trainee will be trained in the removal and replacement of the fabric covering using either the sewn envelope or doped/glued blanket method, including inspection and repair/replacement (as necessary) of inter-rib bracing tapes and anti-chafing tapes

the trainee will be trained on an aeroplane or major subassembly that requires the application of reinforcing tapes, use of rib stitching and application of finishing tapes

the trainee will be trained to inspect structural members, wiring, grommets and system components and initiate any necessary repair or rectification action before a new covering is applied

the trainee will apply fabric surface finishes making use of base or sealing coats, UV protective coats and final colour coats.

### Assessment criteria

The trainee will be expected to:

- refer to relevant maintenance documentation in order to make an airworthiness assessment

- make appropriate preparation of the aircraft and establish access to the aircraft structure to allow for proper inspection in accordance with maintenance documentation

- inspect/test aircraft fabric for signs of damage, deterioration or loss of tensile strength in accordance with maintenance documentation and approved procedures

- assess damage or deterioration against limits specified by maintenance manual or other approved data to determine if repair, restoration or replacement is required

- inspect structural members, wiring, grommets and system components and initiate any necessary repair or rectification action

- fabricate and fit a fabric covering with appropriately placed inspection panels, zips and drainage holes

- apply a fabric finishing scheme ready for paint

- apply appropriate paint

- recognise defective doping and analyse the cause of the defect.

### Required performance standards

The required performance standards are the following:

- extent of damage is correctly assessed to assist in determining repair procedure

- structure is supported and prepared in accordance with the applicable maintenance manual to ensure personal safety and freedom from damage

- appropriate repair scheme is identified in accordance with maintenance manual and/or approved data

- all materials and equipment required are organised

- fabric repairs are performed, in accordance with approved repair scheme, ensuring that aircraft standard practices are used and process requirements are carried out

- components are adjusted and/or re-balanced, where necessary, to operate within prescribed specifications

- where required, repaired components or assemblies are tagged, sealed and packaged or cradled in accordance with specified procedures.

At the successful completion of the training and assessment in each task, the Quality Manager will be notified that the employee is eligible for issue of a certification authorisation.

# Inspect and repair aircraft wooden structures

## Objectives

On completion of this training, a B1 Part 66 licence holder will be proficient in the application of woodworking hand skills and knowledge and the use of maintenance publications to inspect and repair aircraft wooden structures and wooden components to an acceptable standard for the purpose of applying to CASA for removal of an E10 exclusion.

## Reference material

- FAA AC 43-13: current edition

- Applicable aircraft maintenance data

## Theoretical knowledge

A trainee will be trained to understand the types of timbers and plywoods that are acceptable for aircraft use, their properties, strengths and limitations and attain the knowledge levels listed in section 10.

If a trainee has passed the CASA Basic examination FD Wooden Structures, the COA holder may deem the employee as having met the relevant required knowledge standards for this section.

Table 9 - Theoretical knowledge

| Topic | level | Achieved |  |
| --- | --- | --- | --- |
|  |  | Y/N | Name/Signature/Date |
| The types of glues suitable for aircraft use | 2 |  |  |
| The differences between: caseins resorcinols urethanes epoxies | 1 |  |  |
| With regard to the various glue types, their: strengths limitations storage requirements mixing precautions spreading requirements assembly times clamping requirements environmental protection requirements compatibilities | 2 |  |  |
| The significance of: grain slope, density and straightness sap pockets, shakes and knots moisture content and stability | 2 |  |  |
| With regard to wood structures the significance of: compression cracks splits rot fungal attack | 2 |  |  |

## Practical training wood structure inspection and repair.

The trainee will be trained to:

- use typical hand tools including:

  - saws

  - planes

  - chisels

  - clamps

  - drills

- inspect a timber structure using:

  - tap tests to detect voids

  - feeler gauges to determine debonding or delamination

  - borescope and ink tests to test for compression cracks

- fabricate

  - a scarf splice in a spar (may be performed on a practice piece)

  - a plywood scarf joint (may be performed on a practice piece)

- repair a damaged wing rib (may be performed on a training aid)

- repair surface protection (may be performed on a training aid).

At the successful completion of the training and assessment in each task, the Quality Manager will be notified that the employee is eligible for issue of a certification authorisation.

# Practical performance assessment

To be used when assessing an employee in accordance with sections 2, 4 and 5 of this manual

Trainee………………………………………………

AIRCRAFT type/registration…………………...

## Activity

Trainee objectives assessment form (for use by the assessor)

| USE REPORTS & INDICATIONS | PERFORMANCE ASSESSMENT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ATTEMPTS |  |  | RESULT * |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee reads the available reports and indications (maintenance task) |  |  |  |  |  |
| Trainee interprets the reports and indications correctly (Opens proper Manuals/takes right actions to start the process) |  |  |  |  |  |
| FIND & USE AIRCRAFT DOCUMENTATION | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee finds proper troubleshooting procedure, if necessary |  |  |  |  |  |
| Trainee makes the correct interpretation on TSM, AMM and other related procedures (this shows in the actions the trainee takes) |  |  |  |  |  |
| CORRECTLY PERFORM ACTIONS | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee follows the procedure steps |  |  |  |  |  |
| Trainee make sure that actions are properly done |  |  |  |  |  |
| Trainee uses required tooling |  |  |  |  |  |
| OPERATE IN COMPLIANCE WITH ENVIRONMENT | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee scans the environment before starting the task to ensure safety |  |  |  |  |  |
| Trainee reads/interprets safety warnings correctly |  |  |  |  |  |
| Trainee informs people of his/her work, if necessary |  |  |  |  |  |
| Trainee continuously scans environment during task performance |  |  |  |  |  |
| Trainee reacts properly to changes during task performance to ensure safety |  |  |  |  |  |
| SYSTEM INTERACTION | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee ‘analyses’ the consequences of other systems before performing an action (trainee can do this him/herself or by asking the assessor or a knowledgeable colleague) |  |  |  |  |  |
| Trainee takes other systems into account when acting on a system |  |  |  |  |  |
| PERFORMS AIRCRAFT FINAL / CLOSE-UP | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee restores the aircraft back to initial condition (or appropriate condition depending on the circumstances) |  |  |  |  |  |
| REPORTS IN MAINTENACE RECORDS | ATTEMPTS |  |  | RESULT |  |
|  | 1st | 2nd | 3rd | U | S |
| Trainee fills the proper field in the maintenance record |  |  |  |  |  |
| Trainee uses proper references and descriptions in the maintenance record |  |  |  |  |  |

| Coaching comments | Assessors Name |
| --- | --- |
| 1st attempt |  |
| 2nd attempt |  |
| 3rd attempt |  |

| RESULT OF THE ASSESSMENT |  | Succeeded Remedial |
| --- | --- | --- |
| TRAINEE NAME: |  |  |
| TRAINEE SIGNATURE: |  | DATE: |
| ASSESSOR NAME: |  |  |
| ASSESSOR SIGNATURE: |  | DATE: |

| Unsatisfactory | Satisfactory |
| --- | --- |
| Observed performance not adequate/ had safety implications | Observed performance was adequate |

DATA: The trainee will be required to demonstrate a comprehensive understanding of the maintenance manuals and other data related to the aircraft on which training is being given.

###### Aircraft for which excluded type training may be provided

Aircraft for which excluded type training may be provided:

Small or non-rated aircraft  (Bristol Centaurus)

Small or non-rated aircraft (De Havilland Goblin 35)

Small or non-rated aircraft (PW R1830/R2000)

Small or non-rated aircraft (PW R2800)

Small or non-rated aircraft  (Rolls Royce/Packard Merlin)

Small or non-rated aircraft (Wright R1820)

Small or non-rated aircraft (WSK PZL (Kalisz) Asz 62IR-M18)

Small or non-rated aircraft (Wright R2600)

Small or non-rated aircraft (Wright R3350)

###### Certification authorisation form

XXXXX  Maintenance.

COA no ……………………………

Certification authorisation

Issued pursuant to subsection 7 / subsection 8/ subsection 10 of CAO 104-1

(delete whichever does not apply)

Issued to ………………………

(Name of holder) …………………….is authorised to carry out maintenance and  perform maintenance certification for the following Part 1 WHR aircraft / excluded aircraft type/ excluded system on behalf of xxxxxx Maintenance

Aircraft

1…………………………………………….

2…………………………………………….

3……………………………………………..(expand if required)

System (delete if not applicable)

1. Wood aircraft structures

2. Fabric aircraft coverings

This authorisation expires at midnight on (date)…………………….

Issued by………………………

Position:   Quality Manger/Chief Engineer/Training Manager. (delete all which do not apply)

Signed……………………………………

Date………………………………………

###### Manual holder’s acknowledgement of receipt of amendment

CAO 104.0 training manual amendment-acknowledgement

Amendment Number…………………………		Date…………………………..

Manual Number…………………….		Manual Holder……………………….

I have actioned the amendment and have read and understand the change(s)

Signed……………………………………	Date…………………………….

Each manual holder, on receipt of a manual amendment must action the amendment as required (pen amendment or page insertion/replacement) and having done so, return this signed acknowledgement to ..................................

The acknowledgement may be returned to …………….. via post, courier or emailed as a PDF document.